3GPP TSG-RAN WG2 Meeting #129	R2-2501506

# Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0147 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDCP SN Gap report Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description of the transmit operation of the PDCP SN Gap report is innacurate: - Not clear what “the” from the current Gap-report trigger condition “the PDCP SDU(s) are discarded as specified in clause 5.3”, refers to since there is no prior occurrence of “PDCP SDU(s)” in that section. - Not clear whether the references to “the … discarded PDCP SDU” are to “the discarded PDCP SDU(s)…” in the preceding trigger condition that “…have not been submitted by RLC to lower layers” or not. - Depending on #2, not clear how the bits of the bitmap should be set. - It was agreed in RAN#127 that the change to cover the case for DC and CA can be added when there is a PDCP CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following corrections are made: - Delete “the” from the current Gap-report trigger condition “the PDCP SDU(s) are discarded as specified in clause 5.3”, since there is no prior occurrence of “PDCP SDU(s)” in that section. - Clarify that the references to “the … discarded PDCP SDU” are to “the discarded PDCP SDU(s)…” in the preceding trigger condition that “…have not been submitted by RLC to lower layers” to ensure that only a PDCP SDU or its discard indication is transmitted at MAC, but not both. - Correct the setting of the bits in the bitmap accordingly. - The phrase ‘any RLC entity’ is included to cover the CA/DC cases. Impact analysis Impacted 5G architecture option: NR SA. Impacted functionality: PDCP SN Gap Report. Inter-operability: - If the network is implemented according to the CR and the UE is not, an uplink SDU still being transmitted might be signalled as discarded by the PDCP SN gap report from the UE. - If the UE is implemented according to the CR and the network is not, a downlink SDU still being transmitted might be signalled as discarded by the PDCP SN gap report from the gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An SDU still being transmitted might be signalled as discarded by the PDCP SN gap report in parallel, potentially resulting in an unnecessary discard. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.16.1	Transmit operation

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

If a PDCP SN gap report is triggered for these discarded PDCP SDU(s), the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with these discarded PDCP SDU(s);

-	if more than one PDCP SDU is discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first of these discarded PDCP SDU(s) up to and including the last of these discarded PDCP SDU(s), rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '1' for these discarded PDCP SDU(s);

-	setting in the discard bitmap field as '0' for all other PDCP SDU(s).

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

End of Changes