3GPP TSG-RAN2 Meeting #129	R2-2501538

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1906 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optional feature to support RAN-based Notification Area Update for Multimedia Priority Service (MPS) [MPS_RNAU_ResumeCause] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, FirstNet, AT&T, Verizon, Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The purpose of this CR is to add an optional feature without UE radio access capability parameters to address the handling of the MPS-subscribed UE during RAN-based Notification Area Updates (RNAUs). In RRC_INACTIVE state, a UE remains CM-CONNECTED and can move within a notification area previously-configured by NG-RAN. A RNA update is triggered when the UE moves out of its notification area by receipt of a SIB1. An update may also be triggered periodically. In 3GPP TS 36.300, the ng-eNB handles access attempts with resume cause "highPriorityAccess" with high priority. However, the resume cause assigned to MPS-subscribed UEs during RNAU is “rna-Update”, not “highPriorityAccess”. As such, the RRCConnectionResumeRequest sent by the MPS-subscribed UE during the RNAU procedure may be dropped when the ng-eNB is congested. Furthermore, if the ng-eNB is congested and sends a RRCConnectionReject to the UE, the UE starts timer T302 (waitTime: 1-16 seconds) and is barred until T302 expires. Only access categories 0 and 2 (i.e., paging response and emergency) are exempt from UAC while timer T302 is running. However, access category 8 is used during RNAU. Therefore, all resume procedures in INACTIVE mode are blocked (except for paging response and emergency), including mobile origination and MPS resumes, until timer T302 expires. This issue is solved by adding an optional feature to support using the highPriorityAccess resume cause for RNAU for MPS subscribed UEs such that the ng-eNB does not reject the RNAU procedure triggered by an MPS-subscribed UE when congested and the MPS-subscribed UE does not need to start timer T302. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional feature to exempt MPS-subscribed UEs from congestion control measures at the ng-eNB during RNAU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPS-subscribed UEs will be blocked at the ng-eNB during RNAU if the ng-eNB is congested. A RRCConnectionReject will trigger the MPS-subscribed UE to block all resume procedures, except paging response and emergency, until timer T302 expires. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 5090 TS 38.306 CR 1244 TS 38.331 CR 5207 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 6.7.x	RAN-based Notification Area Update for high priority UEs

It is optional for a UE configured with Access Identities equal to 1, 2, or 11-15 to use a high priority access resume cause if the resumption of the RRC connection is triggered due to an RAN-based Notification Area Update, as specified in TS 36.331 [5].

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>