3GPP TSG-RAN WG2 Meeting #129bis	R2-2501799

Wuhan, China, April 2025

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discovery and relay (re)selection for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the discovery and relay (re)selection for multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

Multi-hop U2N Discovery

For the discovery transmission at the intermediate relay UE, whether Uu and PC5 threshold conditions should be defined was discussed.

For Uu threshold condition, in RAN2 #128, it was agreed that the lower bound of Uu RSRP is not needed and upper bound Uu RSRP can be configured by the network. And it was further agreed in RAN2 #129 that the cross-cell topology is not supported in the baseline procedure. The only left issue is the details of the upper bound of Uu RSRP.

Agreements: A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound.

For this FFS, we understand there are the following 3 options:

      - a new/separate parameter for acting as intermediate relay UE.

      - reuse the same upper bound parameter as acting as remote UE.

      - reuse the same lower bound parameter as acting as last relay UE.

For the 3 options, option 2 seems more reasonable and easier since:

-  it doesn’t require a new parameter.

- for L2 multi-hop U2N relay, the intermediate relay must receive paging and SI from the last relay, which means each intermediate relay UE functions simultaneously as a remote UE, making it straightforward to verify the same conditions as the remote UE.

- as shown in the following specification, the Uu threshold condition is used together with a hysteresis parameter, if reusing the lower bound of the relay UE, the redefinition of parameter is needed.

#### 5.8.14.2 NR sidelink U2N Relay UE threshold conditions A UE capable of NR sidelink U2N Relay UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRelay by hystMaxRelay if configured; and 2> if threshLowRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshLowRelay by hystMinRelay if configured: 3> consider the threshold conditions to be met (entry); 1> else: 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRelay if configured; or 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshLowRelay if configured; 3> consider the threshold conditions not to be met (leave); … #### 5.8.15.2 NR Sidelink U2N Remote UE threshold conditions A UE capable of NR sidelink U2N Remote UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or 2> if the UE has no serving cell: 3> consider the threshold conditions to be met (entry);

- Reusing the Uu RSRP upper bound of remote UE for the intermediate relay UE can: 1) save the effort for a new parameter; 2) be in line with the fact that intermediate relay UE for L2 case also acts as remote UE; 3) save the redefinition of the lower bound parameters for acting as U2N Relay UE.

For the upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, reuse the same upper bound of Uu RSRP parameter for acting as remote UE.

The next question is how for the UE to operate if the parameter is not configured. In R17 U2N Relay, if the related parameter is not configured, remote UE considers the condition to be met.

#### 5.8.15.2 NR Sidelink U2N Remote UE threshold conditions A UE capable of NR sidelink U2N Remote UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or 2> if the UE has no serving cell: 3> consider the threshold conditions to be met (entry);

In R17, if threshHighRemote is not configured, the remote UE considers the threshold conditions to be met.

By following R17 principle and the agreement in the last RAN2 meeting, the UE capable of U2N intermediate Relay UE operation and cannot act as last relay UE/ U2N Relay UE considers the threshold condition to be met if the Uu RSRP upper bound is not configured in the serving cell.

If upper bound of Uu RSRP is not configured by the network, the UE acting as U2N intermediate Relay UE operation considers the threshold condition to be met.

Then for the PC5 threshold condition, RAN2 agree no PC5 threshold condition is needed for Model-A and Model-B when the PC5 link has been established. And FFS for the case when the PC5 link between the intermediate relay and its parent is not established in Model-B.

Agreements:

For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient).

For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.  FFS if the PC5 link is not already established.

For L2 Relay case, a UE always needs to find a cell for camping (i.e., paging/SIB reception). For an UE who can act as intermediate relay which is out of coverage, no matter it is acting as intermediate relay (with remote UE connected) or not yet, it needs to rely on a relay link for SI/Paging/data reception/transmission, which means it firstly acts as a remote UE.

For L2 Multi-hop U2N Relay, the intermediate relay UE first acts as a remote for SI/Paging/data reception/transmission.

Therefore, the UE capable of acting as an intermediate relay firstly acts as a remote UE, and it has already evaluated the link quality towards the parent node and decided whether to select/connect to the parent UE before discovery message forwarding for the child remote UE.

The intermediate relay UE, before forwarding the discovery message for the remote UE has already evaluated the link quality towards the parent node via relay (re)selection procedure.

In other words, in L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves the remote UE no matter for Model-B or Model-A. Which means the FFS case is not reasonable for L2 multi-hop U2N Relay.

In L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves a remote UE regardless of Model-B or Model-A. Which means the FFS case (PC5 link not established) is not a reasonable case for L2 multi-hop U2N Relay.

Since the link quality is evaluated by the intermediate relay UE via relay (re)selection criteria, no additional discovery transmission condition is needed. If the link quality is bad, the intermediate relay UE should trigger relay reselection procedure.

In addition, the discovery type (e.g., whether it is Model-A or Model-B) have no impact from AS layer perspective, they are both carried via SL-SRB4. So it is preferred to have a unified approach for Model-A and Model-B as in R17 single-hop U2N Relay.

In R17 single-hop U2N Relay, there is no different handling of Model-A and Model-B discovery, i.e., both are carried via SL-SRB4.

RAN2 to discuss for L2 multi-hop U2N Relay, no PC5 RSRP threshold condition is needed for the discovery response message forwarding at the intermediate relay UE. Intermediate relay UE checks the quality of PC5 link on which the discovery response message is received based on relay (re)selection criteria.

For the PC5 link between the remote UE and the intermediate relay UE, it can be evaluated by the remote UE based on relay (re)selection criteria.

The PC5 link quality between the remote UE and the intermediate relay UE is checked by the remote UE based on relay (re)selection criteria.

Therefore, there is no need for a double evaluation of the PC5 link quality at the intermediate relay upon Model-B solicitation message reception.

For Model-B Solicitation message forwarding by the intermediate relay, no PC5 threshold configuration is needed.

There is also a discussion about whether the last relay needs to check the SD-RSRP of the received Model-B solicitation message to decide whether to respond. As discussed in Proposal 3, the link quality between the intermediate relay and the last relay is evaluated by the intermediate relay via relay (re)selection, i.e., if the link quality between the intermediate relay and the last relay is bad, the intermediate relay should perform relay reselection to another last relay. Therefore, as in single-hop U2N Relay, there is no PC5 condition to be checked at the last relay UE, i.e., the PC5 threshold configuration is not needed for Model-B response at the last relay UE.

For Model-B response transmission by the last relay, no PC5 threshold configuration is needed.

Relay (re)selection

For the relay (re)selection trigger condition, the following agreements are made in RAN2 #129 meeting:

Agreements:

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current first relay UE.

-	When U2N Remote UE detects PC5 RLF with the current first relay UE.

-	Indicated by upper layer.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

-	FFS: link quality degradation of the upstream links resulting in a notification.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS applicability to L3.

The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE.

Upon intermediate relay UE performing relay (re)selection, it can notify its child node.  If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

FFS if the notification is required in all cases

FFS the content in the notification

FFS the child node behaviour after receiving this notification

FFS other condition/timing that the intermediate relay UE notifies its child node

The first FFS point is on the notification message triggered relay reselection at the remote UE. On whether the link quality degradation results in a notification, it is not needed since we already have the link quality degradation handling mechanism:

- For RRC_IDLE/RRC_INACTIVE case: it will trigger relay reselection by the intermediate relay, and thus the notification message upon relay (re)selection would be sufficient

- For RRC_CONNECTED case: up to network to handle based on MR by first relay UE upon event X2/X1/Z1

The link quality degradation can be handled based on existing relay (re)selection and measurement report procedure, no need for the new notification indication.

Not pursue the link quality degradation triggered notification in multi-hop U2N Relay.

For the recovery by the intermediate relay UE, there is no link failure recovery mechanism defined, which means the intermediate relay has to change the upstream relay link and a path switching/mobility happens at the intermediate relay UE.

When the failure happens, there is no mechanism defined to recover the current link, the intermediate relay has to change the upstream relay link which means a path switching/mobility procedure at the intermediate relay UE.

For the path switching at the intermediate relay, the link-related information of the source link and target link may be different, which impacts the connected child UE. Here, the link-related information includes both AS and upper layer information related to relay (re)selection criterion, e.g., the cell ID, hop number, QoS information … The link-related information change may make the target relay link not suitable for the child UEs.

The path switching at the intermediate relay may cause change of serving cell ID, and hop number, which may invalidate both AS/upper layer relay (re)selection criterion at the child UE.

Therefore, considering the above issue of not notifying the child UE and doing self-recovery, the self-recovery with path switching at the intermediate relay should not be supported.

Not pursue self-recovery without/before notifying the child node when relay reselection at the intermediate relay UE happens.

For the signalling contents of the notification message it depends on the control plane discussion.

How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure, and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

At last, for the applicability to the L3 case, in R17, the notification message can be used for both L2 and L3 U2N Relay except the notification triggered by the L2 U2N Relay UE’s RRC connection failure.

##### 5.8.9.10.2 Initiation The Relay UE may initiate the procedure when one of the following conditions is met: 1> if the UE is acting as U2N Relay UE: 2> upon Uu RLF as specified in 5.3.10; 2> upon reception of an RRCReconfiguration including the reconfigurationWithSync; 2> upon cell reselection; 2> upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5; … ##### 5.8.9.10.3 Actions related to transmission of NotificationMessageSidelink message The Relay UE shall set the indication type as follows: 1> if the UE is acting as U2N Relay UE: 2> if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF: 3> set the indicationType as relayUE-Uu-RLF; 2> else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync: 3> set the indicationType as relayUE-HO; 2> else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection:

3> set the indicationType as relayUE-CellReselection; 2> if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure: 3> set the indicationType as relayUE-Uu-RRC-Failure; 2> submit the NotificationMessageSidelink message to lower layers for transmission. ##### … ##### 5.8.9.10.4 Actions related to reception of NotificationMessageSidelink message Upon receiving the NotificationMessageSidelink, the Remote UE shall: 1> if the UE is acting as U2N Remote UE: 2> if the indicationType is included: 3> if the UE is L2 U2N Remote UE in RRC_CONNECTED: 4> if MP is configured and MCG transmission (i.e. direct path) is not suspended; 5> if the indicationType is relayUE-HO; 6> suspend indirect path transmission; 5> else: 6> initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure; 4> else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7; 3> else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE): 4> if the PC5-RRC connection with the U2N Relay UE is determined to be released: 5> indicate upper layers to trigger PC5 unicast link release; 4> else (i.e., maintain the PC5 RRC connection): 5> if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection: 6> consider cell re-selection occurs; ##### NOTE 1: For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

In R17 U2N, Notification message can be used for both L2 and L3 U2N Relay, except for the notification triggered by the RRC connection failure of the L2 U2N Relay UE.

As in R17, Notification message can also be applied for L3 multi-hop U2N Relay, except for the notification triggered by the RRC connection failure of the intermediate relay UE.

For the FFS related to relay (re)selection at the intermediate relay UE, the concern is on how/whether the intermediate relay notifies its child node.

The relay reselection procedure includes the following operations:

- Intermediate relay detects link quality degradation or failure and triggers relay reselection.

- Intermediate relay releases the link towards the source serving relay.

- Intermediate relay completes the link establishment towards the target relay.

The relay reselection procedure at the intermediate relay includes 3 operations:

- fulfil relay reselection trigger condition.

- release the source link.

- setup the new link.

As the relay reselection at the intermediate relay UE is a path switching/mobility procedure, same as the discussion in Observation-9/10, it should notify the child nodes to avoid group mobility.

The path switching at the intermediate relay needs to be indicated to the child UEs in all cases to avoid group mobility.

Regarding the content of the notification message or whether a new cause is needed, since no matter what triggers the relay reselection, the PC5 connection towards the source relay will be released, the simplest way would be to reuse the legacy cause value used in R18 U2U Relay for the PC5 connection release (relayUE-PC5-RLF).

No matter what triggers relay reselection at the intermediate relay, the PC5 link towards the source parent relay is to be released.

For the child node behavior after receiving this notification, if the child node is a remote UE, the behavior should be the same as R17. If the child node is another intermediate relay UE, the behavior is pending control plane discussion, i.e., whether it first process the received notification message or it forward directly to the child node.

For the child node behavior regarding notification message reception, if the child UE is a remote UE, the behavior was agreed in RAN2 #129 meeting. If the child node is an intermediate relay UE, the behavior is pending control plane discussion.

At last, for the other condition/timing that the intermediate relay notifies its child node, it is not clear on what other condition/timing, so the FFS can be removed unless a necessary condition is identified.

In all relay reselection scenarios, upon PC5 link release with the source parent relay node due to relay reselection at the intermediate relay UE, it notifies its child node on the release of the PC5 link with its parent relay using the legacy cause value (i.e., relayUE-PC5-RLF).

Conclusion

We have the following observations:

Observation 1	Reusing the Uu RSRP upper bound of remote UE for the intermediate relay UE can: 1) save the effort for a new parameter; 2) be in line with the fact that intermediate relay UE for L2 case also acts as remote UE; 3) save the redefinition of the lower bound parameters for acting as U2N Relay UE.

Observation 2	In R17, if threshHighRemote is not configured, the remote UE considers the threshold conditions to be met.

Observation 3	For L2 Multi-hop U2N Relay, the intermediate relay UE first acts as a remote for SI/Paging/data reception/transmission.

Observation 4	The intermediate relay UE, before forwarding the discovery message for the remote UE has already evaluated the link quality towards the parent node via relay (re)selection procedure.

Observation 5	In L2 muti-hop U2N Relay, a UE should always have an available link towards the network before it serves a remote UE regardless of Model-B or Model-A. Which means the FFS case (PC5 link not established) is not a reasonable case for L2 multi-hop U2N Relay.

Observation 6	In R17 single-hop U2N Relay, there is no different handling of Model-A and Model-B discovery, i.e., both are carried via SL-SRB4.

Observation 7	The PC5 link quality between the remote UE and the intermediate relay UE is checked by the remote UE based on relay (re)selection criteria.

Observation 8	The link quality degradation can be handled based on existing relay (re)selection and measurement report procedure, no need for the new notification indication.

Observation 9	When the failure happens, there is no mechanism defined to recover the current link, the intermediate relay has to change the upstream relay link which means a path switching/mobility procedure at the intermediate relay UE.

Observation 10	The path switching at the intermediate relay may cause change of serving cell ID, and hop number, which may invalidate both AS/upper layer relay (re)selection criterion at the child UE.

Observation 11	How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure, and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

Observation 12	In R17 U2N, Notification message can be used for both L2 and L3 U2N Relay, except for the notification triggered by the RRC connection failure of the L2 U2N Relay UE.

Observation 13	The relay reselection procedure at the intermediate relay includes 3 operations:

-	fulfil relay reselection trigger condition.

-	release the source link.

-	setup the new link.

Observation 14	The path switching at the intermediate relay needs to be indicated to the child UEs in all cases to avoid group mobility.

Observation 15	No matter what triggers relay reselection at the intermediate relay, the PC5 link towards the source parent relay is to be released.

Observation 16	For the child node behavior regarding notification message reception, if the child UE is a remote UE, the behavior was agreed in RAN2 #129 meeting. If the child node is an intermediate relay UE, the behavior is pending control plane discussion.

We have the following proposals:

Proposal 1	For the upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, reuse the same upper bound of Uu RSRP parameter for acting as remote UE.

Proposal 2	If upper bound of Uu RSRP is not configured by the network, the UE acting as U2N intermediate Relay UE operation considers the threshold condition to be met.

Proposal 3	RAN2 to discuss for L2 multi-hop U2N Relay, no PC5 RSRP threshold condition is needed for the discovery response message forwarding at the intermediate relay UE. Intermediate relay UE checks the quality of PC5 link on which the discovery response message is received based on relay (re)selection criteria.

Proposal 4	For Model-B Solicitation message forwarding by the intermediate relay, no PC5 threshold configuration is needed.

Proposal 5	For Model-B response transmission by the last relay, no PC5 threshold configuration is needed.

Proposal 6	Not pursue the link quality degradation triggered notification in multi-hop U2N Relay.

Proposal 7	Not pursue self-recovery without/before notifying the child node when relay reselection at the intermediate relay UE happens.

Proposal 8	As in R17, Notification message can also be applied for L3 multi-hop U2N Relay, except for the notification triggered by the RRC connection failure of the intermediate relay UE.

Proposal 9	In all relay reselection scenarios, upon PC5 link release with the source parent relay node due to relay reselection at the intermediate relay UE, it notifies its child node on the release of the PC5 link with its parent relay using the legacy cause value (i.e., relayUE-PC5-RLF).