3GPP TSG-RAN WG2 Meeting #129bis	R2-2501805

Wuhan, China, April 8th –11th , 2025

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Discussion on on-demand SSB

Document for:	Discussion and Decision

1	Introduction

In RAN2#127bis meeting, we had some further discussion on OD-SSB and achieved the following agreement.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we would like to further discuss the procedures and signalling to support on-demand SSB and have corresponding observations and proposals.

2	Discussion

2.1 Activation/deactivation of OD-SSB

2.1.1 RRC based activation/deactivation

Also according to RAN1 discussion, deactivation of OD-SSB transmission is supported and there are the following alternatives to indicate the deactivation of OD-SSB transmission explicitly or implicitly.

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Regarding whether to support RRC based deactivation, with the following agreement we achieved in previous meeting, deactivation of OD-SSB transmission of the cell can be realized via reconfiguration of OD-SSB activation/deactivation state indication, with no additional specification impact.

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

Proposal 1: Deactivation of the OD-SSB transmission of the SCell by RRC can be supported by reconfiguration of the OD-SSB activation/deactivation state indication. No special specification effort is required.

2.1.2 MAC CE based activation/deactivation

According to RAN1 agreement, for OD-SSB SCell, RRC configures the following parameters

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

RAN1 has already agreed to indicate the applicable periodicity with MAC CE, in addition, for implicit deactivation, the indication of the number N of on-demand SSB bursts should be indicated as well. Therefore, at least periodicity and number N are included in MAC CE. Since it is not sure if any other parameters will be indicated by MAC CE, it is better to introduce an ID which can be used to associate with a set of applicable parameters.

Proposal 2: Define a configuration ID associated with at least periodicity and number N of OD-SSB bursts/timer of OD-SSB transmission.

Similar as enhanced SCell activation/deactivation MAC CE, where for each activated SCell, there is an associated TRS ID, for each SCell of which OD-SSB is indicated as activated, a configuration ID should be included in the MAC CE.

Proposal 3: For each SCell with OD-SSB transmission activated, include the configuration ID in MAC CE.

In previous RAN2 meeting, we agreed to indicate the OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

One alternative is to use single bitmap to indicate both the activation and deactivation of OD-SSB. However, since RAN1 agreed to support both explicit deactivation and implicit deactivation. For SCells of which OD-SSB is activated but relying on implicit deactivation, the number “N”/configuration ID should be indicated. When explicitly deactivating the OD-SSB of some SCells, the indications related to the OD-SSB activated SCells need to be kept as “activated”. In addition, the number “N”/configuration ID associated with the OD-SSB activated SCell needs to be contained as well. However, this number “N”/configuration ID needs to be updated for the OD-SSB SCells which is “re-activated”, since SSB has transmitted several bursts since when UE last time receives the activation/deactivation MAC CE. Take an example as below.

At T1, one OD-SSB A/D MAC CE is to activate OD-SSB transmission on SCell1, SCell2 and SCell3 with each SCell associated with a number N of bursts to support implicit deactivation, i.e., N1, N2 and N3 for each SCell. At T2, another OD-SSB A/D MAC CE is to explicitly deactivate the OD-SSB transmission on SCell1, however, the bits related to SCell2 and SCell3 should be kept as “activated” and the number N of bursts associated with SCell2 and SCell3 needs to be indicated as well. However, compared to that contained in T1, the number N of bursts associated with SCell2 and SCell3 i.e., N2’ and N3’ in T2 needs to be updated, otherwise, if the same value as N2 and N3 are used, UE may misunderstand the OD-SSB transmission on SCell2 and SCell3 are extended. Therefore, a single bit map to indicate both activation and deactivation cause some issue on the interpretation of the associated parameters for the SCells of which OD-SSB is “re-activated”.

Observation 1: A single bit map to indicate both activation and deactivation causes ambiguity on the interpretation of the associated parameters for the SCells of which OD-SSB is “re-activated”.

To solve this issue, we think the following alternatives can be considered.

- Alternative1: separate MAC CEs for OD-SSB activation and deactivation.

For this alternative, two MAC CEs will be defined separately. To support implicit deactivation, number N is associated with a SCell when the OD-SSB is activated and NW can use the deactivation MAC CE to deactivate a SCell explicitly. In this case, separate MAC CEs are used for implicit deactivation and explicit deactivation, thus solve the issue mentioned above. For the activation MAC CE, the legacy SCell activation/deactivation MAC CE format can be reused, i.e., bitmap and associated parameters. Each bit within the bitmap with the value “1” indicating to activate the OD-SSB of the corresponding SCell, while UE just ignores the bit with the value “0”. For the deactivation MAC CE, single bitmap is sufficient. Each bit within the bitmap with the value “1” indicating to deactivate the OD-SSB of the corresponding SCell, while UE just ignores the bit with the value “0”. One example with serving cell index less than 8 is shown below.

OD-SSB activation MAC CE

OD-SSB deactivation MAC CE

- Alternative2: single MAC CE for OD-SSB activation and deactivation with two bitmaps

For this alternative, single MAC CE with two bitmaps will be defined. The first bitmap is used to indicate the SCells should apply the MAC CE and the second bitmap is used to indicate the activation/deactivation status of the SCell. If the corresponding bit in the first bitmap is set to “0”, UE just ignores the MAC CE, while if the corresponding bit in the first bitmap is set to “1”, UE further checks the second bitmap to determine the activation/deactivation status of the OD-SSB of each SCell. One example with serving cell index less than 8 is shown below.

- Alternative3.1: NW ensures either implicit deactivation or explicit deactivation is used for all OD-SSB SCells

For this alternative, single MAC CE with one bitmap, e.g., similar as legacy SCell activation/deactivation MAC CE will be defined. However, it is up to NW implementation to ensure either implicit deactivation or explicit deactivation is used for all OD-SSB SCells. In this case, if NW chooses implicit deactivation, number N is associated with each SCell of which OD-SSB is indicated as activated, afterwards, it is not allowed to use this MAC CE for explicit deactivation, e.g., it is not allowed to change the corresponding bit from 1 to 0. If NW chooses explicit deactivation, number N is not associated with the SCells of which OD-SSB is activated, UE considers the OD-SSB as activated until reception of deactivation indication.

- Alternative3.2: NW updates the associated number N for the SCells of which OD-SSB is “re-activated”

For this alternative, single MAC CE with one bitmap, e.g., similar as legacy SCell activation/deactivation MAC CE will be defined. However, it is up to NW implementation to update the associated number N for the SCells of which OD-SSB is “re-activated”. From the UE’s perspective, it always applies the latest information carried in the MAC CE.

We think all the above alternatives can work. Each alternative has pros and cons. For alternative1, two MAC CEs need to be defined and the definition of each MAC CE needs to be specified. For alternative2, two bitmaps need to be defined and the relationship between these two bitmaps need to be specified, e.g., second bitmap should be skipped if the corresponding bit in the first bitmap is set to “0”. For alternative3, there is strong limitation on the usage of the MAC CE, i.e., implicit deactivation and explicit deactivation cannot be supported simultaneously, which is not aligned with RAN1 agreement and motivation. For alternative4, this will introduce additional complexity on NW to remember the number of SSB transmission burst each time after the MAC CE is transmitted. Since the MAC CE is UE-specific, NW should remember this information for each UE, which causes significant complexity. Based on the above analysis, we suggest RAN2 to discuss and further down select among the alternatives listed above.

Proposal 4: For OD-SSB activation and deactivation, RAN2 to discuss and further down select among the following options:

- Alternative1: separate MAC CEs for OD-SSB activation and deactivation.

- Alternative2: single MAC CE for OD-SSB activation and deactivation with two bitmaps

- Alternative3: single MAC CE for OD-SSB activation and deactivation with one bitmap (similar as legacy SCell activation/deactivation MAC CE)

  - Alternative3.1: NW ensures either implicit deactivation or explicit deactivation is used for all OD-SSB SCells

  - Alternative3.2: NW updates the associated number N for the SCells of which OD-SSB is “re-activated”

2.2 RRC impact

No matter of which scenario and which trigger to be supported, UE should be informed of whether a SCell is a normal SCell or on-demand SSB SCell. This is because without this information, UE may derive incorrect measurement results with the assumption that the SCell is normal SCell and SSB is transmitted. It may cause problems on RRM procedure as the accuracy of measurement may be significantly degraded. Therefore, we think upon SCell addition, NW should indicates whether the SSB transmission on the SCell is on-demand manner.

Proposal 5: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

2.3 L3 RRM measurement

2.3.1 Impact on SMTC

In last RAN2 meeting, we discussed the impact on RRM measurement for SSB adaptation and achieved the following agreement.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

Regarding the impact on RRM measurement for OD-SSB, we prefer to adopt unified solution as SSB adaptation to simplify the workload of both RAN2 and RAN4. Therefore, for OD-SSB, additional SMCT configurations are introduced according to the different OD-SSB periodicities. When performing RRM measurement, UE adjusts the SMCT accordingly based on the indicated OD-SSB periodicity.

Proposal 6: For L3 measurement, unified solution as SSB adaptation is adopted for OD-SSB, e.g., additional SMTC configurations are introduced according to different OD-SSB periodicities.

2.3.2 Located frequency of OD-SSB

According to the agreement from RAN1, see below. At least when the AO-SSB is CD-SSB and on a sync raster, the frequency of AO-SSB and OD-SSB are different.

Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

According to the RRC specification, for SSB based measurement, there is at most one measurement object and only one SSB frequency is configured per MO to derive the cell level measurement result. Since the located frequency of AO-SSB and OD-SSB is different, one alternative is to define separate MO for AO-SSB and OD-SSB which are both associated with the same cell. Another alternative is to add additional frequency to the original serving cell MO. We suggest RAN2 to discuss the impact and make down selection between the alternatives.

Proposal 7: When AO-SSB and OD-SSB are on different frequencies, RAN2 to consider

- Alternative1: separate MO for AO-SSB and OD-SSB which are both associated with the same cell.

- Alternative2: add additional frequency to the original serving cell MO.

Regarding the impact on the measurement procedure, considering there are still some open issues under RAN1/RAN4 discussion, e.g.,

- RAN1 is still FFS whether AO-SSB and OD-SSB are on the same frequency or different frequencies when AO-SSB is CD-SSB but not on a sync raster

- RAN4 is still discussing whether AO-SSB and/or OD-SSB are used for measurement when both are transmitted in case 2.

It is better to wait for more RAN1/RAN4 progress before we start the discussion on the impact of measurement procedure.

Proposal 8: Wait for more RAN1/RAN4 progress before we start the discussion on the impact of measurement procedure for OD-SSB case 2.

2.3.3 Measurement report

In previous meeting, there were proposals to trigger measurement report when OD-SSB is activated. The motivation is to quickly measure and report the latest measurement results when OD-SSB is activated, which can change the SCell from unknown cell to known cell to accelerate the activation of SCell.

However, we think for scenario#2, when to trigger SCell activation is up to gNB, even a measurement report is triggered when OD-SSB is activated, it cannot ensure the SCell is always known, e.g., the duration between the triggered measurement report and the SCell activation may exceed the limitation. Therefore, this optimization is not needed for scenario#2. While for Scenario#2A, we think legacy principle can be reused here e.g., after receiving the SCell activation command, if UE has valid L3 measurement results available but not reported on the unkown SCell, the L3 measurement report will be initiated. Nothing new for OD-SSB case 1 and case 2.

Proposal 9: No need to trigger measurement report when OD-SSB is activated for Scenario#2.

2.3.4 Neighbour cell measurement

For neighbour cells measurements, we think according to the WID, the impact on neighbour cell measurement with OD-SSB is out of scope. From RAN2 perspective, there is no need to discuss this aspect and we can assume the SSB status of neighbour cell is transparent to UE and how to handle the reported neighbour cell measurement is up to gNB implementation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Proposal 10: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

3	Conclusion

In this contribution, we discussed the procedures and signalling to support on-demand SSB and have corresponding observations and proposals:

RRC based activation/deactivation

Proposal 1: Deactivation of the OD-SSB transmission of the SCell by RRC can be supported by reconfiguration of the OD-SSB activation/deactivation state indication. No special specification effort is required.

MAC CE based activation/deactivation

Proposal 2: Define a configuration ID associated with at least periodicity and number N of OD-SSB bursts/timer of OD-SSB transmission.

Proposal 3: For each SCell with OD-SSB transmission activated, include the configuration ID in MAC CE.

Observation 1: A single bit map to indicate both activation and deactivation causes ambiguity on the interpretation of the associated parameters for the SCells of which OD-SSB is “re-activated”.

Proposal 4: For OD-SSB activation and deactivation, RAN2 to discuss and further down select among the following options:

- Alternative1: separate MAC CEs for OD-SSB activation and deactivation.

- Alternative2: single MAC CE for OD-SSB activation and deactivation with two bitmaps

- Alternative3: single MAC CE for OD-SSB activation and deactivation with one bitmap (similar as legacy SCell activation/deactivation MAC CE)

  - Alternative3.1: NW ensures either implicit deactivation or explicit deactivation is used for all OD-SSB SCells

  - Alternative3.2: NW updates the associated number N for the SCells of which OD-SSB is “re-activated”

RRC impact

Proposal 5: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

L3 RRM measurement

Proposal 6: For L3 measurement, unified solution as SSB adaptation is adopted for OD-SSB, e.g., additional SMTC configurations are introduced according to different OD-SSB periodicities.

Proposal 7: When AO-SSB and OD-SSB are on different frequencies, RAN2 to consider

- Alternative1: separate MO for AO-SSB and OD-SSB which are both associated with the same cell.

- Alternative2: add additional frequency to the original serving cell MO.

Proposal 8: Wait for more RAN1/RAN4 progress before we start the discussion on the impact of measurement procedure for OD-SSB case 2.

Proposal 9: No need to trigger measurement report when OD-SSB is activated for Scenario#2.

Proposal 10: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

4	Reference

[1] RAN1#118bis Chair notes.

[2] RAN2#127bis meeting minutes

[3] R4-2416911

[4] R4-2416913