3GPP TSG RAN WG2 #129bis                                                                            R2-2501919

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT

Title:	MHI Enhancement for SCG Activation and Deactivation

Agenda Item:	8.10.5

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, MHI enhancement for SCG activation and deactivation was discussed and the following agreements were achieved [1].

We will introduce a UE based solution for SCG act/deact MHI, as an optional UE feature. UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

In this contribution, we will continue to discuss the details about how to log the time the UE has spent in PSCell with SCG activated.

Discussion

In last meeting, RAN2 has agreed that a UE based solution for SCG activation/deactivation is supported and UE reports the time the UE has spent in PSCell with SCG activated in MHI, FFS whether it is percentage of time or absolute time. Considering that reporting the absolute time can provide more precise time information to network than reporting the percentage of time, thus, we prefer UE reporting the absolute time the UE has spent in PSCell with SCG activated in MHI.

Proposal 1: UE reports the absolute time the UE has spent in PSCell with SCG activated in MHI.

Another issue needs to be considered is how to log the time information in case of the SCG is activated/deactivated multiple times when connected to the SCG, an example in Figure 1 below.

Figure 1 SCG activation/deactivation when UE connected to the SCG

In Figure 1, the SCG activation/deactivation when UE connected to the SCG is described as follow:

T1: UE accesses the SCG1 with SCG1 is activated;

T2/T4: SCG1 is deactivated due to no data transmission is needed on SCG1;

T3: SCG1 is activated due to data transmission is needed on SCG1;

Based on above, it can be seen that the time the UE has spent in SCG1 with SCG activated includes two periods of time i.e., T1~T2 and T3~T4. How to log the two periods of time in MHI needs to be discussed. The following two options can be considered:

Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

For Option 1, it is a straight way to give all the details on SCG activation/deactivation procedure, but it will increase UE logging complexity. For Option 2, it is a simpler way, but the network cannot get the detailed information about SCG activation/deactivation. From our perspective, both the two options can be accepted.

Proposal 2: RAN2 to discuss the following two options about logging the SCG activation/deactivation time for a PSCell in MHI:

- Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

- Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

Conclusion

According to the discussion in section 2, the proposals are summarized as follows:

Proposal 1: UE reports the absolute time the UE has spent in PSCell with SCG activated in MHI.

Proposal 2: RAN2 to discuss the following two options about logging the SCG activation/deactivation time for a PSCell in MHI:

- Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

- Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

Reference

- R2-2501702, RAN2#129 Meeting Report