3GPP TSG-RAN WG2 Meeting #129bis											R2-2501946

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.7.4.1

Source: 	Sharp

Title:  	Discussion on LCP Enhancements

Document for: 	Discussion

# Introduction

In RAN2#129 meeting, RAN2 made the following agreements on LCP enhancement:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

This document discusses remaining open issues.

# Discussion

## SRB Impact of Additional Priority

It may be possible that LCH with LCH priority-adjusted data prioritizes data from SRB when the additional priority is higher than that of SRB. In this case, SRB transmission can be delayed until other DRBs finish all pending transmissions.

However, it is not a big issue or can be resolved by proper priority configuration by the network. The default priority of SRB1 is 1 which is the highest priority. Even if an additional priority for other logical channel is configured with a very high priority value, it is not higher priority than that of SRB1. Thus, SRB1 will not experience additional transmission delay at all. Other SRBs could experience some delayed transmission. But it is possible even without additional priority that a logical channel for a DRB is configured with a very high priority value. If the network thinks it has a big problem, the situation can be avoided by not configuring high additional priority.

The impact of additional priority to SRB can be handled by network implementation.

## Intra-UE Prioritization

In RAN2#129 meeting, RAN2 agreed that intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. An open issue is for which time point LCH priority is used. As the legacy MAC procedure assumes the static LCH priority, the current specification texts are ambiguous in case of priority adjustment by additional priority.

Figure 1. Priority issue in LCH-based prioritization

Figure 1 shows the issue on whether the additional LCH priority is used. The example assumes LCH1 has priority value of 10 as the normal priority and priority value of 3 as the additional LCH priority. When there is LCH priority-adjusted data whose discard timer expiry time is below the threshold, LCP is performed with priority value of 3. Since there is a time delay until the actual transmission, it is not clear which priority value is used for priority of the uplink grant. From data usage point of view, additional priority which was used during LCP procedure needs to be used to prioritize the uplink grant carrying LCH priority-adjusted data. However, from LCP point of view, all LCH priority-adjusted data has been included in MAC PDUs, the priority may fall back to the normal priority. Then, both the initial transmission and retransmission have the same UL grant priority.

For UL grant priority determination, the priority used at LCP procedure of the uplink grant is used.

An FFS point is whether the additional LCP priority is used for SR priority determination in intra-UE prioritization. Scenarios of determining SR priority is a resource overlap between SR and UL-SCH resource. In RAN2#129 meeting, RAN2 agreed to use the additional LCH priority for determining the priority of an UL grant. In case of resource overlap between SR and UL-SCH resource, it is reasonable and fair that both SR and UL-SCH resource (i.e., UL grant) uses the same rules. Thus, we see that the additional LCP priority should be used for SR priority determination.

The additional LCP priority is used for SR priority determination.

## Data Usage for Negative Bj

In RAN2#129 meeting, whether an LCH with an upgraded priority and negative Bj value can participate in the first round of LCP. In LCP procedure, negative Bj value could happen when the logical channel is having sufficient resources set by PBR and BSD from the requirement and QoS point of view. The reason why the legacy LCP procedure does not allow LCH with negative Bj to participate in the first round of LCP is to avoid starvation of lower priority LCHs and guarantee minimum fairness. Hence, we need to be very careful to allow any exceptional case.

An LCH with an upgraded priority does not participate in the first round of LCP if Bj is negative.

## Joint Configurations of DSR and Additional Priority

- In RAN2#127bis, RAN2 agreed not to introduce any setting restrictions of the new remaining time threshold for additional priority with relation to DSR triggering threshold remainingTimeThreshold. But there was no discussion/conclusion on whether there is any restriction on the configuration of additional priority and remainingTimeThreshold for DSR. In other words, it is not clear whether additionall priority and remainingTimeThreshold for DSR can be configured independently or shall be jointed configured.

- In RAN2#129 meeting, the impact of additional priority on BSR was discussed, and it was not agreed because we can rely on DSR. That means that a common understanding is that additional priority is configured only if remainingTimeThreshold for DSR is configured.

Additional priority is configured only if remainingTimeThreshold for DSR is configured.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- The impact of additional priority to SRB can be handled by network implementation.

- For UL grant priority determination, the priority used at LCP procedure of the uplink grant is used.

- The additional LCP priority is used for SR priority determination.

- An LCH with an upgraded priority does not participate in the first round of LCP if Bj is negative.

- Additional priority is configured only if remainingTimeThreshold for DSR is configured.