3GPP TSG-RAN WG2 Meeting #129bis											R2-2501949

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.7.6

Source: 	Sharp

Title:  	Discussion on Rate Control in XR

Document for: 	Discussion

# Introduction

In RAN2#129 meeting, RAN2 made the following agreements:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

This document discusses the remaining issues of XR rate control.

# Discussion

The XR MAC running CR [1] has the following Editor’s Notes:

Editor’s Note: FFS whether a new rate control MAC CE is needed. Editor’s Note: FFS granularity of the bit rate indicated in the MAC CE (e.g. either for a logical channel or a QoS flow in a logical channel.

In RAN2#129 meeting, RAN2 agreed to introduce a new table for XR rate control. A reply LS from SA4 [2] mentions that granularity of uplink bit rate can be as small as 1% depending on algorithms and 1% is generally preferred. The size of Bit Rate field in the Rel-15 MAC CE is 6 bits which support 56 steps of bit rate. This cannot meet 1% granularity, so bit rate field can be extended to 7 bits or 8 bits. This means that the Rel-15 MAC CE cannot be reused, so RAN2 should define a new MAC CE format.

Rel-19 UL Rate Control MAC CE is introduced.

In Rel-19 UL Rate Control MAC CE, the size of bit rate field is 7 bits or 8 bits.

The SA4 LS indicated that a value range covers from 100kbps to 40Mbps, so we can support it.

Bit rate field supports the value range from 100kbps to 40Mbps.

There is another Editor’s Note on bit rate, as follows:

Editor’s Note: FFS whether this bit rate is physical-layer bit rate (as in the legacy recommended bit rate procedure) or some other type of bit rate.

Rel-15 recommended bit rate procedure uses physical-layer bit rate. The reasons for it are that the gNB may not be fully aware of the application layer code rate, and the UE’s AS layer does not need to interact with the application layer on the preferred codec rate and application status. We see that the legacy principle can be reused unless any critical issue is found.

Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

Another aspect is the distribution of values of the bit rate field. The SA4 LS indicated 1% granularity is preferred. The preferred granularity is expressed as a percentage, not exact amount of difference of the bit rate. It means that similar to the Rel-15 MAC CE, an exponential distribution is required.

The value table of bit rate field uses an exponential distribution.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- Rel-19 UL Rate Control MAC CE is introduced.

- In Rel-19 UL Rate Control MAC CE, the size of bit rate field is 7 bits or 8 bits.

- Bit rate field supports the value range from 100kbps to 40Mbps.

- Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

- The value table of bit rate field uses an exponential distribution.

# Reference

[1] R2-2501761	Introduction to R19 XR enhancements, Qualcomm Incorporated.

[2] S4-242182	Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, SA4