3GPP TSG-RAN WG2 Meeting #129bis	R2-2502046

Wuhan, China, 7th April – 11th April 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN impacts for IoT-NTN SF Operations

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following are the agreements in RAN2 on the SF operation for IoT-NTN in RAN2-128 and RAN2-129.

RAN2-128 Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable.

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

RAN2-129 Agreements: 1. RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change 2. The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) 3. Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). 4. A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In this paper we analyse the remaining open issues for SF operation related to mode transition and access /paging procedure impacts in SF operation.

# 2	Discussion

## 2.1 Remaining Issues related to SF Operation indication

When SF mode parameter is configured in the IoT-NTN cell, it is up to UE implementation to stay in the current cell or start reselection for neighbouring cell (either TN or normal cell) based on the priority of the application data. This UE behaviour needs not be captured in RAN2 specification. In the similar lines introduction of parameters in neighbour-cell configuration on whether neigbour cell is operating in SF mode or not is not needed.

Proposal 1: Cell reselection when SF mode is active in current cell is up to UE implementation. No specification changes needed to enhance the cell reselection procedure for SF mode operation.

In RAN2-129, there were views expressed on reusing the T-service parameter introduced for indication of cell-switch time of quasi earth fixed cells also for SF mode transition.  However, SF mode transition is linked to feeder link connectivity only and the service towards UE remain even when the mode changes. The timer for this transition is completely different from the purpose of T-service. Hence we propose to keep the timer information related to SF functionality independent of the T-service parameter.

Proposal 2: The timer for SF mode transition is provided as separate parameter without any re-use with T-Service parameter.

In RAN2-129 meeting it is agreed to introduce the remaining time for store-forward mode information in SIB31. The UE will acquire this information similar to other contents of SIB31 and no system information modification procedure is needed regarding change of this parameter. The UE behaviour on use of this time information requires further discussion.

This timer indicates the transition of SF mode to normal and this information is relevant for higher layers to decide on resuming the pending transmissions. Hence we suggest that this timer information along with the current  SF operation mode is indicated to higher layers. The higher layers can decide on further actions on whether to delay NAS signalling and connection setup procedure for real time services. This behavior is also applicable for Rel-19 UE that can read this information  irrespective of whether UE support SF mode operation or not.

Proposal 3: The UE (capable of SF operation) behaviour on SF mode and the timer status is decided by higher layers for UE in RRC-IDLE mode. RAN2 assumes no impact to AS layer operations.  FFS connected mode.

When the satellite operates in normal mode no additional indication is included in the system information as per current RAN2 agreements on the SF mode operation. On change of SF mode from normal to SF mode the gNB may need to include barring information in system information to avoid legacy UE camping to this cell. This update is optional and used by the NW if it wants to enable access barring for legacy UE. The SF capable UE will know the mode transition from SIB1 modification. This can be used to modify the NAS behaviour for new transactions. However, if the UE wants to start some real-time data transmissions which can be completed within the duration of normal mode operation is useful at the UE to defer such transactions that may lead to failure.  Hence, we propose that RAN2 needs to discuss the usefulness of the timer for switching from normal mode to SF mode based on application scenarios.

Proposal 4: RAN2 to discuss the timer (e.g., the remaining time) for normal mode for benefits of applications requiring normal mode’

Need for additional parameters in the system information to indicate the maximum expected delay for the services was proposed from companies during RAN2-127bis.  As the NAS level registration procedure for SF capable UE already includes information about wait timer for next cell access and also the list of satellite-ID for next access that can help the UE to estimate the time of storage this additional information is not essential. Moreover whether the response can be delivered for MO traffic in next access is not fully guaranteed and it depends on the storage and other UE traffic at the time of feeder link connectivity. So this information will not be accurate.

Proposal 5: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

## 2.2 Access Control and Paging Changes for SF operation

When NTN node is serving new area, it may need to handle the new data transmissions from the coverage area and also complete the data transmission in downlink that was received and stored from CN at the time of earlier connectivity. In such cases the access control and data transmission across these two variants of users need to be differentiated. Mainly to avoid the delivering the stored data becoming invalid or time-out at UE.  In the same way for the uplink transmissions depending on the storage available at NTN node to receive and buffer the packets the access control and scheduling of transmissions needs to be handled. This can be achieved by explicit indication of support for MO or MT traffic along with SF operation parameter in system information.

Proposal 6:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation.

When the mobile originated IoT data transmission that requires application level ack are sent to eNB operating in store and forward mode the UE may expect the corresponding downlink packets after connecting to the next eNB. In this case UE monitors the next gNB for receiving ACK and new MT packets if any stored in the eNB. The data packet available at this eNB may contain ACK for earlier MO traffic and MT traffic toward the UE. In some other cases, the next eNB may not have any downlink ACK for earlier transmission depending on its CN connection time. In different scenarios based on the network awareness of CN connectivity time and the actual stored downlink data to be delivered towards UE, it is possible to improve the paging monitoring towards UE. With additional indication on whether ENB have stored ACK for MO traffic can also be used by UE application to avoid early retransmissions of MO data.

Proposal 7:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

In addition to paging enhancements, it is also beneficial for the UE battery life to consider that a satellite may not have more MT data for the UE. The typical case is that if the feeder link is not available, the satellite has no more MT data for the UE if the satellite buffer is drained since the satellite will not receive further MT data from CN. Until the feeder link recovery, the UE will not benefit from continuing paging monitoring on the current cell, nor will it benefit from reselecting to and monitoring paging from other cells of the same satellite. Thus, RAN2 can consider whether Idle mode procedures can be enhanced when there is no more MT data to be delivered in S&F operation.

Proposal 8: RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

# 3	Conclusion

In this paper we analyse the radio signalling procedure impacts for store and forward operation. Following proposals are

made based on the analysis.

Remaining issues for SF Operation

Proposal 1: Cell reselection when SF mode is active in current cell is up to UE implementation. No specification changes needed to enhance the cell reselection procedure for SF mode operation.

Proposal 2: The timer for SF mode transition is provided as separate parameter without any re-use with T-Service parameter.

Proposal 3: The UE (capable of SF operation) behaviour on SF mode and the timer status is decided by higher layers for UE in RRC-IDLE mode. RAN2 assumes no impact to AS layer operations.  FFS connected mode.

Proposal 4: RAN2 to discuss the timer (e.g., the remaining time) for normal mode for benefits of applications requiring normal mode’

Proposal 5: Additional information related to maximum delay impact of SF operation is not broadcasted in system information. UE can deduce this information from other parameters related SF operation provided in NAS level procedures.

Access control and Paging for SF operation

Proposal 6:  RAN2 to consider inclusion of additional information for access control for MO or MT in SF operation.

Proposal 7:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 8: RAN2 to consider idle mode procedure enhancements, when there is no MT data to be delivered by a store & forward satellite.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements