3GPP TSG-RAN WG2 Meeting #129bis	R2-2502058

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.8.4

Source: 	CATT

Title: 	Further discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

Based on current progress and agreements [1], this contribution further discusses the UE behaviour on MCCH acquisition, service continuity aspects and ETWS geo-fencing for NR NTN.

Discussion

MCCH acquisition

The service area of a certain MBS session is not likely to change when the MBS service in on-going. Hence, once the UE acquires the MBS session ID and corresponding intended service area, it could store the information for future use, e.g., to determine whether it needs to re-acquire updated MCCH, when receiving MCCH change notification.

This approach is similar with what has been specified for MBS in TN cells. In legacy MBS mechanism, according to TS 38.331 [2] (marked in yellow in the box below), if UE interested to receive any MBS broadcast service and the cell is providing SIB20 for any MBS broadcast service, the UE shall acquire MCCH upon entering the cell. The intention is that the UE could determine whether the cell is broadcasting the service it is interested in, by checking whether the TMGI of this service is broadcast in MCCH. One may wonder whether the UE could use the stored service area in USD to avoid MCCH acquisition, when it is outside the service area of the MBS service it interested in. In TS 38.331 [2], only one NOTE is added for that (marked in pink in the box below). In legacy MBS broadcast mechanism, how to enable the UE to avoid (re-)acquiring the MCCH outside the intended service area is up to UE implementation relying on the service area in USD, rather than depending on any specified UE behaviour in normative text.

#### 5.9.2.2 Initiation A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s). NOTE 1: It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification. NOTE 2: It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

Observation 1: According to TS 38.331, how to avoid unnecessary MCCH when outside the service area is left to UE implementation relying on the service area in USD, but not depending on any specified UE behaviour in normative texts.

For the MBS broadcast service in NTN scenario, the only difference is that the service area now is acquired by reading MCCH and the new SIB upon entering the cell, rather than being acquired via higher layer signalling and stored in USD. More specifically, when UE enters an NTN cell, it reads the MCCH according to current spec and acquires the service areas of MBS sessions also based on the new SIB. Considering that an intended service area acquired via MCCH and new SIB in NR NTN can cover those included in a number of cells (as agreed before in RAN2 #128), it can be regarded as the information globally applied to the associated MBS broadcast service within a given region. Once the UE acquires the service area info, it could store this information to determine whether to re-acquire the up-to-date MCCH later. The UE is not required to acquire the MCCH when it is outside any service area of its interested MBS sessions. How to use the stored service area to avoid re-acquiring MCCH, as illustrated above, can be up to UE implementation. We can incorporate this approach in a NOTE, similar to the existing "NOTE 2" cited above.

Proposal 1: Add a NOTE to illustrate that it is up to UE implementation to use the intended service areas in the new SIB and the association relationship between the intended service area ID and mbs-SessionId in MCCH to avoid acquiring the MCCH, when the UE is outside the MBS service area of the MBS broadcast service.

The Spec change needed by Proposal 1 is shown in the TP in Annex for companies' reference.

Service continuity enhancements

In the past meetings, MBS service continuity enhancements in the context of NTN were also discussed. Based on our analyses as follows, we found these enhancements are not essential and have marginal benefits.

Potential enhancements on service area for FSAI

Some companies proposed to introduce service area for FSAI, so that the UE could handle the frequency priority with the consideration of whether it is within the service area of neighbour frequencies/cells. However, it is hard to introduce the service area for FSAI into SIB21 based on the current signalling structure.

According to TS 38.331 [2], the FSAI list in SIB21 is configured in a per frequency manner.

SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 }

In addition, according to TS 29.571 [3], one MBS session is associated with a list of FSAI, and one FSAI is allowed to be associated with multiple MBS sessions.

- 5.9.4.6 Type: MbsSession Table 5.9.4.6-1: Definition of type MbsSession Attribute name; Data type; P; Cardinality; Description. mbsSessionId; MbsSessionId; C; 0..1; MBS session identifier (TMGI and/or SSM, and NID for an SNPN) (NOTE 1). [unrelated part is omitted]. mbsFsaIdList; array(MbsFsaId); O; 1..N; List of MBS Frequency Selection Area Identifiers, for a broadcast MBS session. This attribute may be present if the "serviceType" attribute indicates a broadcast MBS session..

Above quotation implies that, if service areas were to be introduced specifically for FSAI in SIB21, the service area should be provided per FSAI and per MBS session. This brings excessive signalling overhead for the system information considering the SI size limit, making such signalling design unacceptable.

Observation 2: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable due to the SI size limitation concern.

Having checked the existing FSAI mechanisms, we found that the intention raised by company can already be realized by UE and NW implementation. In RAN2#128, it was agreed that the intended service area may include geographic areas across the current serving cell and overlapping neighbour cell(s).

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

That is, the UE could obtain the service area info of an MBS session which covers not only the service areas of the serving cell but also those of the neighbour cells. Combining the info in USD and SIB21, the UE can determine whether the UE can obtain the service from neighbour cells. Specifically:

- For a certain MBS session, the UE acquires the FSAI list of an MBS session ID via upper layer procedure [3][4], and acquires related frequencies of FSAI via SIB21. This means, the UE, can know the frequencies applicable for each MBS session based on current USD and SIB21.

- Based on the intended service areas associated with the MBS session, the UE can know the intended service area within the scope of both serving cell and neighbour cells for each MBS session.

Then, the UE could determine whether it may prioritize the frequency(ies) associated with its intended service area (as per bullet "i"), based on whether it is still located within its intended service as determined by its current GNSS location and bullet "ii". That is, the UE may further prioritize a frequency(ies) in SIB21 for its interested service, if it finds itself located in the intended service area. The information in the exiting USD and the introduced service areas info in new SIB are already sufficient, and additional ASN.1 impact is not needed.

Proposal 2: Do not introduce service area information specific for FSAI in SIB21.

Also, since anyway this MBS specific frequency prioritization behaviour is up to UE implementation (with "... the UE may consider that frequency to be the highest priority during the MBS broadcast session…" in TS 38.304 [5]), we think RAN2 can also confirm that the UE can still consider whether to prioritize a frequency providing its interested MBS service (based on existing SIB21 and USD) as per the current procedure in TS 38.304 [5], and discuss whether any informative text is needed to clarify the use of the intended service area related information (e.g. clarifying that the UE may also take into account whether it is within the intended service area based on the introduced intended service area for the MBS session in the MCCH).

Proposal 2a: The UE can determine whether to prioritize the frequency in SIB21 for its interested MBS service(s) (based on the existing SIB21 and USD) as per the current procedure in TS 38.304. RAN2 discusses whether any informative text is needed, e.g. to further clarify that UE may take into account whether it is located in the intended service area based on the introduced intended service area for the MBS session in MCCH.

Introduction of service area for neighbour cell list in MCCH

Some companies proposed to introduce service area specific for neighbour cells in MCCH, so that the UE could determine whether to request unicast before reselecting to a neighbour cell.

On the one hand, introducing service area for the neighbour cell will lead to potentially excessive signalling overhead, especially if the service areas are contained in SIB. On the other hand, the intention raised by company can also be realized by implementation without ASN.1 impact. In detail, for a certain MBS session, its intended service area may cover geographic areas across the current serving cell and overlapping neighbour cell(s) as per previous meeting agreements. Based on this, the UE could determine whether a neighbour cell is providing this MBS session based on its GNSS location. If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE considers the neighbour cell providing this MBS session. No need to further introduce per neighbour cell service area distinction.

Observation 3: It was agreed that the intended service area for an MBS session may include geographic areas within neighbour cell(s). If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE can consider the neighbour cell providing this MBS session.

Also, we think whether/how the UE considers the service area related configuration in MCCH in addition to the neighbour cell list in MCCH can still be left to UE implementation, when it determines whether the reselected cell providing its interested MBS session at its location.

Proposal 3: Do not introduce service area per neighbour cell in MCCH.

Proposal 3a: In addition to the neighbour cell list provided in MCCH, it can be left to UE implementation whether/how the UE considers the intended service area configured in MCCH for the MBS session, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

ETWS geo-fencing for NTN

In RAN2#129, it was agreed that the warning area coordinates is introduced in SIB6 and SIB7. Similar as SIB8, the Warning Area Coordinates in SIB6 and SIB7 can be defined in and encoded according to TS 23.041, and its IE format is OCTET string.

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

Based on the agreement, segmentation of SIB6 is not supported. Then the IE warningAreaCoordinates in SIB6, if present, carries the coordinates, with one or more octets, of the geographical area where the ETWS primary notification is valid as defined in ATIS 0700041. The first octet of the first warningAreaCoordinates is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 and so on. No segmentation related parameters (e.g. sequence number, segment type, etc.) are needed.

Proposal 4: The IE warningAreaCoordinates in SIB6, if present, carries the coordinates, with one or more octets, of the geographical area where the ETWS primary notification is valid as defined in ATIS 0700041. The first octet of the first warningAreaCoordinates is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 and so on.

SIB7 itself already supports segmentation of the warning message, so it is straightforward that the segmentation of warning area coordinates can also be supported. The IE warningAreaCoordinatesSegment in SIB7, if present, carries a segment, with one or more octets, of the geographical area where the ETWS Secondary notification is valid as defined in ATIS 0700041. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 and so on. Additionally, if warning area coordinates are provided for the warning message, then the field warningMessageSegmentNumber applies to both warning message segment and warning area coordinates segment.

Proposal 5: The IE warningAreaCoordinatesSegment in SIB7, if present, carries a segment, with one or more octets, of the geographical area where the ETWS Secondary notification is valid as defined in ATIS 0700041. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 and so on.

Proposal 5a: If warning area coordinates are provided for the warning message, then the field warningMessageSegmentNumber applies to both warning message segment and warning area coordinates segment.

Conclusion

In this contribution, supportive of broadcast service via NR NTN is discussed with giving corresponding proposals:

MCCH acquisition

Observation 1: According to TS 38.331, how to avoid unnecessary MCCH when outside the service area is left to UE implementation relying on the service area in USD, but not depending on any specified UE behaviour in normative texts.

Proposal 1: Add a NOTE to illustrate that it is up to UE implementation to use the intended service areas in the new SIB and the association relationship between the intended service area ID and mbs-SessionId in MCCH to avoid acquiring the MCCH, when the UE is outside the MBS service area of the MBS broadcast service.

Service continuity enhancements

Observation 2: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable due to the SI size limitation concern.

Observation 3: It was agreed that the intended service area for an MBS session may include geographic areas within neighbour cell(s). If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE can consider the neighbour cell providing this MBS session.

Proposal 2: Do not introduce service area information specific for FSAI in SIB21.

Proposal 2a: The UE can determine whether to prioritize the frequency in SIB21 for its interested MBS service(s) (based on the existing SIB21 and USD) as per the current procedure in TS 38.304. RAN2 discusses whether any informative text is needed, e.g. to further clarify that UE may take into account whether it is located in the intended service area based on the introduced intended service area for the MBS session in MCCH.

Proposal 3: Do not introduce service area per neighbour cell in MCCH.

Proposal 3a: In addition to the neighbour cell list provided in MCCH, it can be left to UE implementation whether/how the UE considers the intended service area configured in MCCH for the MBS session, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

ETWS geo-fencing for NTN

Proposal 4: The IE warningAreaCoordinates in SIB6, if present, carries the coordinates, with one or more octets, of the geographical area where the ETWS primary notification is valid as defined in ATIS 0700041. The first octet of the first warningAreaCoordinates is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 and so on.

Proposal 5: The IE warningAreaCoordinatesSegment in SIB7, if present, carries a segment, with one or more octets, of the geographical area where the ETWS Secondary notification is valid as defined in ATIS 0700041. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 and so on.

Proposal 5a: If warning area coordinates are provided for the warning message, then the field warningMessageSegmentNumber applies to both warning message segment and warning area coordinates segment.

Reference

- RAN2#129 ChairNotes.

- 3GPP TS 38.331: "NR: Radio Resource Control (RRC) protocol specification".

- 3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces".

- 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

- 3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

- Annex: TP on TS 38.331

#### 5.9.2.2	Initiation

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

NOTE 1:	It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

NOTE 2:	It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

NOTE 3:	It is up to UE implementation to use the [intended service areas] in [SIBX] and the association relationship between the [intended service area ID] and mbs-SessionId in MCCH to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.