3GPP TSG-RAN WG2 Meeting #129bis                                  R2-2502137

Wuhan, China, April 7th – 11st , 2025

Agenda item:		8.1.2.3

Source:				Samsung

Title:					Discussion on LCM for POS use case

Document for:		Discussion & Decision

1	Introduction

This contribution provides our view on LCM for POS use case according to the objective in WID [1]. Please, note that the WID was updated in RAN#107 to delete the objectives for 2nd priority as below.

Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: o Direct AI/ML positioning:  (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning  (2nd priority) Case 2b: UE assited/LMF-based positioining with LMP side model, dirct AI/ML positioining  (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning o AI/ML assisted positioning  (2nd priority) Case 2s: UE-assisted/LMF-based positioining with UE-side model, AI/ML assited positioning  (1st priority)Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

2	Discussion

According to WID [1], there are three positioning sub use cases, which can be summarized as below.

| Case | Positioning type | Model entity | AI/ML for Pos type |
| --- | --- | --- | --- |
| 1 | UE-based positioning | UE-side model | Direct |
| 3a | NG-RAN node assisted positioning | gNB-side model | assisted |
| 3b | NG-RAN node assisted positioning | LMF-side model | Direct |

Until now, the following agreements were made for the use case 3a/b with gNB/LMF-side model.

For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

According to the agreement above, we do not see any potential RAN2 spec. impact for the case 3a/b (with gNB/LMF-side model). Thus, from RAN2 perspective, the case 3a/b can be deprioritized compared to the case 1 (with UE-side model). Also, since the case 2a/b were already deleted in the revised WID [1], RAN2 can focus on the use case 1 for now and further discuss other cases later if needed. Therefore, we discuss the LCM aspect mainly considering the case 1 with UE-side model in the following part.

2.1 Baseline LCM procedures for Case 1

In RAN2#127bis, the following baseline procedure is agreed for LCM for Case 1.

1: 	Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

2: 	Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

3: 	Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

4: 	FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

5:   RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Among the open issues with FFS above, the yellow ones were resolved by the following agreements in RAN2#129.

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

However, there are the remaining green-colored issues marked as FFS and we would like to further discuss them one by one.

Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress):

According to the latest FL summary on POS case in RAN1 [2], “network-side additional condition” can be understood as below.

Understanding of “network-side additional condition”: Info about network side implementation details which the network side is not willing to send to UE explicitly, but such info is needed for UE-side model to ensure consistency between training and inference.

With the understanding above, RAN1 has discussed 
1) What information can be explicitly provided to UE as assistance data,
2) What information can be implicitly provided to UE as NW-side additional condition
and the following agreement was made in RAN1#119.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

Based on the agreement, the Info #7 can be regarded as candidate NW-side additional condition, but RAN1 is still discussing whether/how Info #7 is provided from LMF to UE as NW-side additional condition. RAN2 can just wait for RAN1’s conclusion on this.

Proposal. 1: RAN2 can wait for RAN1’s conclusion on NW-side additional condition.

Whether any additional LMF control is needed for UE sending unsolicited LPP provide capabilities for applicability report:

The motivation of LMF's control over the UE's applicability report is to prevent too frequent applicability reports from the UE. To achieve this, the proponent suggests introducing a new indicator to prevent the UE from sending unsolicited UE capability information messages for applicability reporting. However, in our view, the issue of message flooding is not new for AI/ML; it has existed since the adoption of the unsolicited UE capability report mechanism in LPP. We understand that the unsolicited UE capability report mechanism was introduced in LPP with the confidence that UE will not trigger the unsolicited capability report unnecessarily often and not cause the message flooding issue. With the same belief, we see no problem with reusing the unsolicited reporting mechanism for applicability reports as it is. Some might argue that most existing UE capability parameters are relatively static, making UE capability updates for them less dynamic compared to applicability reports. However, even though we acknowledge that applicability can change dynamically depending on the UE's internal conditions, the situation remains similar for existing UE parameters. For example, every time a user turns Bluetooth on or off, the UE must report its updated capabilities to the LMF.

Based on the discussion above, we do not see a need for LMF control over unsolicited LPP ProvideCapabilities for applicability reports and therefore propose to confirm this as follows.

Proposal. 2: UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

2.2 Performance monitoring

For UE-side model case, the UE who deploy the model can do performance monitoring as baseline, but it may not be the only entity for performance monitoring. From our understanding, RAN1 is still considering at least the LMF as another monitoring entity for UE-side model case.

In RAN1#116bis, it is agreed to study the following options for label-based (model output based) monitoring.

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Also, in RAN1#119, RAN1 agreed to support at least Option A as below.

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

Since RAN1 agreed to support at least Option A, RAN2 can start to discuss the signaling aspect to support the option A. For each sub-options (A-x) above, the expected impact on LPP signalling can be summarized as below.

- Option A-1: LMF provides the target UE with the information on ground truth label of the target UE. Considering that this is the assistance information for performance monitoring, it seems reasonable that the LMF provides the information on ground truth label via LPP ProvideAssistanceData message.

- Option A-2: LMF provides the target UE with the position calculation assistance data. Considering that the existing information for UE-based positioning method is provided to UE via LPP ProvideAssistanceData message, any new position calculation assistance data for performance monitoring can be included in the same message.

- Option A-3: LMF provides the target UE with the PRU measurement & location. For the PRU information delivery, the existing Rel-18 PRU information (i.e., nr-PRU-DL-Info) in LPP ProvideAssistanceData message can be reused as already described in the RAN1 agreement above.

- Option A-4: PRU provides the target UE with the PRU measurement & location. According to RAN1 agreement, it can be realized by implementation in a manner transparent to specification. No spec. impact is foreseen now.

Based on the observation above, for all the sub-options A-x, the assistance data for performance monitoring can be provided by LMF to the target UE via LPP ProvideAssistanceData.

Proposal. 3: For POS Case 1, LMF can provide UE with the assistance data for performance monitoring metric calculation via LPP ProvideAssistanceData message.

Meanwhile, RAN2 can also discuss how the UE reports the outcome of performance monitoring to the LMF. Based on the RAN1 discussion until now, it is understood that the UE can calculate the performance metric by evaluating the difference between the location estimate based on AI/ML method and the ground truth label. For the ground truth label, the UE can secure the label after receiving the assistance data from LMF via LPP ProvideAssistanceData. By the way, for the location estimate based on AI/ML method, the UE can obtain the location estimate as an output of AI/ML model inference after receiving a request from LMF via LPP RequestLocationInformation. Therefore, it seems reasonable for UE to report the output of performance monitoring via LPP ProvideLocationInformation as a response to the inference request from the LMF.

Proposal. 4: For POS Case 1, UE can provide LMF with the outcome of performance monitoring via LPP ProvideLocationInformation message.

Since the selection of which sub-options (A-x) to support is still under discussion in RAN1, we can await RAN1's conclusion regarding the detailed content of the assistance data and the outcomes to prevent any potential duplication of efforts. Once RAN1 agrees to support a specific option and provides a detailed list of parameters to support that option, RAN2 can proceed with implementing the signaling details based on the RAN1 parameter list.

Proposal. 5: For performance monitoring in POS Case 1, the content of assistance data for monitoring metric calculation and the content of monitoring outcome can be determined/implemented later based on RAN1 parameter list.

2.3 Functionality management

In RAN2#125bis, RAN2 agreed the following for common LCM framework/signaling.

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

Open issue is whether/how we support “UE autonomous, decision reported to the network” and “Network decision, UE-initiated”. In our understanding, one difference between “UE autonomous, decision reported to the network” and “Network decision, UE-initiated” would be who makes final decision. In case of “UE autonomous, decision reported to the network”, UE applies decision and report to gNB i.e. UE makes final decision, while in case of “Network decision, UE-initiated”, UE requires to get gNB decision to update functionality.

For “UE autonomous, decision reported to the network”, we can consider the following scenario in the POS case 1.

- The LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

- The UE performs the location estimation using AI/ML model, but the performance monitoring result can be ‘fail’.

- The UE can determine to fall back to the legacy positioning method (i.e., “UE autonomous decision”) and reports the estimated location with the fallback indication (i.e., “report to the network”) via LPP ProvideLocationInformation.

For “Network decision, UE-initiated”, we can consider the following scenario for the POS case 1.

- The LMF can request UE to estimate its location using AI/ML model via LPP RequestLocationInformation message.

- The UE performs the location estimation using AI/ML model, but the performance monitoring result can be ‘fail’.

- The UE reports some error cause (e.g., performanceMonitoringFail) via LPP ProvideLocationInformation (i.e., “UE-initiated”).

- The LMF can determine to fall back to some legacy positioning method (i.e., “NW decision”). The LMF requests UE to estimate its location using the legacy method via LPP RequestLocationInformation message.

- The UE reports the estimated location via LPP ProvideLocationInformation.

In the expected signalling flow above, it is assumed that the UE performs the monitoring since both UE autonomous decision and initiation from UE make sense only if the performance monitoring is done at UE side. On the other hand, for “network decision, network-initiated”, it is more practical with LMF-side performance monitoring.

For the UE-side monitoring case, both “UE autonomous, decision reported to the network” and “Network decision, UE-initiated” can work, but the latter can bring additional delay from step4/5 in POS procedure. Thus, in our view, “UE autonomous, decision reported to the network” can be prioritized over “Network decision, UE-initiated”.

Observation. 1: For POS Case 1, for the functionality management, “Network decision, UE-initiated” can lead to additional delay in positioning procedure compared to “UE autonomous, decision reported to the network”.

Based on the observation above, we would like to propose the following.

Proposal. 6: For POS Case 1, for the functionality management, RAN2 support “UE autonomous, decision reported to the network”.

RAN1 also discussed whether UE can make the management decision by itself or not before. However, after RAN1#118, RAN1 seems to have not made any progress on this issue. Also, this issue is about overall signalling flow to support the functionality management which can be regarded as RAN2 scope. In that sense, we think RAN2 can make the decision on this issue and share our decision with RAN1 if needed.

RAN1 has previously discussed whether the UE can autonomously make management decisions or not. However, following RAN1#118, it appears that RAN1 has not made any further progress on this matter. Additionally, this issue pertains to the overall signaling flow supporting functionality management, which falls within the realm of RAN2. In light of this, we believe that RAN2 can make a decision on this issue and, if necessary, share our decision with RAN1.

2.4 Fallback operation

During the last RAN2#129, there was the discussion on whether to support UE autonomous switching/fallback, but the decision was postponed due to the different understanding on on the case where LMF provides the configuration for multiple positioning methods. For further discussion on the fallback issue, we can first review the legacy positioning procedure with UE-based mode as below.

Legacy positioning procedure with UE-based mode (i.e., UE estimates its location)

- LMF can provide the assistance information for one or more positioning methods in step 3 via LPP ProvideAssistanceData

- LMF can request UE to estimate its location and report the results in step 5 via LPP RequestLocationInformation.

  - Note 1: In the LPP RequestLocationInformation, the LMF can just request UE to perform the location estimate with some QoS requirement on the positioning accuracy and/or delay. There is no additional information for LMF to indicate which method UE should use for the estimation with UE-based mode.

  - Note 2: If the assistance data is provided for only one method in step 3, the UE should use that method for location estimate. Otherwise (i.e., if the assistance data is provided for multiple methods in step 3), it’s up to UE implementation how to select the method to use considering the QoS requirement from LMF.

- UE can report the estimated location to LMF via LPP ProvideLocationInformation.

  - UE can indicate which method is used for the location estimate in the report.

First, for the case where the assistance data is provided only for AI/ML method in step 3, it is clear that UE should use AI/ML method for location estimate based on the Note 2 above. Therefore, we would like to propose the following.

Proposal. 7: For POS Case 1, if the assistance data is provided only for AI/ML method in step 3, the LMF can request UE to perform location estimate as in legacy (i.e., without any new indication for AI/ML) in step 5. Then, UE shall return a location estimate based on AI/ML method if possible, or indicate a location error if not possible. I.e., Any kinds of switching & fallback are not allowed if LMF provides only AI/ML method.

Now let’s consider the scenario where the assistance data is provided for multiple methods including AI/ML method in step 3. If we follow the legacy procedure above, LMF can not configure UE to use AI/ML method via LPP RequestLocationInformation and it is up to UE implementation which method to use for location estimate. I.e., UE autonomous switching (or selection) is allowed. This also means that LMF does not have any controllability on the use of AI/ML method, which can be also understood as LMF-based functionality management is not supported.

Now, let us consider the scenario where assistance data is provided for multiple methods, including the AI/ML method, in step 3. If we adhere to the legacy procedure outlined above, the LMF cannot configure the UE to use the AI/ML method via LPP RequestLocationInformation, leaving it up to the UE's implementation to decide which method to employ for location estimation. In other words, UE autonomous switching (or selection) is permitted. This also implies that the LMF does not have any controllability on the use of the AI/ML method, which can be interpreted as indicating that LMF-based functionality management is not supported.

Observation. 2: With the legacy LPP procedure, if the assistance data is provided for multiple methods including AI/ML method in step 3, it is up to UE implementation whether to use AI/ML method or not when LMF requests location estimate. I.e., LMF-based functionality management is not supported and UE autonomous switching (or selection) is allowed.

However, given that we have already agreed to support "network decision, network-initiated" as the baseline for functionality management, there needs to be some enhancement to enable LMF-based functionality management. Firstly, for activating the AI/ML method, the LMF should have the capability to configure the UE to prioritize the use of the AI/ML method over other methods for location estimation. Subsequently, the UE can attempt to estimate the location using the AI/ML method and report the results if available. If not, the UE can fallback to any non-AI method and return location estimates based on that. By adopting this approach, we can also support "UE autonomous, decision reported to the network," as proposed in P6 above.

Proposal. 8: For POS Case 1, if the assistance data is provided for multiple methods including AI/ML method in step 3, LMF can indicate UE to prioritize AI/ML method for location estimate in step 5. Then, UE shall return a location estimate based on AI/ML method if possible, or instead return location estimates based on other available position methods. I.e., UE autonomous fallback can be supported if LMF provides multiple methods including AI/ML method.

Next, for deactivation of AI/ML method, LMF should be able to configure UE not to use AI/ML method (i.e., fallback to non-AI method) for location estimate. Then, UE can try to estimate the location using one of the non-AI methods and return location estimates based on it.

Proposal. 9: For POS Case 1, if the assistance data is provided for multiple methods including AI/ML method in step 3, LMF can request UE to fallback to non-AI/ML method for location estimate in step 5. Then, UE shall return a location estimate based on non-AI/ML method if possible, or indicate a location error if not possible. I.e., LMF-based fallback can be supported.

One might argue that LMF can indicate the fallback by releasing the assistance data for AI/ML method via LPP ProvideAssistanceData message. However, in that case, LMF would have to send two consecutive LPP messages (i.e., LPP ProvideAssistanceData and LPP RequestLocationInformation) every time it makes the functionality management decision (i.e., activate/deactivate), which appears highly inefficient to us. It does not make sense to transmit a substantial amount of assistance data merely to activate/deactivate the AI/ML method.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Baseline LCM procedure for Case 1:

Proposal. 1: RAN2 can wait for RAN1’s conclusion on NW-side additional condition.

Proposal. 2: UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

Performance monitoring:

Proposal. 3: For POS Case 1, LMF can provide UE with the assistance data for performance monitoring metric calculation via LPP ProvideAssistanceData message.

Proposal. 4: For POS Case 1, UE can provide LMF with the outcome of performance monitoring via LPP ProvideLocationInformation message.

Proposal. 5: For performance monitoring in POS Case 1, the content of assistance data for monitoring metric calculation and the content of monitoring outcome can be determined/implemented later based on RAN1 parameter list.

Functionality management:

Observation. 1: For POS Case 1, for the functionality management, “Network decision, UE-initiated” can lead to additional delay in positioning procedure compared to “UE autonomous, decision reported to the network”.

Proposal. 6: For POS Case 1, for the functionality management, RAN2 support “UE autonomous, decision reported to the network”.

Fallback operation:

Proposal. 7: For POS Case 1, if the assistance data is provided only for AI/ML method in step 3, the LMF can request UE to perform location estimate as in legacy (i.e., without any new indication for AI/ML) in step 5. Then, UE shall return a location estimate based on AI/ML method if possible, or indicate a location error if not possible. I.e., Any kinds of switching & fallback are not allowed if LMF provides only AI/ML method.

Observation. 2: With the legacy LPP procedure, if the assistance data is provided for multiple methods including AI/ML method in step 3, it is up to UE implementation whether to use AI/ML method or not when LMF requests location estimate. I.e., LMF-based functionality management is not supported and UE autonomous switching (or selection) is allowed.

Proposal. 8: For POS Case 1, if the assistance data is provided for multiple methods including AI/ML method in step 3, LMF can indicate UE to prioritize AI/ML method for location estimate in step 5. Then, UE shall return a location estimate based on AI/ML method if possible, or instead return location estimates based on other available position methods. I.e., UE autonomous fallback can be supported if LMF provides multiple methods including AI/ML method.

Proposal. 9: For POS Case 1, if the assistance data is provided for multiple methods including AI/ML method in step 3, LMF can request UE to fallback to non-AI/ML method for location estimate in step 5. Then, UE shall return a location estimate based on non-AI/ML method if possible, or indicate a location error if not possible. I.e., LMF-based fallback can be supported.

4	Reference

- RP-250792, ‘Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface’, Qualcomm

- R1-2410718, ‘Summary #5 of specification support for positioning accuracy enhancement’, Ericsson