3GPP TSG-RAN WG2 Meeting #129bis					     R2-2502208

Wuhan, China, 07 – 11 April, 2025

Source: 	ZTE Corporation, Sanechips

Title: 	Open issues for data transmission and MAC PDU design

Agenda item:	8.2.4

Document for: 	Discussion and Decision

# Introduction

In this contribution we discuss the details of Open issues for data transmission and MAC PDU design for A-IoT data transmission.

# General design for A-IoT MAC layer

# Introduction

During SID study, RAN2 has made assumption that, on A-IoT interface between the device and the reader, there is A-IoT MAC layer only and no other L2 AS layer. Furthermore, there is no CP/UP protocol stack differentiation. In the following sections, we will discuss the main procedures supported by A-IoT MAC layer, and the associated MAC Protocol Data Units, formats and parameters.

# MAC Entities

Given the requirement for simplicity of A-IoT device, RAN2 can assume that only one MAC entity is supported by the device which would not be configured by the reader and instead, the MAC entity can be initialized by the device itself upon power-on.

Proposal 1a: Only one MAC entity is supported by the A-IoT device which will not be configured by the reader but be initialized by the device itself upon power-up.

Based on the study progress from physical layer, it’s straightforward to have the following further proposal:

Proposal 1b: The MAC entity of the A-IoT device handles the following transport channels:

-	RDCH, the R2D Shared channel for transmission from reader to device;

-	DRCH, the D2R Shared channel for transmission from device to reader.

# MAC services

For A-IoT device, the MAC layer is between the upper layer and the physical layer, so the MAC layer will provide services to upper layer and expect the services from the physical layer.

Since no other L2 AS layer would be supported, here the upper layer is NAS-like layer in RAN2 assumption. According to the SID study, we assume the data segmentation and ACK-NACK feedback for the transmission (in D2R direction) would be supported by the MAC layer itself, therefore no ACK-NACK feedback is expected from the physical layer.

The initial consideration on the services that the MAC layer can provide and obtain are as below.

Proposal 2:

The MAC layer provides the following services to upper layers:

- Data transfer services;

- Radio resource allocation

The MAC layer expects the following services from the physical layer:

- Data transfer services;

# MAC functions

Since no other AS protocol layers, such as RRC, PDCP, and RLC is supported, all A-IoT functionality agreed during the study should be directly supported by MAC layer. In the following proposal we propose the MAC layer functionality based on the agreements reached during the study item.

Proposal 3: The MAC protocol supports at least the following functions:

- Connection control:

- Paging;

- Random access;

- Radio resource selection for D2R transmission;

- Data transmission from/to transport channels;

- Data segmentation, at least for D2R;

- Transmission error correction through ACK-NACK feedback.

The relevance of MAC functions for uplink, downlink is indicated in following Table 1

Table 1: The link direction association of MAC functions.

| MAC function | R2D | D2R |
| --- | --- | --- |
| Connection control-Paging | X |  |
| Connection control-Random access | X | X |
| Radio resource selection |  | X |
| Data transmission from/to transport channels | X | X |
| Data packet segmentation |  | X |
| Error correction through ACK-NACK scheme |  | X |

# MAC procedures

# Device state

It was agreed during the study item that from RAN2 perspective, the device is always monitoring PRDCH (as long as there is enough energy). However, the device should be receiving different types of R2D messages depending on various conditions. The following aspects need to be considered:

- Before paging: Before receiving a paging indication, the device should monitor PRDCH for paging messages. This is the only type of message that the device should be monitoring and responding to in this state. Once the device receives paging and determines that the paging is for the device, it should store the transaction ID included in the paging message.

- After paging: Once paging is received and a transaction ID is stored on the device side, the device shall monitor the PRDCH for both broadcast messages (e.g. MSG1 trigger messages, MSG2 etc) and unicast messages (addressed to the devices ASID) that carry the stored transaction ID. Thus, it is necessary that all R2D messages carry the transaction ID.

- Completion of the AS procedure: In Rel-19 only inventory and command procedures are supported. So, once the device is paged, the AS procedure for inventory or command is initiated and the transaction ID and any ASID associated with the transaction ID are stored in the device until the AS procedure is successfully completed or terminated due to failure. Upon both successful completion and failure, the device shall delete the stored transaction ID and the ASID and start monitoring paging again (i.e. go back to step 1 above).

So, based the above, the following proposal is made:

Proposal 4a: When the device has sufficient energy, the device shall:

- If there is no stored transaction ID:

  - Monitor PRDCH for paging messages and store the received transaction ID once a paging message addressed to the device is received

- If there is a stored transaction ID:

  - Monitor PRDCH for broadcast (e.g. MSG2 and MSG1 trigger messages) and unicast messages (addressed to the device’s ASID) that include the stored transaction ID until the AS procedure (Inventory/Command) is completed (either successfully or unsuccessfully)

Proposal 4b: For inventory use case, the AS procedure is considered to be successfully completed upon detecting that the MSG3 has been successfully received at the reader (i.e. No NACK for the MSG3).

Proposal 4c: For command use case, the reader includes an explicit indication in R2D message to indicate the successful completion of the AS procedure.

Proposal 4d: All R2D messages include a transaction ID

# MAC PDU formats and parameters

At RAN2#129 the following message types were agreed:

    - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF))

    - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not)

    - R2D MAC PDU (MSG2)

    - D2R MAC PDU (MSG3 and data)

    - R2D MAC PDU (R2D data)

    - Other message types are FFS.  The message types may evolve based on functionality agreements.

In addition to the above, scheduling information message is also needed in R2D direction.

The general message structure should be as shown below:

Figure 1: General Message structure in R2D and D2R

Proposal 5: The general message structure for MAC includes the following fields

- Message type field (FFS if this is needed for all messages – e.g. for RN16)

- Header fields (these fields contain the information that it terminated in MAC layer) – i.e. not passed to upper layers

- Payload (the information in the payload is passed to upper layers)

- Padding (the need for padding is FFS for now)

# Example MAC PDU formats for some messages

Figure 2 below shows the detailed structure of a few example messages in R2D and D2R:

Figure 2: Example MAC PDU structure for messages

Proposal 6: The above message structures proposed in the figure should be taken as baseline

# Handling of ASID

As agreed at the last meeting, the ASID is needed for the command use case. So, the assignment of ASID should not impact the inventory procedure. It is important to keep the inventory procedure light weight. So, we make the following proposal:

Proposal 7: If the reader assigns an ASID, it should be included in the message carrying the command message

Proposal 8: If the reader does not assign a new ASID in the message carrying the command message, the device uses the RN16 (included in MSG1) as the ASID during the command procedure

# Segmentation and retransmissions

For D2R transmissions, it was agreed at the last meeting that segmentation and segment retransmissions are based on the offset indication from the reader. One issue that was left FFS is whether the unsegmented message can also be retransmitted. In case of unsegmented message, it is possible to retransmit this message either by repeating the command or by indicating an offset of 0 in the next grant. RAN2 should select one of these options.

Proposal 9: For the retransmission of the unsegmented packet (e.g. first packet), RAN2 should select one of the following options:

Option 1: reader to resend the command message

Option 2: reader to send a new D2R grant with indicating the offset of 0 bits.

Then, finally we also need to agree the granularity of the offset indications. Since the offset indicates the total number of bits received so far, the granularity of the offset should be multiples of D2R message sizes. So, the granularity of the offset should be selected such that 1 or more D2R messages with max D2R message size can be signalled using the offset. Note that extremely fine granularity is not necessary for the offset (as this increases the R2D message size).

Proposal 10: The granularity of the offset should be designed such that it covers 1 or more D2R TB sizes (including the max D2R TB size)

# Conclusion and proposals

The following proposals are made in this contribution:

Proposal 1a: Only one MAC entity is supported by the A-IoT device which will not be configured by the reader but be initialized by the device itself upon power-up.

Proposal 1b: The MAC entity of the A-IoT device handles the following transport channels:

-	RDCH, the R2D Shared channel for transmission from reader to device;

-	DRCH, the D2R Shared channel for transmission from device to reader.

Proposal 2:

The MAC layer provides the following services to upper layers:

- Data transfer services;

- Radio resource allocation

The MAC layer expects the following services from the physical layer:

- Data transfer services;

Proposal 3: The MAC protocol supports at least the following functions:

- Connection control:

- Paging;

- Random access;

- Radio resource selection for D2R transmission;

- Data transmission from/to transport channels;

- Data segmentation, at least for D2R;

- Transmission error correction through ACK-NACK feedback.

Proposal 4a: When the device has sufficient energy, the device shall:

- If there is no stored transaction ID:

- Monitor PRDCH for paging messages and store the received transaction ID once a paging message addressed to the device is received

- If there is a stored transaction ID:

- Monitor PRDCH for broadcast (e.g. MSG2 and MSG1 trigger messages) and unicast messages (addressed to the device’s ASID) that include the stored transaction ID until the AS procedure (Inventory/Command) is completed (either successfully or unsuccessfully)

Proposal 4b: For inventory use case, the AS procedure is considered to be successfully completed upon detecting that the MSG3 has been successfully received at the reader (i.e. No NACK for the MSG3).

Proposal 4c: For command use case, the reader includes an explicit indication in R2D message to indicate the successful completion of the AS procedure.

Proposal 4d: All R2D messages include a transaction ID

Proposal 5: The general message structure for MAC includes the following fields

- Message type field (FFS if this is needed for all messages – e.g. for RN16)

- Header fields (these fields contain the information that it terminated in MAC layer) – i.e. not passed to upper layers

- Payload (the information in the payload is passed to upper layers)

- Padding (the need for padding is FFS for now)

Proposal 6: The message structures proposed in the figure should be taken as baseline

Proposal 7: If the reader assigns an ASID, it should be included in the message carrying the command message

Proposal 8: If the reader does not assign a new ASID in the message carrying the command message, the device uses the RN16 (included in MSG1) as the ASID during the command procedure

Proposal 9: For the retransmission of the unsegmented packet (e.g. first packet), RAN2 should select one of the following options:

Option 1: reader to resend the command message

Option 2: reader to send a new D2R grant with indicating the offset of 0 bits.

Proposal 10: The granularity of the offset should be designed such that it covers 1 or more D2R TB sizes (including the max D2R TB size)

# References

[1] 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), December, 2024

[2] 3GPP RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN Plenary#106, December, 2024