3GPP TSG-RAN WG2 Meeting #129bis	R2-2502265

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.7.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	LCP enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

# 2	Discussion

## 2.1	Handling of negative Bj for LCHs with upgraded priority

For the open issue on Bj enhancement, it is worth noting that the two round LCP procedure was introduced to ensure fairness as well as avoid starvation. PBR and token bucket were introduced to guarantee the GBR bearer can meet its data rate before allocating resources for best effort bearers.

It will only work with the assumption of the NW would configure the PBR and token bucket duration reasonably even if with some scheduling delay. Nothing new comes with XR traffic as it needs to meet the data rate and PDB QoS requirements. Thus, it should be enough to rely on sensible NW implementation to configure PBR and token size according to the traffic characteristic, and QoS requirements on data rate and PDB since the delay critical data in the buffer, caused by scheduling delay, would either be discarded, if not scheduled on time, or cleared with accumulated token bucket.

Observation 1: sensible NW implementation would configure PBR and token bucket size properly to ensure it will meet the QoS requirements without resulting in data stuck in the buffer.

Besides, it should be noticed that in current specifications Bj is already allowed to go negative during the first round of the LCP procedure. This is primarily to enable the UE to avoid unnecessary segmentation, but in theory there is no constraint on how much negative Bj can go during the first round of the LCP procedure. Of course, if Bj is decreased to a very large negative value (which in practice means the PBR bucket is exceeded), the corresponding LCH may be precluded from being allocated resources in the first round of the LCP procedure the next time the UE is granted resources. Again, this can be avoided by proper configuration of the PBR and token bucket duration.

Observation 2: allowing Bj to go negative during first round of the LCP procedure is already allowed in current MAC specifications to avoid unnecessary segmentation.

Additionally, the enhanced DSR (Rel-19) provides the gNB with detailed knowledge of delay-critical data and associated remaining time in the UE buffer, hence the gNB can schedule a grant so that delay-critical data can be allocated in either the first or second step of the LCP procedure.

Observation 3: with enhanced DSR and proper configuration of PBR and bucket size, NW has all the flexibility needed to ensure it can allocate enough resources for the transmission of delay-critical data in either rounds of the LCP procedure.

Proposal 1: no enhancement needed for Bj handling for LCH priority adjustment.

## 2.2	Impact of additional priority on intra-UE prioritization and SR priority determination

For intra-UE prioritization, it was agreed the UE shall also use the additional LCP priority for UL grant priority determination if it is used for LCP when there is delay critical data to be transmitted. We understand the intention of the agreement is the priority of the LCH that is used in the LCP is used to determine the priority of the LCH for UL grant priority determination. Since there is no LCP for retransmission, it should be the priority that has been used for initial transmission.

Proposal 2: for intra-UE prioritization for both initial transmission grant and retransmission grant, the priority of the LCH is the one applied for LCP for both initial transmission since there is no new LCP for retransmission.

Possible rewording for the text from the running CR could be considered. “highest” is not needed for the LCH as in the existing text refers to highest of all the LCHs. Besides, “applied” would be only for the case that the PDU is already stored but it is worth noting that for a deprioritized UL grant, the LCP would not be performed but only for the determination of UL grant priority, “to be applied” might also be needed for such case or for the MAC PDU to be built once UL grant is determined to be prioritized.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this determination, the priority of a data logical channel shall be the priority applied or to be applied for it in the LCP procedure for this uplink grant (see clause 5.4.3.1.3). The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Proposal 3: revise the text for intra-UE prioritization as “In this determination, the priority of a data logical channel shall be the priority applied or to be applied for it in the LCP procedure for this uplink grant (see clause 5.4.3.1.3).”

For priority determination for BSR triggered SR, since it was agreed not to do any change for BSR trigger/report and there is no LCP performed when SR is triggered, it makes sense to avoid impact on SR priority as well. Otherwise, each time when the UE sends SR, it needs to check whether there is delay critical data for the LCH that would change the priority of the SR itself. Since the NW knows the mapping of SR to LCHs and which LCHs are delay critical, it could be enough to rely on DSR reporting mechanisms to get the delay critical data information.

Proposal 4: LCH priority adjustment has no impact on SR priority determination.

## 2.3	LCH priority adjustments

Point 2 of the RAN2#128 agreements states that, as an optional capability, the UE can also support to fallback to default priority in the 2nd round of the LCP procedure. What is not clear from the agreement, is whether the capability is with or without signalling and whether it will be enabled based on NW configuration.

The RAN2 guidelines for UE capability definitions in R2-2002378 include indication to avoid defining any functionality that has no RRC configuration but is dependent on capability bits.

Observation 4: Based on the guidance, RAN2 should avoid defining any functionality that has no RRC configuration but is dependent on capability bits.

So, in case of capability with signalling, the capability should enable a specific RRC configuration. The only RRC configuration that such capability could enabled is the NW configuring a UE indicating capability to support fallback to default priority in the 2nd round of the LCP procedure, whether it shall fallback to default priority in case all the delay-critical data has been allocated in the 1st round.

On the other hand, the LCP prioritization is done after the NW allocates resources to the UE. The NW makes use of all the available information to make the best decision such that delay-critical data can be transmitted within the delay budget. Used information for that purpose are coming from UE reports such as BSR, and enhanced BSR in Rel-18, DSR, and currently under discussion enhanced DSR with multiple remaining time thresholds. Each of those provide a certain level of granularity for LCGs buffer size and buffer size together with remaining time.

Observation 5: NW decides on the resource allocation based on the available information it has from UE reports such as BSR and DSR.

Observation 6: It is not clear based on what information or specific condition the NW would decide to configure a UE, which indicates support to fallback to default priority in the 2nd round of LCP procedure, not to do so.

In case of capability without signaling, this essentially corresponds to leave for UE implementation whether to fallback to default priority in the 2nd round of the LCP procedure.

Observation 7: Capability without signaling corresponds to leave for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure.

Therefore, we suggest RAN2 revisit the agreement from RAN2#128 based on one of the following options:

- Option #1: No capability

  - It is agreed that the additional LCH priority is always applied to both the first round and the second round of the LCP procedure, or that all UEs support fallback to default priority in the 2nd round of the LCP procedure

- Option #2: Capability without signaling

  - It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

- Option #3: Capability with signaling

  - In which case the NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

Proposal 5: RAN2 to choose from one of the following options:

- Option #1: No capability

  - Option #1a: the additional LCH priority is always applied to both the 1st and 2nd round of the LCP procedure,

  - Option #1b: all UEs support fallback to default priority in the 2nd round of the LCP procedure

- Option #2: capability without signaling

  - It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

- Option #3: capability with signaling

  - The NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

# 3	Conclusion

Remaining issues on LCP are discussed in this contribution with the following observations/proposals proposed:

Observation 1: sensible NW implementation would configure PBR and token bucket size properly to ensure it will meet the QoS requirements without resulting in data stuck in the buffer.

Observation 2: allowing Bj to go negative during first round of the LCP procedure is already allowed in current MAC specifications to avoid unnecessary segmentation.

Observation 3: with enhanced DSR and proper configuration of PBR and bucket size, NW has all the flexibility needed to ensure it can allocate enough resources for the transmission of delay-critical data in either rounds of the LCP procedure.

Proposal 1: no enhancement needed for Bj handling for LCH priority adjustment.

Proposal 2: for intra-UE prioritization for both initial transmission grant and retransmission grant, the priority of the LCH is the one applied for LCP for both initial transmission since there is no new LCP for retransmission.

Proposal 3: revise the text for intra-UE prioritization as “In this determination, the priority of a data logical channel shall be the priority applied or to be applied for it in the LCP procedure for this uplink grant (see clause 5.4.3.1.3).”

Proposal 4: LCH priority adjustment has no impact on SR priority determination.

Observation 4: Based on the guidance, RAN2 should avoid defining any functionality that has no RRC configuration but is dependent on capability bits.

Observation 5: NW decides on the resource allocation based on the available information it has from UE reports such as BSR and DSR.

Observation 6: It is not clear based on what information or specific condition the NW would decide to configure a UE, which indicates support to fallback to default priority in the 2nd round of LCP procedure, not to do so.

Observation 7: Capability without signaling corresponds to leave for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure.

Proposal 5: RAN2 to choose from one of the following options:

- Option #1: No capability

  - Option #1a: the additional LCH priority is always applied to both the 1st and 2nd round of the LCP procedure,

  - Option #1b: all UEs support fallback to default priority in the 2nd round of the LCP procedure

- Option #2: capability without signaling

  - It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

- Option #3: capability with signaling

  - The NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure