3GPP TSG-RAN WG2 Meeting #129bis	R2-2502365

Wuhan, China, April 7 – April 11, 2025

Agenda item:		8.7.5

Source:		Lenovo

Title:		AM RLC enhancement

Document for:		Discussion and Decision

- Introduction

In this contribution we discuss RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

- Discussion

2.1 Avoiding Unnecessary Retransmissions

At the RAN2#129 meeting, the following agreements regarding unnecessary retransmission avoidance were achieved:

Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

When the receiver RLC entity abandons outdated RLC SDUs upon new RLC timer expiration, it is beneficial to timely trigger RLC status report and send it to peer RLC. For example, if RX_NEXT is updated to a value equal to or larger than current RX_NEXT + AM_Window_Size when the new timer expires, it means the transmission window is probably stacked. It is similar as the legacy RLC status report trigger case when the receiver RLC entity receives polling request associated with an AMD PDU with SN >= RX_Next + AM_Window_Size. Furthermore, it was also previously agreed in RAN2 that both transmission as well as reception windows need to be in sync, and therefore, we think that the expiry of the new timer needs to be considered as a new trigger for RLC status report.

Observation 1. If the RX_NEXT is updated to a value equal to or larger than current RX_NEXT + AM_Window_Size upon the new timer expiration, it means the transmission window is probably stacked.

Proposal 1. The expiry of the new timer in RLC receiver is considered as a new trigger for RLC status report.

During the previous RAN2 meeting, it was also agreed that the value of the new timer will not be lower than that of the t-Reassembly timer. This means that there could exist a corner case where the new timer is set to the same value of the t-Reassembly timer. It has also been agreed that the abandoned SDUs as determined by the new timer will be positively acknowledged in a status report, whereas the expiry of t-Reassembly will also trigger a status report containing negative acknowledgments for incomplete and/ or missing SDUs. Since both the new timer as well as the t-Reassembly have the same start condition, this could lead to discrepancy in the information provided to the RLC Tx entity depending on which status report is received at the transmitter side. Hence, we think RAN2 should discuss a way forward to prevent such a discrepancy, and we think that a simple solution would be to capture a note in the spec indicating that such a discrepancy should be prevented, e.g., by giving precedence to the status report constructed based on the expiry of the new timer over the expiry of t-Reassembly for the same RLC SDUs.

Proposal 2. RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

In TS 28.552, Layer 2 performance measurement for DL Packet Loss rate is based on the number of DL packets, for which at least a part has been transmitted over the air but not positively acknowledged. However, RAN2 assumes that the legacy RLC status report can be reused to notify the transmission RLC entity of transmission window update due to abandoned RLC SDUs by setting fake ACKs. The transmission RLC entity cannot differentiate between abandoned SDUs and the data which have actually been successfully received. And as a result, this impacts the accuracy of the layer 2 measurement for DL transmission delay and DL Packet Loss rate from point view of QoS monitoring. Therefore, RAN2 needs to discuss the impact of Layer 2 performance due to the avoidance of unnecessary RLC retransmission.

Observation 2. When the receiver RLC entity abandons the outdated RLC SDU based on new timer, it sends the fake ACK for the outdated RLC SDU to the transmitter RLC entity, which impacts at least DL packet error rate measurement.

Proposal 3. RAN2 to discuss the issue on DL packet error rate measurement due to the fake ACK of the abandoned RLC SDUs in the RLC status report.

2.2 Timely Retransmissions

At the RAN2#129 meeting, the following agreements were made regarding timely retransmissions:

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling.

RAN2 has agreed that autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. RAN2 further discuss if remaining time is determined based on discardTimer at PDCP or new timer at RLC.

- Option 1 is to determine remaining time based on a new RLC timer.

In such implementation, additional RLC timer is needed to maintain for each RLC PDU. RLC entity needs to determine when to start a new timer for each RLC SDU. For example, when first transmission of each RLC PDU is performed, RLC entity starts a new timer. When the remaining time of the timer is below a threshold, UE considers the AMD PDU as for retransmission. When the remaining time of the timer is below another threshold, UE considers the AMD PDU as for polling trigger. Also, RLC entity needs to determine whether to stop the new timer if receiving discard indication from PDCP entity.

- Option 2 is to determine remaining time based on PDCP discard timer.

In such implementation, an indication similar to the delay-critical indication is needed from PDCP entity to RLC entity. The PDCP entity transmits an indication to the RLC entity if the remaining time of a PDCP SDU falls below a threshold of enhanced poll. The PDCP entity transmits another indication to the RLC entity if the remaining time of a PDCP SDU falls below a threshold of autonomous retransmission. One possibility, when receiving the indication from PDCP entity, if the initial transmission of the RLC PDU has not been transmitted. RAN2 needs to discuss whether to ignore the indication.

Since RAN2 has already defined PDCP indication for delay-criticality previously, we think this is a simpler and more straight-forward approach for autonomous retransmission and enhanced polling. Additionally, we think that with this approach, the specification complexity of introducing new timers at the RLC along with the new RLC behaviour, can be avoided.

Proposal 4. If the remaining time is determined based on RLC timer, RLC entity starts a new timer when initial transmission of each RLC PDU is performed and stop the timer if receiving discard indication from PDCP entity.

Proposal 5. If the remaining time is determined based on discardTimer at PDCP, the PDCP entity transmits corresponding indication to the RLC entity if the remaining time of a PDCP SDU based on discard timer falls below a corresponding threshold. FFS if discardTimeforLowImportance is considered.

For XR burst data, if the remaining time of a PDU within a PDU set is below a threshold, the remaining time of the subsequent PDUs in the PDU set are also likely to quickly fall below the threshold. We think that excessive polling overhead should be avoided. One possible enhancement to limit the excessive triggering of enhanced polling is by introducing a new parameter. For example, if the remaining time of one RLC SDU is below the enhanced poll trigger threhold, UE increments the variable by one. If the variable is equal to a defined value, UE triggers the next enhanced poll.

Proposal 6. RAN2 to discuss whether to limit the excessive triggering of enhanced polling based on remaining time.

Additionally, RAN2 should also consider mechanisms to cancel either enhanced polling or autonomous retransmission or both for a packet to avoid overloading of the link. For example, if a RLC entity is configured with both features and a legacy status report is received for a packet with a negative acknowledgement before the enhanced poll or autonomous retransmission were triggered, the enhanced poll and autonomous retransmission of that packet can be cancelled since a legacy retransmission would already be triggered in response to the NACK. Or, the cancellation of an enhanced poll or autonomous retransmission can be considered when such was already triggered and an ACK (e.g., HARQ ACK or a RLC status-report containing an ACK for the corresponding PDU) is received before the enhanced poll or autonomous retransmission is submitted to lower layers.

Proposal 7. RAN2 to agree to cancel the triggered enhanced polling and autonomous retransmission based on the reception of status report.

RAN2 needs to discuss whether autonomous retransmission could be counted to the total number of the retransmission. There is no motivation to relax the RLF detection criteria for the autonomous retransmission since RAN2 has already agreed that only one autnomous retransmission is done per RLC SDU. In our view, autonomous retransmission should be counted to total number of the retransmission as legacy retransmision. In addition, RAN2 needs to discuss whether autonomous retransmission can be triggered if the maximum number of RLC retransmission based on status report has reached. We think that autonomous retransmission should not be triggered in such case. We consider this since the autonomous retransmission thershold value should be smaller than the packet delay budget and the maximum number of RLC retransmission is set by taking account of the packet delay budget. Theoretically, autonomous retransmission is most probably triggered before the maximum number of RLC retransmission is reached.

Proposal 8. RAN2 to agree that autonomous retransmission is counted into RETX_COUNT.

Proposal 9. RAN2 to agree that autonomous retransmission should not be triggered if the maximum number of RLC retransmission has reached.

Additionally, another condition that can be considered to enable autonomous retransmissions is the PSI value. For example, only those packets with PSI value greater than a preconfigured threshold are eligible for autonomous retransmissions. Lastly, RAN2 can also look into scheduling enhancements (e.g., LCP enhancements) for packets that are autonomously retransmitted to avoid any delays in transmission of such data before the discardTimer expires.

Proposal 10. To achieve autonomous retransmissions on RLC layer, RAN2 to agree additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

- Conclusion

In this contribution, we discuss the RLC AM enhancement for XR traffic with small delay budget. We have the following observation and proposals:

Observation 1. If the RX_NEXT is updated to a value equal to or larger than current RX_NEXT + AM_Window_Size upon the new timer expiration, it means the transmission window is probably stacked.

Observation 2. When the receiver RLC entity abandons the outdated RLC SDU based on new timer, it sends the fake ACK for the outdated RLC SDU to the transmitter RLC entity, which impacts at least DL packet error rate measurement.

Proposal 1. The expiry of the new timer in RLC receiver is considered as a new trigger for RLC status report.

Proposal 2. RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

Proposal 3. RAN2 to discuss the issue on DL packet error rate measurement due to the fake ACK of the abandoned RLC SDUs in the RLC status report.

Proposal 4. If the remaining time is determined based on RLC timer, RLC entity starts a new timer when initial transmission of each RLC PDU is performed and stop the timer if receiving discard indication from PDCP entity.

Proposal 5. If the remaining time is determined based on discardTimer at PDCP, the PDCP entity transmits corresponding indication to the RLC entity if the remaining time of a PDCP SDU based on discard timer falls below a corresponding threshold. FFS if discardTimeforLowImportance is considered.

Proposal 6. RAN2 to discuss whether to limit the excessive triggering of enhanced polling based on remaining time.

Proposal 7. RAN2 to agree to cancel the triggered enhanced polling and autonomous retransmission based on the reception of status report.

Proposal 8. RAN2 to agree that autonomous retransmission is counted into RETX_COUNT.

Proposal 9. RAN2 to agree that autonomous retransmission should not be triggered if the maximum number of RLC retransmission has reached.

Proposal 10. To achieve autonomous retransmissions on RLC layer, RAN2 to agree additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

- References

TS 38.322 v 18.0.0

R2-2410287, AM RLC enhancement, Lenovo