3GPP TSG-RAN WG2 #129bis	R2-2502368

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Data collection issues related to RAN3 and SA5

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss the issues that may have RAN3/SA5 impact. And RAN3/SA5 is expected to discuss the relevant impacts in their upcoming meetings.

# Discussion

## 2.1	Management-based MDT or signalling-based MDT

RAN2#126 Agreement Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In RAN2#126 meeting, it was agreed that immediate MDT will be taken as the baseline framework for OAM centric data.

In the meanwhile, the legacy MDT framework, only supports the gNB being requested to collect L3 measurements. In order to request the gNB to collect L1 measurements, some enhancement on network interface signalling may be necessary, e.g., NG-AP interface signalling between AMF and gNB.

Legacy MDT framework only support gNB being requested to collect L3 measurement results. Enhancements to network interface signalling may be needed to support the request of L1 measurement results.

At the same time, there are two kinds of MDT framework, namely management-based MDT and signalling-based MDT. For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria. Comparatively, for signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Comparing to signalling-based MDT, management-based MDT framework seems more suitable for training data collection among a group of UEs that fulfils certain criteria (e.g., cell or frequency or channel condition as discussed for the CSI/BM/Positioning use cases). In addition, management-based MDT is inherently anonymized meaning OAM does not need to understand the data is from which UE, as such it is user privacy friendly.

For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Since management-based MDT and signaling-based MDT would have different implications and specification impacts to RAN3/SA5, it is suggested that RAN2 decides if management-based MDT or signaling-based MDT or both are assumed.

For OAM-centric data collection, RAN2 discusses if management-based MDT and/or signalling-based MDT are assumed to be supported.

## 2.2	Logged data handling after HO/RLF/RRC state transition

RAN2#129 Agreement UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). FFS how to handle idle/inactive and RLF cases and whether we have a unified. UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

In the previous RAN2 meeting, it was agreed that the UE may retain the logged data during handover. If UE retains the logged data during handover, UE may later report the logged data to the NW.

On the other hand, we believe whether UE retains the logged but not report data is dependent on network’s demand and control. For example,

- If UE has already reported most of its logged data to network with few remaining in its buffer, network may not be interested in still fetching the rest of the logged data.

- If UE has not reported at all any logged data to network since logging started, there will be a strong need for network to fetch the logged data in UE buffer.

- If network is interested in the cell edge scenario measurements, it is possible that handover (or RLF) happens before UE can report the logged data to the network.

With respect to the RRC transition or RLF scenarios, we believe the same design as for handover scenario can be applied for the similar reasoning as observed above.

It is upon source cell’s configuration if UE retains the remaining logged data during HO, RLF, or RRC state transition.

After HO/RLF/RRC transition and UE is connected to a new serving gNB, if logged data is available, UE shall indicate the data availability in the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComoplete or RRCSetupCompelete message correspondingly.

After UE connects to the new serving cell after RLF/RRC state transition, and if UE has logged data available, UE indicates the logged data availability in the RRCReestablishmentComplete, RRCResumeComoplete or RRCSetupCompelete message correspondingly.

Another issue is that since both gNB-centric and OAM-centric data collection are supported, the remaining logged data at UE during HO/RLF/RRC transition shall be forwarded by the new serving gNB to either old serving gNB or OAM depending on the case. Note that in legacy logged MDT, the logged data is always transferred to OAM, which is a bit simpler case.

Therefore, from RRC configuration point of view, when the serving gNB configures the UE to retain the logged data during HO/RLF/RRC transition, serving gNB should also provide information to distinguish the consumer of the logged data source (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

After HO/RLF/RRC transition and UE is connected to a new serving gNB, and upon the request from the new serving gNB, when UE transfers the logged data to the network, the same consumer information shall be reported to the new serving gNB, so the serving gNB can determine and forward the logged data to the correct destination.

The consumer of the logged data can be old serving gNB (for gNB-centric data collection) or OAM (for OAM-centric data collection).

When configuring UE to retain the logged data during HO/RLF/RRC state transition, the serving gNB should also provide information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

Upon the request of the new serving gNB, when transferring the logged data to the new serving cell after HO/RLF/RRC state transition, UE also includes the information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

RAN2 sends LS to RAN3/SA5 to trigger the relevant discussion as proposed in the Annex.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Legacy MDT framework only support gNB being requested to collect L3 measurement results. Enhancements to network interface signalling may be needed to support the request of L1 measurement results.

Observation 2	For management-based MDT, OAM configures the MDT configuration to gNB directly without pointing to a specific UE, and it is upon gNB to select the UE according to the configured criteria.

Observation 3	For signalling-based MDT, the data collection is pointing to a specific UE via CN, and gNB configures the UE measurement report accordingly.

Observation 4	The consumer of the logged data can be old serving gNB (for gNB-centric data collection) or OAM (for OAM-centric data collection).

Based on the discussion above, we propose:

Proposal 1	For OAM-centric data collection, RAN2 discusses if management-based MDT and/or signalling-based MDT are assumed to be supported.

Proposal 2	It is upon source cell’s configuration if UE retains the remaining logged data during HO, RLF, or RRC state transition.

Proposal 3	After UE connects to the new serving cell after RLF/RRC state transition, and if UE has logged data available, UE indicates the logged data availability in the RRCReestablishmentComplete, RRCResumeComoplete or RRCSetupCompelete message correspondingly.

Proposal 4	When configuring UE to retain the logged data during HO/RLF/RRC state transition, the serving gNB should also provide information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

Proposal 5	Upon the request of the new serving gNB, when transferring the logged data to the new serving cell after HO/RLF/RRC state transition, UE also includes the information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

Proposal 6	RAN2 sends LS to RAN3/SA5 to trigger the relevant discussion as proposed in the Annex.

# Draft LS to RAN3/ SA5

# Title:	LS on NW side model training data collection for AI based beam management

# Response to:

# Release:	Rel-19

# Work Item:	NR_AIML_Air

Source:	RAN2

To:	RAN3, SA5

Cc:	RAN1, SA2

Contact Person:

#### Name:

#### E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

For the use case of AI based beam management, an AI model could be deployed at gNB to predict beam quality in either spatial domain (“BM-Case1”) or temporal domain (“BM-Case2”).

In order to collect required data for the training of such gNB-sided model, RAN2 agreed to support gNB-centric and OAM-centric data collection. The following agreements have been made:

RAN2#125bis: 1 For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches 2 We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection. 3 RAN2 supports enhancements to MDT for data collection framework for training. RAN2#126: 1. For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. 2. Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model RAN2#127: 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. 2 UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. This is across all use cases 3 When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. 4 Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. 7 On-demand request from the network is supported. 1 The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements) 2 Data collection report will not be transmitted over SRB1. RAN2#127bis: 1 Periodic logging is supported for training data collection procedure in R19 2 Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. 3 Periodic reporting of logged data is not supported. 4 On-demand reporting of the logged measurements will be specified 5 UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. 6 The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. 7 Low priority SRB will be used. 8 Periodic logging is supported for training data collection procedure in R19 9 Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. 10 Periodic reporting of logged data is not supported. 11 On-demand reporting of the logged measurements will be specified 12 UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. 13 The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. 14 For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE. RAN2#128: 1. Focus on the following three radio condition event based logging  L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)  Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)  L1 beam level measurement 2. Measurements on aperiodic CSI resources are not reported for NW sided data collection. 3. Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. 4. The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). 5. The UE reports to the network when buffer is or may become full. 6. The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). RAN2#129:  Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported.  Low power bit indication is supported  Data availability indication is supported.  As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.  UE retains logged data during handover (HO).  UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

Based on RAN2 discussion, RAN2 believes the following aspects may have RAN3 and SA5 impact:

- In OAM centric data collection, MDT framework may be enhanced to support the new type of data logging for training a gNB-sided model for beam prediction.

- In gNB centric data collection, network signalling may be enhanced to support the transfer of logged data from new serving gNB to old serving gNB in case of HO/RLF/RRC state transition.

2. Actions:

To WG RAN3, WG SA5

ACTION:

RAN2 kindly asks RAN3 and SA5 to take the above progress into account in their future work and provide further feedback if any.

3. Date of Next RAN2 Meetings:

RAN2#130		19th – 23rd May 2025	Malta, MT

RAN2#131		25th – 29st Aug 2025	TBC, IN