3GPP TSG-RAN WG2 #129bis	R2-2502369

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Data collection for NW-sided model training

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management and positioning use cases.

# Discussion

## 2.1	Data Collection for Beam Prediction

### 2.1.1	Data collection configuration

Agreements 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

Based on the agreement from last meeting, the UE can be configured to not initiate data collection and logging upon receiving the configuration. In this case, in our perspective the network can dynamically ask the UE to activate a particular configuration using L1/L2 indication. This enables the network to start/stop the data collection and logging flexibly depending on the respective functionality or specific scenario. For example, the trigger for data collection and logging by the network may as well be dependent on some network-side conditions. Without any support for dynamic activation/deactivation, either the UE may be required to collect and log data for a long period than necessary and eventually lead to excess amount of logged data or the data collection and logging must be reconfigured/deconfigured often.

The initial state of data collection and logging configuration can be activated/deactivated. The configurations which are not activated upon configuration can be activated dynamically.

The active data collection and logging configurations can be deactivated by the network.

### 2.1.2	Data Availability Indication

RAN2#128 Agreement - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). RAN2#129 Agreement Low power bit indication is supported Data availability indication is supported. FFS when this would be triggered

#### 2.1.2.1	Trigger for Data Availability Indication

In the previous RAN2 meeting, it was agreed that UE will indicate the data availability to gNB, such that gNB can fetch the logged data. It is still open when the data availability indication should be triggered by the UE, which depends on the specific case:

- Case#1: For both periodic and event-triggered logging, when UE buffer is about to be full (for example, some buffer size threshold can be configured to trigger the report before the buffer is full) or is already full, UE can indicate to the gNB, such that gNB can fetch the logged data in time.

- Case#2: For periodic logging, which is activated by the gNB, the serving gNB naturally knows if logged data is available at UE. In this case, additional trigger of data availability seems not necessary.

- Case#3: For event-triggered data logging, for which gNB is not aware of the start of logging, and the logged data size may not be large enough to trigger the data availability indication in Case#1. In this case, additional trigger of data availability could be beneficial, e.g., after the instance of event-triggered data logging starts or finishes (i.e., the L3 serving cell measurement becomes worse/better than a threshold for TTT time).

- Case#4: After HO/RLF/RRC state transition (discussed with more details in paper R2-2502368), if there is logged data retained, UE will indicate the data availability to the new serving gNB.

- Case#5: the battery is low.

To avoid too frequent data availability indication for considering case#1 #3 #5, a prohibit timer can be configured.

RAN2 confirms/agrees that UE can trigger the data availability report in the following cases (as a complete list):

  - a percentage of maximum buffer size is reached

  - UE’s maximum buffer size is reached (agreed)

  - the event-triggered data logging starts or finishes

  - UE connects to new gNB after HO/RLF/RRC state transition and there is still logged data retained

  - low battery state (agreed)

RAN2 supports prohibit timer to avoid too frequent data availability indication.

RAN2 RRC Running CR 1> if configured to provide assistance information related to logging of L1 measurements: 2> if the UE determines to be in low battery state; or 2> if the memory reserved for the logging of radio measurements becomes full; or 2> if the UE has logged L1 radio measurements available for transmission: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide assistance information related to logging of L1 measurements. Editor's Note: FFS the trigger for the availability indication. Editor's Note: FFS the need to introduce further procedures, e.g. prohibit timers, indication that battery state is not low any longer, etc. UEAssistanceInformation message

In the current RAN2 running CR, the description implies that the low battery state and memory full indication can be indicated to gNB separately from the data availability indication. In our understanding, the low battery state indication and memory full indication is additional “cause” of the data availability indication, which cannot be sent to the gNB if data is not available.

The current running CR indicates that the low battery state can be indicated separately from data logging availability.

RAN2 clarifies if there is a scenario where UE reports “lowBatteryState” or “memoryFull” in LoggedDataCollectionAssistance IE even if no logged data is available thus no “csi-LogMeasAvailable”.

#### 2.1.2.2	Content of Data Availability Indication

The importance of reporting data availability indication to indicate the presence of logged data at the UE for AI/ML training, has been acknowledged and agreed. We would like to highlight the technical efficiency of the two options RAN2 briefly discussed last meeting with respect to the granularity of availability indication:

(a) a single bit indication only,

(b) more than one bit, i,e., composed of additional information related to the logged data.

We understand that although a single bit indication could be a simple solution, it is not future proof considering the large amounts of data logging for training and the limited size of the UE buffer. Besides, in logged MDT, relying on one single bit indication will enforce the gNB to send UEInformationReqeuest multiple times to fetch the logged data chunk by chunk (instead of a burst of UEInformationResponse messages), which is considered inefficient. In our opinion, for the network to efficiently fetch data from the UE, an indication of data availability after the buffer is full is too late, while an indication as soon as any data is available is too early. Without any further information beyond a single bit, the network can only blindly schedule resources for the transfer of training data as the UE buffer management is up to UE implementation, and the actual buffer size of the UE cannot be known to the network.  Additionally, this limited information about data availability gives rise to further challenges in the case of mobility. The UE might either stop logging data completely, e.g., due to a full buffer, or eventually report the availability indication too often after handover if the network is unaware of any information related to the logged data size.

It should be emphasized that the legacy BSR reporting procedure is not suitable for providing NW information on logged AI/ML data, which is stored in a separate buffer, since BSR only reports data being available for transmission at PDCP level and/or RLC level. Therefore, only RRC messages, e.g., UEInformationResponse message, containing logged data which are submitted to PDCP layer would be accounted for in a BSR. Furthermore, the BSR would not distinguish between RRC messages being comprised of logged AI/ML data and other RRC messages pending for transmission in UE’s buffer. Therefore, we think some AI/ML specific availability indication containing more information, at least data size, is necessary.

UE can optionally indicate data size information together with the data availability indication for gNB to fetch the logged data efficiently.

### 2.1.3	Reporting related aspects based on availability indication

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- On-demand request from the network is supported.   FFS details on signalling

According to the last time RAN2 meeting, after logging the data, UE can report it to network based on the on-demand request, and FFS if event-based reporting and if data availability indication should be sent beforehand. In the following figure, we describe the overall data reporting procedure that would work efficiently in our view.

- Step 1: UE can be configured with event that would trigger the data report after UE starts logging the required measurement results. For example, some buffer size threshold (e.g., percentage of maximum buffer size) can be configured to trigger the report before the buffer is full.

- Step 2: UE will first send an availability indication to gNB, such that gNB can be prepared to reserve radio resources for scheduling the UL transmission of logged data. Comparing to let UE transmit the logged data straight away once triggered, this is considered a safer option to not interfere with other regular UL transmissions from the same or other UEs, gNB has the flexibility to fetch the logged data when the UL traffic in the cell is low. Since UE may have logged data for different data collection configurations, e.g. data logged for different UE/NW-side conditions, it is beneficial to provide gNB information on the data collection configurations for which UE has some logged data available for transmission, e.g. by signalling an ID identifying the data collection configuration. The availability indication should - similar to a buffer status report - indicate the amount of logged data (e.g. in bytes) for each data collection configuration (identified by an ID) in order to enable gNB to request some corresponding logged data and allocate UE resources accordingly.

- Step 3: Upon reception of the availability indication, gNB can request the UE to transmit some specific, e.g. for some specific data collection configuration(s), or all the logged data stored in its buffer via UEInformationRequest message

- Step 4: UE will generate and transmit UEInformationResponse message that contains the requested logged data stored in its buffer. If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively.

If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

### 2.1.4	Release of logged data in the buffer

Another aspect that has not been discussed yet is the release of the logged data in the UE buffer. Since it could be possible that the logged data is retained in UE buffer, while the gNB does not fetch for a long time.

#### 5.5a.1.3 Reception of the LoggedMeasurementConfiguration by the UE Upon receiving the LoggedMeasurementConfiguration message the UE shall: 1> discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2; 1> store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig; #### 5.5a.1.4 T330 expiry Upon expiry of T330 the UE shall: 1> release VarLogMeasConfig; The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry. ### 5.5a.2 Release of Logged Measurement Configuration #### 5.5a.2.1 General The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information. #### 5.5a.2.2 Initiation The UE shall initiate the procedure upon receiving a logged measurement configuration in same or another RAT. The UE shall also initiate the procedure upon power off or upon deregistration. The UE shall: 1> stop timer T330, if running; 1> if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

In legacy logged MDT, the logged data will be released upon

- New logged measurement configuration is received

- Release of logged measurement configuration

- Power off or deregistration

- 48 hrs after logging duration is reached

For the logged data in RRC connected state, since multiple data logging configurations (e.g., for different use cases) can be configured at UE, the receiving or release of logged measurement configuration is no longer a proper trigger of logged data release.

RAN2#128 Agreement: Duration is not supported

Also, since RAN2 agreed that the logging duration is not supported, then it is not a relevant factor in this case neither.

On the other hand, considering the handover scenario, where the UE connects to a new serving gNB with logged data retained before handover. The new serving gNB has to always fetch the retained logged data (imagining UE buffer is full) before configuring UE with any new data logging configuration. Considering the consumer of the logged data could be the old gNB, and it is after all the new serving gNB’s decision whether to fetch the logged data retained in UE’s buffer, it could be beneficial and a clean approach to introduce dedicated indication for the serving gNB to release the logged data in UE’s buffer.

The logged data in UE’s buffer will be released upon

  - Power off or deregistration

  - 48 hrs after the release of logged measurement configuration

  - Explicit indication from the serving gNB

  - HO/RLF/RRC state transition if configured by gNB

## 2.2	Data collection for Positioning 3a and 3b

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. RAN1#120 - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

(1st priority) case 3a) NG-RAN node assisted positioning with gNB-side model:

RAN1 has agreed that for case 3a, the label and its related data can be generated by at least LMF. Since it’s an AIML model deployed at gNB-side, gNB can retrieve the training data from LMF as discussed in RAN3.

From RAN2 point of view, RAN2 can assume LPP procedure is used by LMF to collect the training data from UE/PRU. The possible LPP impact could be upon RAN1 discussion. For example, if label comprises LOS/NLOS indication, there may be no further LPP impacts, however if the label also includes PRU SRS Tx Location, then possible LPP impacts are expected to retrieve/update the PRU location. RAN2 is thus suggested to wait for RAN1 progress before discussing the possible LPP impact.

For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. It is only for UL based direct positioning.

During the RAN1#116bis and RAN1#117 meetings, RAN1 assumes the training data (e.g., label and time stamp) for sub use case 3b can be generated by PRU or non-PRU UEs, and how the training data is transferred from PRU to NW is out of RAN1 scope.

From RAN2 point of view, since case 3b is related to LMF-sided AIML model, it seems natural for LMF to collect the required training data (e.g., label and time stamp) from PRU via LPP protocol.

RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved.

For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE (e.g., with estimated SRS Tx transmission location) via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	The current running CR indicates that the low battery state can be indicated separately from data logging availability.

Based on the discussion above, we propose:

Proposal 1	The initial state of data collection and logging configuration can be activated/deactivated. The configurations which are not activated upon configuration can be activated dynamically.

Proposal 2	The active data collection and logging configurations can be deactivated by the network.

Proposal 3	RAN2 confirms/agrees that UE can trigger the data availability report in the following cases (as a complete list):

a.	a percentage of maximum buffer size is reached

b.	UE’s maximum buffer size is reached (agreed)

c.	the event-triggered data logging starts or finishes

d.	UE connects to new gNB after HO/RLF/RRC state transition and there is still logged data retained

e.	low battery state (agreed)

Proposal 4	RAN2 supports prohibit timer to avoid too frequent data availability indication.

Proposal 5	RAN2 clarifies if there is a scenario where UE reports “lowBatteryState” or “memoryFull” in LoggedDataCollectionAssistance IE even if no logged data is available thus no “csi-LogMeasAvailable”.

Proposal 6	UE can optionally indicate data size information together with the data availability indication for gNB to fetch the logged data efficiently.

Proposal 7	If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

Proposal 8	The logged data in UE’s buffer will be released upon

a.	Power off or deregistration

b.	48 hrs after the release of logged measurement configuration

c.	Explicit indication from the serving gNB

d.	HO/RLF/RRC state transition if configured by gNB

Proposal 9	For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion.

Proposal 10	For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 supports LMF collecting training data from PRU or non-PRU UE (e.g., with estimated SRS Tx transmission location) via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation). The detailed LPP impact is pending on RAN1 discussion