3GPP TSG-RAN WG2 Meeting #129bis		         R2-2502400

Wuhan, China, 7th - 11th April 2025

Agenda Item:	8.5.3

Source: 	ITRI

Title:  	Discussion on on-demand SIB1 for handling RLF

Document for:	Discussion

Introduction

In RAN plenary #106 meeting, whether a connected mode UE could support on-demand SIB1 procedure was discussed in RP-243297. The conclusion of this discussion was “RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission”.

In last RAN2 meeting, the agreement related to the RLF and request OD-SIB1 transmission procedure are made as following:

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we will discuss how to verify the validity about the stored UL WUS configuration in SIB-X.

Discussion

When a connected mode UE declares Radio Link Failure (RLF), it attempts to recover the connection by triggering the connection re-establishment procedure. During this procedure, the UE performs cell selection and attempts to camp on a suitable cell within the T311 timer. If it fails to find a suitable cell within this timer, the UE enters idle mode.

To verify the suitability of a selected cell, the UE needs to check the PLMN information within System Information Block 1 (SIB1). Consequently, when a connected mode UE triggers the re-establishment procedure and selects a NES cell that currently does not provide SIB1, it should be able to request On-Demand SIB1 (OD-SIB1) transmission. Otherwise, the UE might spend additional time selecting another cell that does provide SIB1.

In last RAN2 meeting, it was agreed that “When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.” Therefore, the connected mode UE could support OD-SIB1 requests during RLF-triggered re-establishment, if the UE has a valid UL WUS configuration.

As previously agreed, the UL WUS configuration is carried within a new SIB (e.g., SIB-X). Consequently, the validity of the UL WUS configuration should depend on the validity of the stored SIB.

One method for verifying the validity of a stored SIB is the SI change scheme. The network is only permitted to modify SI messages (excluding SI messages for ETWS, CMAS, positioning assistance data, and certain NTN-specific information as specified in the field descriptions) during the modification period following the transmission of the SI change indication. In other words, if a UE receives an SI change notification, it should consider the stored SIB valid until the modification period boundary (MPB) following the reception of the SI change indication.

Therefore, the UE can determine the validity of the stored SIB by checking whether it has received an SI change indication. This same scheme should also be applied to SIB-X.

Proposal 1:

For the stored UL WUS configuration validty check, RAN2 should consider the following:

- If UE receives the SI change notification, the UE can consider the stored UL WUS configuration may be valid before the MPB following the one where SI change indication is received.

- If the UE doesn’t receive the SI change notification, the UE can consider UL WUS configuration may be valid in the next two MPB following the one where SI change indication is received.

Conclusion

Proposal 1:

For the stored UL WUS configuration validty check, RAN2 should consider the following:

-	If UE receives the SI change notification, the UE can consider the stored UL WUS configuration may be valid before the MPB following the one where SI change indication is received.

-	If the UE doesn’t receive the SI change notification, the UE can consider UL WUS configuration may be valid in the next two MPB following the one where SI change indication is received.