3GPP TSG-RAN WG2 Meeting #129bis	R2-2502430

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT random access

Document for:	Discussion and Decision

1 Introduction

In RAN2#129 meeting, RAN2 discussed the random access for A-IoT and achieved the following agreements [1]:

Down-select 3-step CBRA or unified solution For Rel-19, only 3-step CBRA is supported for A-IoT Alignment of CBRA and CFRA We will specify both CBRA and CFRA Assignment for re-access resource Re-use the subsequent paging message to trigger re-access. There is no need to differentiate msg1 resource for initial access vs re-access Success/failure indication NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback MSG1 Resource Selection for FDMA RAN2 assumes that device randomly selects among FDMA occasions as the baseline

In this contribution, we will continue discussion the random access of Ambient IoT.

2 Discussion

## 2.1 Contention-based random access

In last meeting, RAN2 agreed that only 3-step CBRA is supported for A-IoT in Rel-19. For CBRA procedure, slotted-ALOHA based access is baseline. 3-step CBRA random access for A-IoT is shown in Figure 1.

Msg0: Access trigger. The reader sends signal to trigger A-IoT device to perform access. The trigger signal may be paging or other R2D message. The resource for device subsequent transmission may also include in the message.

Msg1: The device sends a random ID to the reader. When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader.

Figure 1 3-step CBRA for A-IoT

Msg2: The reader responds with the successfully received random ID. If reader successful reception the RN16, it sends an ACK which at least carries the same RN16 from the A-IoT device.

Msg3: Device sends Device ID and any other upper layer data (depending on upper layer request). If the A-IoT device receives the same RN16 that it sent in Msg1, it may perform the upper layer data transmission with the reader, which can be the device ID and/or any other upper layer data.

After Msg3 transmission, reader may optional send explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.

In last meeting, for the failure/success feedback indication, NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. But needs for the further study details including whether we need a timer or explicit message and when reader sends feedback. We think explicit message for NACK feedback is better. If no NACK feedback is received from the NW before subsequent paging, the device assumes that the D2R transmission is successful.

Proposal 1: Support explicit message for NACK feedback.

Proposal 2: If no NACK feedback is received from the NW before subsequent paging, the device assumes that the D2R transmission is successful.

## 2.2 Contention-free random access

For dedicated inventory or command for a identified A-IoT device, the specific A-IoT device ID will included in trigger signal in Msg0. Only the device ID of A-IoT device match can initiate access, so it can be a CFRA. CFRA based access procedure is shown in Figure 2.

Figure 2 CFRA for A-IoT

Msg 0: Access trigger. The reader sends trigger signal to trigger A-IoT device to perform access. The trigger signal may be paging or command message which the device ID and resource for RACH are include in.

Msg 1: A-IoT device sends device ID and any other upper layer data to reader.

RAN2 discussed the need of AS ID for CFRA and one of the candidate options is that the device includes a random ID in Msg 1 same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID.

There is no guarantee that, for CFRA procedure, subsequent D2R transmission is needed. If there is no subsequent D2R transmission, then transmission of the random ID in msg1 is totally waste of energy and signalling overhead.

Proposal 3: Does not support the device includes a random ID in Msg 1 for CFRA.

## 2.3 Random access type determination

The A-IoT device determines the random access type (CBRA or CFRA) from the A-IoT paging message. Reader can implicitly indicate access type by access resources, since paging message can include a number of msg1 resources. For CBRA, the number of access occasion (s) is more than one. For CFRA, R19 AIoT only support to page one device. So the number of access occasion for CFRA is only one.

Proposal 4: Device can determine random access type (CBRA and CFRA) through the allocated access resources.

3 Conclusion

In this contribution we discuss the random access procedure of A-IoT device, with the following proposals:

Proposal 1: Support explicit message for NACK feedback.

Proposal 2: If no feedback is received from the NW before subsequent paging, the device assumes that the D2R transmission is successful.

Proposal 3: Does not support the device includes a random ID in Msg 1 for CFRA.

Proposal 4: Device can determine random access type (CBRA and CFRA) through the allocated access resources.

- References

- Draft report of 3GPP TSG RAN WG2 #129, RAN2 chair (interDigtal), Athens, Greece, Feb. 17th – 21st, 2025.