3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502440

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.0

Source:	Huawei, HiSilicon

Title:	R19 ASN.1 identifier naming conventions and check list

Document for: Discussion and Decision

1	Introduction

Following issue is discussed about the ASN.1 identifier naming last RAN2 meeting.

R2-2500881	Miscellaneous non-controversial corrections Set XXIV	Ericsson	CR	Rel-18	38.331	18.4.0	5236	-	F	NR_newRAT-Core, TEI18

-	Ericsson indicates that there a few other issues and a discussion on ASN.1 naming problems.  Lenovo explains that we should be careful on fixing these issues as it impacts other specs.

-	Ericsson thinks that we should fix inconsistencies but fixing “-“ etc can create more errors.

- We will schedule a training next meeting for ASN.1 naming and other rules for Rel-19 rapporteurs

- For Rel-18, we will not do a big exercise to fix minor RRC parameter naming problems.

- Aim to align the parameters across the specs if we find an inconsistency.  Take this case by case and we aim to minimize specs and groups impacted.

- For Rel-19, we should pay careful attention and check within the ASN.1 review phase.  We should do this better in Rel-19.

In this contribution, we clarify again the ASN.1 identifier naming convention principles, and additionally give some typical inappropriate naming for information.

2	Discussion

2.1	ASN.1 identifier naming convention principles

The ASN.1 identifier naming convention principles are somehow captured in TS 38.331 A.3.1.2.

### A.3.1.2 ASN.1 identifier naming conventions The naming of identifiers (i.e., the ASN.1 field and type identifiers) should be based on the following guidelines: - Message (PDU) identifiers should be ordinary mixed case without hyphenation. These identifiers, e.g., the RRCConnectionModificationCommand, should be used for reference in the procedure text. Abbreviations should be avoided in these identifiers and abbreviated forms of these identifiers should not be used. - Type identifiers other than PDU identifiers should be ordinary mixed case, with hyphenation used to set off acronyms only where an adjacent letter is a capital, e.g., EstablishmentCause, SelectedPLMN (not Selected-PLMN, since the "d" in "Selected" is lowercase), InitialUE-Identity and MeasSFN-SFN-TimeDifference. - Field identifiers shall start with a lowercase letter and use mixed case thereafter, e.g., establishmentCause. If a field identifier begins with an acronym (which would normally be in upper case), the entire acronym is lowercase (plmn-Identity, not pLMN-Identity). The acronym is set off with a hyphen (ue-Identity, not ueIdentity), in order to facilitate a consistent search pattern with corresponding type identifiers.

We clarify the above conventions into the check list of principle for different identifiers

2.1.1	Message (PDU) identifier

Message (PDU) identifier follows below principles: - first letter should be capital; - it is ordinary mixed case; - there is no hyphenation; - avoid abbreviation (but acronym is fine).

2.1.2	Type identifier (other than PDU identifier)

Type identifier (other than PDU identifier) follows below principles: - first letter should be capital; - it is ordinary mixed case; - use hyphenation to set off acronyms, where an adjacent letter is a capital; - both abbreviation and acronym can be used.

2.1.3	Field identifier

Field identifier follows below principles: - first letter should be a lowercase letter; - use mixed case thereafter; - if a field identifier begins with an acronym, the entire acronym is lowercase; - use hyphenation to set off acronyms, where an adjacent letter is a capital; - both abbreviation and acronym can be used.

Proposal 1a:	It is proposed to endorse the above clarifications regarding the ASN.1 identifier naming principles, in accordance with TS 38.331 A.3.1.2. These clarifications should be refereed by the ASN.1 review section of the RAN2 handbook. Specifically, RRC running CR rapporteurs are encouraged to take note of these clarifications.

2.2	Check list of typical inappropriate naming

Sometimes, the principles are easy to understand but also easy to be forgotten. In following, we summarize the typical inappropriate examples to remind the RRC running CR rapporteurs to check the used ASN.1 identifier.

2.2.1	Message (PDU) identifier

Some examples of typical inappropriate naming: - first letter is lowercase; ✘ - there is any hyphenation; ✘ - not ordinary mixed case, i.e. the first letter of the ordinary word is not capitalized; ✘

2.2.2	Type identifier (other than PDU identifier)

Some examples of typical inappropriate naming: - first letter is lowercase; ✘ - void - miss hyphenation after acronym, where an adjacent letter is a capital; ✘ - use hyphenation after one ordinary word (or abbreviation), which ends with lowercase letter; ✘ - use lowercase letter after one hyphenation; ✘ - void - not ordinary mixed case, i.e. the first letter of the ordinary word is not capitalized; ✘

The above bullet 3, 4, 5 are quite typical, with some of the examples of bullet 4 shown below:

- Bullet 4: “MultiFrequencyBandListNR-Aerial-SIB-r18”, “AdvancedReceiver-MU-MIMO-r18”, “Aerial-Config-r18”, “SRS-PosTx-Hopping-r18” and “SuccessPSCell-Config-r18”, etc.

2.2.3	Field identifier

Some examples of typical inappropriate naming: - first letter is capital; ✘ - When beginning with an acronym, this entire acronym does not use lowercase; ✘ - miss hyphenation after acronym, where an adjacent letter is a capital; ✘ - use hyphenation after one ordinary word (or abbreviation), which ends with lowercase letter; ✘ - use lowercase letter after one hyphenation; ✘ - miss suffix in the end of the identifier (e.g., “-r18” or “-v18xy”), for versions later than R15; ✘ - not ordinary mixed case, i.e. the first letter of the ordinary word (other than the first word) is not capitalized; ✘

The above bullet 3, 4, 5 and 6 are quite typical, with some of the examples below:

- Bullet 3: “pagingPTWLength-r18”, “ntnCovEnh-r18”, “nesEvent-r18”, “lowerMSDRequest-r18”, etc.

- Bullet 4: “aerial-Config-r18”, “srs-PosTx-Hopping-r18”, “msg1-Repetitions-Priority-r18”, “preambleTransMax-Msg1-Repetition-r18”, etc.

- Bullet 5: “hs-ATG-cellReselectionSet-r18”, “additionalRACH-perPCI-ToAddModList-r18”, “n1-n2-codebookSubsetRestrictionList-r18”, “ssb-perRACH-Occasion-r18”, etc.

- Bullet 6: “p0alpha” in semiPersistentOnPUSCH-r18, “windowDuration” in MeasWindowConfig-r18.

Based on the above summary, we have below observation and proposal:

Observation 1:	The above bullet 3, 4, 5 and 6 are the most common inappropriateness, we observed in the previous ASN.1 reviews and some already used identifier naming.

Proposal 1b:	It is proposed to endorse the above examples of typical inappropriate ASN.1 identifier naming and capture those into TS 38.331 section A.3.1.2 and add reference to this in the RAN2 handbook for ASN.1 review part.

2.3	Autonomous check by the macro code

In this section, we are trying to find if it is possible to autonomous find the possible inappropriate ASN.1 identifier naming by MS word Macro code.

Some of the above typical inappropriate is not that easy to be checked by program, e.g. bullet 2 to check acronym, where manual check is still required.

In the below codes, mainly bullets 1, 3, 4, 5, 6 are checked by the VBA code.

The RRC running CR rapporteur can:

- Open the VBA editor: In Word, press the Alt + F11 key combination, and the VBA editor window will pop up.

- Copy and paste the code: Copy the codes into the newly created module.

- Select the word to be checked: In the Word document, use the mouse to select one word that you want to perform the rule check on.

- Run the macro: Press the Alt + F8 key combination to bring up the "Macros" dialog box. Find the CheckType or CheckField macro in the list, and then click the "Run" button.

- The macro will remind you if it finds any possible inappropriate ASN.1 identifier.

It is proposed:

Proposal 2:	R19 RRC running CR rapporteurs can consider to use the below VBA Macros to autonomously check if there are some typical inappropriate ASN.1 naming.

2.3.1	Type identifier (other than PDU identifier)

The VBA macro code is copied here:

Sub CheckType()

Dim selectedWord As String

Dim issues As String

Dim i As Long

Dim consecutiveCaps As Integer

Dim isConsecutiveCaps As Boolean

Dim hasValidSuffix As Boolean

Dim consecutiveCapStr As String

Dim hyphenBeforeStr As String

Dim hyphenAfterStr As String

' Get the currently selected word

selectedWord = Trim(Selection.Text)

issues = ""

hasValidSuffix = False

' Condition 1: The first letter is lowercase

If Asc(Left(selectedWord, 1)) >= 97 And Asc(Left(selectedWord, 1)) <= 122 Then

issues = issues & "1. The first letter is lowercase" & vbNewLine

End If

' Condition 3: Missing hyphenation after acronym where an adjacent letter is a capital

consecutiveCaps = 0

isConsecutiveCaps = False

consecutiveCapStr = ""

For i = 1 To Len(selectedWord)

If Asc(Mid(selectedWord, i, 1)) >= 65 And Asc(Mid(selectedWord, i, 1)) <= 90 Then

consecutiveCaps = consecutiveCaps + 1

consecutiveCapStr = consecutiveCapStr & Mid(selectedWord, i, 1)

If consecutiveCaps >= 2 Then

isConsecutiveCaps = True

End If

Else

If isConsecutiveCaps And Mid(selectedWord, i, 1) <> "-" Then

issues = issues & "3. Miss hyphenation after acronym, where an adjacent letter is a capital: {" & consecutiveCapStr & "}" & vbNewLine

End If

consecutiveCaps = 0

isConsecutiveCaps = False

consecutiveCapStr = ""

End If

Next i

If isConsecutiveCaps Then

issues = issues & "3. Missing hyphenation after consecutive capital letters. The consecutive capital letters are:{ " & consecutiveCapStr & "}" & vbNewLine

End If

' Condition 4: Using hyphenation after an ordinary word (or abbreviation) ending with a lowercase letter

For i = 1 To Len(selectedWord) - 1

If Asc(Mid(selectedWord, i, 1)) >= 97 And Asc(Mid(selectedWord, i, 1)) <= 122 And Mid(selectedWord, i + 1, 1) = "-" Then

If i + 3 <= Len(selectedWord) Then

If Not ((Mid(selectedWord, i + 2, 1) = "r" Or Mid(selectedWord, i + 2, 1) = "v") And _

IsNumeric(Mid(selectedWord, i + 3, 1)) And IsNumeric(Mid(selectedWord, i + 4, 1))) Then

hyphenBeforeStr = Mid(selectedWord, i, 2)

issues = issues & "4. use hyphenation after one ordinary word (or abbreviation), which ends with lowercase letter. Please check if the word is an acronym:{ " & hyphenBeforeStr & "}" & vbNewLine

End If

Else

hyphenBeforeStr = Mid(selectedWord, i, 2)

issues = issues & "4. use hyphenation after one ordinary word (or abbreviation), which ends with lowercase letter. Please check if the word is an acronym: {" & hyphenBeforeStr & "}" & vbNewLine

End If

End If

Next i

' Condition 5: Using a lowercase letter after a hyphenation

For i = 1 To Len(selectedWord) - 1

If Mid(selectedWord, i, 1) = "-" Then

If i + 3 <= Len(selectedWord) Then

If Not ((Mid(selectedWord, i + 1, 1) = "r" Or Mid(selectedWord, i + 1, 1) = "v") And _

IsNumeric(Mid(selectedWord, i + 2, 1)) And IsNumeric(Mid(selectedWord, i + 3, 1))) Then

If Asc(Mid(selectedWord, i + 1, 1)) >= 97 And Asc(Mid(selectedWord, i + 1, 1)) <= 122 Then

hyphenAfterStr = Mid(selectedWord, i, 2)

issues = issues & "5. Using a lowercase letter after a hyphen: {" & hyphenAfterStr & "}" & vbNewLine

End If

End If

Else

If Asc(Mid(selectedWord, i + 1, 1)) >= 97 And Asc(Mid(selectedWord, i + 1, 1)) <= 122 Then

hyphenAfterStr = Mid(selectedWord, i, 2)

issues = issues & "5. Using a lowercase letter after a hyphen: {" & hyphenAfterStr & "}" & vbNewLine

End If

End If

End If

Next i

' Remove the last newline character

If issues <> "" Then

issues = Left(issues, Len(issues) - 2)

End If

If issues <> "" Then

MsgBox "The naming may be inappropriate. The following issues occurred:" & vbNewLine & vbNewLine & issues, vbExclamation

Else

MsgBox "No abnormalities were found. Please manually check if it meets other unexamined rules.", vbInformation

End If

End Sub

2.3.2	Field identifier

The VBA macro code is copied here:

Sub CheckField()

Dim selectedWord As String

Dim issues As String

Dim i As Long

Dim consecutiveCaps As Integer

Dim isConsecutiveCaps As Boolean

Dim hasValidSuffix As Boolean

Dim consecutiveCapStr As String

Dim hyphenBeforeStr As String

Dim hyphenAfterStr As String

' Get the currently selected word

selectedWord = Trim(Selection.Text)

issues = ""

hasValidSuffix = False

' Condition 1: The first letter is capital

If Asc(Left(selectedWord, 1)) >= 65 And Asc(Left(selectedWord, 1)) <= 90 Then

issues = issues & "1. The first letter is capital" & vbNewLine

End If

' Condition 3: Missing hyphenation after acronym where an adjacent letter is a capital

consecutiveCaps = 0

isConsecutiveCaps = False

consecutiveCapStr = ""

For i = 1 To Len(selectedWord)

If Asc(Mid(selectedWord, i, 1)) >= 65 And Asc(Mid(selectedWord, i, 1)) <= 90 Then

consecutiveCaps = consecutiveCaps + 1

consecutiveCapStr = consecutiveCapStr & Mid(selectedWord, i, 1)

If consecutiveCaps >= 2 Then

isConsecutiveCaps = True

End If

Else

If isConsecutiveCaps And Mid(selectedWord, i, 1) <> "-" Then

issues = issues & "3. Miss hyphenation after acronym, where an adjacent letter is a capital: {" & consecutiveCapStr & "}" & vbNewLine

End If

consecutiveCaps = 0

isConsecutiveCaps = False

consecutiveCapStr = ""

End If

Next i

If isConsecutiveCaps Then

issues = issues & "3. Missing hyphenation after consecutive capital letters. The consecutive capital letters are:{ " & consecutiveCapStr & "}" & vbNewLine

End If

' Condition 4: Using hyphenation after an ordinary word (or abbreviation) ending with a lowercase letter

For i = 1 To Len(selectedWord) - 1

If Asc(Mid(selectedWord, i, 1)) >= 97 And Asc(Mid(selectedWord, i, 1)) <= 122 And Mid(selectedWord, i + 1, 1) = "-" Then

If i + 3 <= Len(selectedWord) Then

If Not ((Mid(selectedWord, i + 2, 1) = "r" Or Mid(selectedWord, i + 2, 1) = "v") And _

IsNumeric(Mid(selectedWord, i + 3, 1)) And IsNumeric(Mid(selectedWord, i + 4, 1))) Then

hyphenBeforeStr = Mid(selectedWord, i, 2)

issues = issues & "4. use hyphenation after one ordinary word (or abbreviation), which ends with lowercase letter. Please check if the word is an acronym:{ " & hyphenBeforeStr & "}" & vbNewLine

End If

Else

hyphenBeforeStr = Mid(selectedWord, i, 2)

issues = issues & "4. use hyphenation after one ordinary word (or abbreviation), which ends with lowercase letter. Please check if the word is an acronym: {" & hyphenBeforeStr & "}" & vbNewLine

End If

End If

Next i

' Condition 5: Using a lowercase letter after a hyphenation

For i = 1 To Len(selectedWord) - 1

If Mid(selectedWord, i, 1) = "-" Then

If i + 3 <= Len(selectedWord) Then

If Not ((Mid(selectedWord, i + 1, 1) = "r" Or Mid(selectedWord, i + 1, 1) = "v") And _

IsNumeric(Mid(selectedWord, i + 2, 1)) And IsNumeric(Mid(selectedWord, i + 3, 1))) Then

If Asc(Mid(selectedWord, i + 1, 1)) >= 97 And Asc(Mid(selectedWord, i + 1, 1)) <= 122 Then

hyphenAfterStr = Mid(selectedWord, i, 2)

issues = issues & "5. Using a lowercase letter after a hyphen: {" & hyphenAfterStr & "}" & vbNewLine

End If

End If

Else

If Asc(Mid(selectedWord, i + 1, 1)) >= 97 And Asc(Mid(selectedWord, i + 1, 1)) <= 122 Then

hyphenAfterStr = Mid(selectedWord, i, 2)

issues = issues & "5. Using a lowercase letter after a hyphen: {" & hyphenAfterStr & "}" & vbNewLine

End If

End If

End If

Next i

' Condition 6: Missing suffix at the end of the identifier for versions later than R15

For i = 1 To Len(selectedWord) - 3

If Mid(selectedWord, i, 1) = "-" Then

If (Mid(selectedWord, i + 1, 1) = "r" Or Mid(selectedWord, i + 1, 1) = "v") And _

IsNumeric(Mid(selectedWord, i + 2, 1)) And IsNumeric(Mid(selectedWord, i + 3, 1)) Then

hasValidSuffix = True

Exit For

End If

End If

Next i

If Not hasValidSuffix Then

issues = issues & "6. Missing the suffix of -r19 or -v19" & vbNewLine

End If

' Remove the last newline character

If issues <> "" Then

issues = Left(issues, Len(issues) - 2)

End If

If issues <> "" Then

MsgBox "The naming may be inappropriate. The following issues occurred:" & vbNewLine & vbNewLine & issues, vbExclamation

Else

MsgBox "No abnormalities were found. Please manually check if it meets other unexamined rules.", vbInformation

End If

End Sub

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1a:	It is proposed to endorse the above clarifications regarding the ASN.1 identifier naming principles, in accordance with TS 38.331 A.3.1.2. These clarifications should be refereed by the ASN.1 review section of the RAN2 handbook. Specifically, RRC running CR rapporteurs are encouraged to take note of these clarifications.

Observation 1:	The above bullet 3, 4, 5 and 6 are the most common inappropriateness, we observed in the previous ASN.1 reviews and some already used identifier naming.

Proposal 1b:	It is proposed to endorse the above examples of typical inappropriate ASN.1 identifier naming and capture those into TS 38.331 section A.3.1.2 and add reference to this in the RAN2 handbook for ASN.1 review part.

Proposal 2:	R19 RRC running CR rapporteurs can consider to use the below VBA Macros to autonomously check if there are some typical inappropriate ASN.1 naming.