3GPP TSG-RAN WG2 Meeting #129-Bis	R2-250xxxx

Wuhan, China, 7 – 11 April, 2025

Agenda item:	8.7.6

Source:	InterDigital

Title:	Discussion on UL congestion signaling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2#128 agreed the following:

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

Furthermore, SA4 replied to RAN2 LS with the typical UL data rates and granularity in R2-2500056/S2-242182:

# 1. Overall Description: In R2-2409273, a question from RAN2 is: RAN2 are discussing MAC-layer signaling enhancements to enable XR applications to adapt bit rates in response to uplink congestion. To proceed with the design, RAN2 would like to request input from SA4 on the appropriate range and granularity of uplink bit rates that need to be supported for the signaling enhancements. RAN2 respectfully requests SA4 to provide input on the appropriate range and granularity of uplink bit rates that need to be supported in the aforementioned signaling enhancements. The SA4 reply is as follows: The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

RAN2#129 further reached the following agreements:

- RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

# 2	Discussion

## 2.1	Indication granularity

In the RAN2#127-Bis meeting, it was discussed whether the UL congestion signaling granularity should be on a per DRB or per QoS flow level, while interworking with L4S (which works in QoS flow level) was to be considered as well. RAN2#128 further agreed the rate indication can be per QoS flow but left FFS whether such indication is provided over MAC or RRC signaling. Since RAN plenary agreed the congestion signaling to be at the MAC layer in the RAN, it seems simplest to use the per DRB/logical channel (LCH) granularity which is what the MAC sees today. Considering in going to per QoS flow level, the signaling layer being the MAC could be easily questioned: MAC does not see QoS flows in neither the UE nor at the NW side; on the other hand, this would also require signaling support in F1 interface to make MAC layer aware of the congested QoS flow(s) in the NW. The RRC signaling can be used to configure which QoS flows to throttle on a per DRB basis. If NW does not provide any configuration on QoS flows to throttle, it can be left to UE to throttle QoS flow(s) by implementation.

Observation: MAC does not see QoS flow(s) in neither the UE nor the NW side, hence, supporting QoS flow level UL congestion signaling at MAC layer would require signaling support in F1 interface.

Proposal 1: The UL congestion indication is signaled on a per DRB/LCH basis by MAC CE and RRC signaling is used to configure which QoS flows to throttle on a per DRB basis.

## 2.2	UL congestion signaling options

The Recommended Bit Rate procedure has been supported since Rel-15, and it can support the requirements of the UL congestion signaling in all other means, but the bit rate values. Hence, the Recommended Bit Rate procedure can be used for the new congestion indication.

Proposal 2: The Recommended Bit Rate procedure is used for the UL congestion signaling.

The Recommended Bit Rate MAC CE is specified as follows (TS 38.321):

#### 6.1.3.20 Recommended bit rate MAC CE The Recommended bit rate MAC CE is identified by a MAC subheader with LCID as specified in Tables 6.2.1-1 and 6.2.1-2 for bit rate recommendation message from the gNB to the UE and bit rate recommendation query message from the UE to the gNB, respectively. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.20-1): - LCID: This field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits; - Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink; - Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate; - X: Bit rate multiplier. For UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331 [5]. - R: reserved bit, set to 0. Figure 6.1.3.20-1: Recommended bit rate MAC CE

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field Index; NR Recommended Bit Rate value [kbit/s]; Index; NR Recommended Bit Rate value [kbit/s]. 0; Note 1; 32; 700. 1; 0; 33; 800. 2; 9; 34; 900. 3; 11; 35; 1000. 4; 13; 36; 1100. 5; 17; 37; 1200. 6; 21; 38; 1300. 7; 25; 39; 1400. 8; 29; 40; 1500. 9; 32; 41; 1750. 10; 36; 42; 2000. 11; 40; 43; 2250. 12; 48; 44; 2500. 13; 56; 45; 2750. 14; 72; 46; 3000. 15; 88; 47; 3500. 16; 104; 48; 4000. 17; 120; 49; 4500. 18; 140; 50; 5000. 19; 160; 51; 5500. 20; 180; 52; 6000. 21; 200; 53; 6500. 22; 220; 54; 7000. 23; 240; 55; 7500. 24; 260; 56; 8000. 25; 280; 57; Reserved. 26; 300; 58; Reserved. 27; 350; 59; Reserved. 28; 400; 60; Reserved. 29; 450; 61; Reserved. 30; 500; 62; Reserved. 31; 600; 63; Reserved. Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given..

Since the X field follows the Bit Rate field in the current MAC CE, it seems not a good option to extend the Bit Rate field – this would mean that a new MAC CE would be required for the purpose. Hence, simplest approach is to define the new bit rate table based on the current 6 bits available. Furthermore, to achieve as close to 1% granularity as possible based on the SA4 LS reply, it seems good to define new values for the bitRateMultiplier – ie., better granularity can be achieved by defining both.

Proposal 3: The new bit rate table is defined based on the 6 bits field as in the legacy.

Proposal 4: To achieve finer granularity, define new values for the bitRateMultiplier on top of the new bit rate table.

It should further be discussed whether the old bit rate table values/bitRateMultiplier values should be able to be used concurrently with the new values defined to support the XR applications. If yes, some dynamic indication would be needed to indicate which values to use (e.g., by exploiting one of the Reserved bits in the MAC CE or define a new (e)LCID). Otherwise, the current MAC CE can be used as is by RRC configuration on which table and bitRateMultiplier values to be used. Since the bit rates to be indicated to XR applications would be generally smaller and with better granularity to the legacy values, the RRC configuration options seems sufficiently dynamic for the purpose if NW intends to use the new UL congestion signalling for the UE.

Proposal 5: RRC configures whether the new bit rate table and/or bitRateMultiplier value is used and the legacy Recommended Bit Rate MAC CE is used as is for the UL congestion signaling.

# 3	Conclusion

This document has made the following observations and proposals:

Observation: MAC does not see QoS flow(s) in neither the UE nor the NW side, hence, supporting QoS flow level UL congestion signaling at MAC layer would require signaling support in F1 interface.

Proposal 1: The UL congestion indication is signaled on a per DRB/LCH basis by MAC CE and RRC signaling is used to configure which QoS flows to throttle on a per DRB basis.

Proposal 2: The Recommended Bit Rate procedure is used for the UL congestion signaling.

Proposal 3: The new bit rate table is defined based on the 6 bits field as in the legacy.

Proposal 4: To achieve finer granularity, define new values for the bitRateMultiplier on top of the new bit rate table.

Proposal 5: RRC configures whether the new bit rate table and/or bitRateMultiplier value is used and the legacy Recommended Bit Rate MAC CE is used as is for the UL congestion signaling.