3GPP TSG RAN WG2 Meeting #129bis                          	   R2-2502540

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SSB SCell Operation

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the follows [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and the potential RAN2 specification work.

# Discussion

## MAC CE for On-demand SSB transmission indication and termination

On-demand SSB is transmitted when it is triggered, e.g., for SCell time/frequency synchronization, L1/L3 measurements, SCell selection or activation, etc.

The following agreement on gNB’s indication of on-demand SSB transmission was agreed at RAN1 #117 [2].

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. FFS: Scenarios where the above signalings are applicable

At RAN1 #119, the following agreements were made for on-demand SSB deactivation [4].

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 FFS: Whether the value of N can be implicitly determined using a timer

Based on RAN1’s agreements, on-demand SSB transmission may be indicated and terminated explicitly by MAC CE.

Observation 1. On-demand SSB transmission may be indicated and terminated explicitly by MAC CE.

To support indication and termination of OD-SSB transmission, there are two design options as the follows:

Option 1: one MAC CE for both indication and termination of OD-SSB transmission

Option 2: two MAC CEs, one for indication and the other for termination of OD-SSB transmission

In our view, OD-SSB transmission indication and termination are two sides of a coin for design consideration, especially with the support of multiple Scells. The MAC CE for OD-SSB transmission indication structure may also be appliable to MAC CE for OD-SSB transmission termination.

Observation 2. To support on-demand SSB transmission indication and termination via MAC CE, a unified MAC CE design should be supported.

At RAN2 #127, the following agreements were made for indication of on-demand SSB transmission [3].

Agreement OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Based on RAN1 and RAN2 agreements, as well as the observations, the MAC-CE for indication and termination of on-demand SSB transmission may include at least the following fields.

- Field 1: bitmap for Scells or Scell groups

- Field 2: 1-bit for indication or termination of OD-SSB transmission for each Scell, e.g., "1" for indicating OD-SSB transmission and "0" for terminating OD-SSB transmission.

- Field 2: a periodicity of the OD-SSB, e.g., one of the candidate OD-SSB periodicity values configured in RRC

  - Option 1: One OD-SSB periodicity for one Scell

  - Option 2: One OD-SSB periodicity for a group of Scells, e.g., a Scell group is configured with the same candidate OD-SSB periodicity values such as Group 1 or Group 2 shown below.

    - Group 1 = {Cell 0, Cell 1} with candidate OD-SSB periodicity values = {5, 20, 40, 80} ms

    - Group 2 = {Cell 2, Cell 3} with candidate OD-SSB periodicity values = {5, 10, 20, 40, 80, 160} ms

  - The size of the field indicating the OD-SSB periodicity may be based on the number of configured candidate OD-SSB periodicity values for an Scell or an Scell group or the maximum number of configured candidate OD-SSB periodicity values for all Scells or Scell groups.

Based on above analysis, we propose the following proposals.

Proposal 1: For OD-SSB MAC CE design, RAN2 supports one unified MAC CE for both indication and termination of OD-SSB transmission.

Proposal 2: For a unified MAC CE design, at least the following fields are supported:

- Field 1: IDs or bitmap for Scells or Scell groups

- Field 2: one bit for indication and termination of OD-SSB transmission for each Scell

- Field 3: one of the candidate OD-SSB periodicity values configured in RRC

  - Option 1: One OD-SSB periodicity for one Scell

  - Option 2: One OD-SSB periodicity for a group of Scells

## L3 measurement for OD-SSB Case #1

At RAN2 #129, the following agreements were made for the L3 RRM measurement for adaptation of SSB in time-domain [5].

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

The SSB burst structure of SSB adaptation in time domain with the periodicity adapted is very similar to OD-SSB, thus the SMTC based framework agreed for SSB adaptation in time domain may also be applied to OD-SSB Case #1.

Observation 3. For OD-SSB case 1 (without always-on SSB), the same SMTC based framework agreed for SSB adaptation in time domain may be applied to SCell.

# Conclusion

In this contribution, we discussed on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and potential RAN2 specification work and concluded with the following observations and proposals.

MAC CE for On-demand SSB transmission indication

Observation 1. On-demand SSB transmission may be indicated and terminated explicitly by MAC CE.

Observation 2. To support on-demand SSB transmission indication and termination via MAC CE, a unified MAC CE design should be supported.

Proposal 1: For OD-SSB MAC CE design, RAN2 supports one unified MAC CE for both indication and termination of OD-SSB transmission.

Proposal 2: For a unified MAC CE design, at least the following fields are supported:

- Field 1: IDs or bitmap for Scells or Scell groups

- Field 2: one bit for indication and termination of OD-SSB transmission for each Scell

- Field 3: one of the candidate OD-SSB periodicity values configured in RRC

  - Option 1: One OD-SSB periodicity for one Scell

  - Option 2: One OD-SSB periodicity for a group of Scells

L3 measurement for OD-SSB Case #1

Observation 3. For OD-SSB case 1 (without always-on SSB), the same SMTC based framework agreed for SSB adaptation in time domain may be applied to SCell.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Chair notes RAN1#117 eom0” RAN1 #117, Fukuoka, Japan, May 20th – 24th, 2024, 2024

[3] R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127, Maastricht, NL, August 19th – 23rd, 2024

[4] “Chair notes RAN1#119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024

[5] “R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129, Athens, Greece, February 17th – 21st, 2025