3GPP TSG-RAN2 Meeting #129bis	R2-2502560

# Wuhan, China, Apr 7th – 12th, 2025

Agenda item:		8.17

Work Item:			IoT_NTN_TDD

Source:	Nordic Semiconductor ASA

Title:					On SI scheduling, H-SFN change and postponing impacts in IoT-NTN TDD mode

Document for:		Discussion and Decision

# Introduction

RAN2#129 started to discuss IoT-NTN TDD mode and had the following agreements:

Agreements:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- RAN2 can continue the discussion also on RAR window

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

This paper discusses SI scheduling principles, H-SFH change and its possible impact to IDLE mode eDRX as well as paging in IoT-NTN TDD mode.

# SI scheduling

RAN1#120 had the following conclusion:

R1-2501614 Feature lead summary #3 on IoT-NTN TDD mode Moderator (Qualcomm Incorporated) Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

The [1] presented and discussed SIB1-NB scheduling in detail and proposed that in general there is no need to update the SIB1-NB scheduling. Used of additionalTransmissionSIB1-r15 is always possible (with a cost of reduced available “D NB-IoT subframes” for other purposes), which would help the acquisition of the SIB1-NB even further when 16 repetitions are configured. The SIB1-NB subframes that are scheduled on “non-D NB-IOT subframes” can plain be dropped while the UE can still receive SIB1-NB in maximum 2560ms time and typically based on our simulations only after few “D NB-IoT subframes” containing a SIB1-NB repetition.

Proposal-1: When operating in IoT-NTN TDD mode the scheduling mechanism of SIB1-NB remains unchanged.

Proposal-2: The SIB1-NB subframes that fall into “non-D NB-IOT subframes” are plain dropped.

A SIB2-NB and a SIB31-NB used in NTN context both are too large to fit into TBS of 120bits or less, thus a minimum of 208bits TBSes must be used. This implies 8 subframes are needed for transmission of SIB2-/SIB31-NB. A 90ms TDD frame has maximum 6 PSDCH and minimum 3 PDSCH “D NB-IoT subframes” available for other SI. With additionalTransmissionSIB1-r15 there are maximum 9 “D NB-IoT subframes” and without additionalTransmissionSIB1 10 “D NB-IoT subframes” PDSCH available in two consecutive TDD frames. This implies at least two 90ms TDD frames (and 160ms or longer si-WindowLength) are always needed to transfer another SI such as SIB2-NB. Other SI subframes that do not fit into one set of available “D NB-IoT subframes” or fall into “non-D NB-IOT subframes” would need to be postponed into the next 90ms TDD frame. The scheduling of other SI should follow the existing procedures but allow postponing the SI subframes that fall on “non-D NB-IOT subframes” next available subframe even if located on the next 90ms TDD frame within the SI-window.

Observation-1: A transmission of one “other SI” message always takes two 90ms TDD frames.

Observation-2: Postponing of other SI subframes to the next 90ms TDD frame is required.

Proposal-3: The scheduling procedure of other SI messages remains unchanged.

Proposal-4: Postponing of other SI subframes to the next 90ms TDD frame is supported.

To avoid further complicating the IoT-NTN TDD mode solution the network configuration should be such that SI repetitions within the SI window never overlap, and in a case SI transmission would overflow the SI-window the excess SI subframes are plain discarded.

Proposal-5: A network configuration shall ensure SI repetitions within a SI-window never overlap.

Proposal-6: Other SI transmission that overflows a configured SI-window shall be truncated.

# H-SFN and IDLE eDRX

RAN1#120 had the following agreement:

Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. R1-2501367 Feature lead summary #2 on IoT-NTN TDD mode Moderator (Qualcomm Incorporated) Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact.

RAN1 has already concluded that a UE can derive the 90ms TDD frame observing the downlink signalling. When the H-SFN changes and a UE wakes up to monitor the PTW the UE typically has already woken up earlier for a moment to sync itself, subject to accuracy of the clock the UE has. Either the UE already is in sync with the 90ms TDD framing or observes it roughly within first few 90ms TDD frames after starting to monitor the PTW. If the UE attempts to sync itself while already within the PTW it may miss a PO if there is only one PO within the PTW.

Observation-3: The UE can sync itself to the 90ms TDD framing prior H-SFN change or shortly after entering the PTW.

Proposal-7: A UE derives the 90ms TDD frame timing by the observation of downlink signals.

During PTW when a UE is monitoring for a page downlink can be packed with control signals, SIs, possible invalid downlink subframes and be overlapped by other search spaces. Assuming existing procedures for calculating POs, they easily occur on a “non-D NB-IoT subframes”, control signals, SIs or collide with other search spaces. Postponing a PO to the next available NPDCCH candidate for the PO has a minimal specification impact and requires a small implementation change on both UE and eNB. The postponing continues until an available NPDCCH “D NB-IoT subframe” candidate for receiving a page is found or the PTW ends. The page is plain missed if the postponed PO falls out of the PTW.

Proposal-8: POs are postponed within the PTW until an available NPDCCH “D NB-IoT subframe” candidate for receiving a page is found or the PTW ends.

Postponing of the PO would apply to pages with or without eDRX being used. Obviously, postponing will lead to unintended collision of postponed POs with existing POs in the “next available NPDCCH candidate for the PO” if it happens to be the same available NPDCCH “D NB-IoT subframe” candidate. There is no simple mitigation to the issue. One potential solution that only impacts the eNB would be need adding more paging UE identities into the combined Paging-NB message (a combination of postponed and/or original paging UE identities).

Observation-4: The postponing of POs causes unintended collision of postponed POs with existing POs in the “next available NPDCCH candidate for the PO”.

In general, the use of eDRX with IoT-NTN TDD mode is questionable. The reachability of LEO satellites is only few minutes and therefore longer sleeps (over one minute) make hardly sense as the satellite is likely already out of vicinity when the UE wakes up. The UE and the network should utilize discontinuous coverage information as part of the eDRX sleeps to avoid attempts to connect back with a satellite that is already unreachable.

Observation-5: The benefit of eDRX with IoT-NTN TDD mode is questionable, especially if discontinuous coverage information is not available.

# Conclusions

In this paper, we discussed IoT-NTN TDD mode SI scheduling impacts and H-SFN change impact with IDLE mode eDRX. Our observations and proposals are listed below:

Observation-1: A transmission of one “other SI” message always takes two 90ms TDD frames.

Observation-2: Postponing of other SI subframes to the next 90ms TDD frame is required.

Observation-3: The UE can sync itself to the 90ms TDD framing prior H-SFN change or shortly after entering the PTW.

Observation-4: The postponing of POs causes unintended collision of postponed POs with existing POs in the “next available NPDCCH candidate for the PO”.

Observation-5: The benefit of eDRX with IoT-NTN TDD mode is questionable, especially if discontinuous coverage information is not available.

Proposal-1: When operating in IoT-NTN TDD mode the scheduling mechanism of SIB1-NB remains unchanged.

Proposal-2: The SIB1-NB subframes that fall into “non-D NB-IOT subframes” are plain dropped.

Proposal-3: The scheduling procedure of other SI messages remains unchanged.

Proposal-4: Postponing of other SI subframes to the next 90ms TDD frame is supported.

Proposal-5: A network configuration shall ensure SI repetitions within a SI-window never overlap.

Proposal-6: Other SI transmission that overflows a configured SI-window shall be truncated.

Proposal-7: A UE derives the 90ms TDD frame timing by the observation of downlink signals.

Proposal-8: POs are postponed within the PTW until an available NPDCCH “D NB-IoT subframe” candidate for receiving a page is found or the PTW ends.

# References

- R2-2500390	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19