3GPP TSG-RAN WG2 #129bis	R2-2502681

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.0.2.17

Source:	Ericsson

Title:	LS response to RAN1 and RAN4 for soft satellite switch with re-sync

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the LS response received from RAN1 [1] and RAN4 [2] response about the possibility to shift back the SSB pattern upon t-Service in soft satellite switch with resync.

Figure 1. Temporary shift of target node SSBs until satellite switch is performed [3].

# 2	Discussion

## 2.1	RAN1 LS

RAN1 has discussed the question in LS R2-2410978, and RAN1 would like to ask RAN2 for further clarification on the question and assumptions below:

- Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed?

- RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

- RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

Regarding RAN1’s first assumption, the original question and RAN2’s intention was that the UE may maintain the DL synchronization according to existing specifications. However, after the initial discussions in RAN1, it is evident that the introduction of this possibility may indeed have some non-trivial RAN1 specification impact. First, there may be a potential limitation of the temporal positions SSBs can be placed in a cell following TS 38.213 (clause 4.1). Secondly, it is unclear what would be the contents of PBCH during the soft switch period. Although this can be discussed in RAN2, these are aspects that need to be decided in RAN1. Thus, we suggest that RAN2 confirms to RAN1 that there might be specification work involved with this feature.

RAN2 to confirm there might be specification work in RAN1 when SSBs are shifted back upon t-Service for the soft satellite switch with resync.

RAN1’s second assumption relates to the meaning of changing the SSB pattern. As illustrated in Figure 1, RAN2’s understanding is that the timing of the cell is aligned between source and target satellites. In addition, RAN2 already discussed and rejected (RAN2#126) the possibility that different SSB indices are used by the target satellite. Therefore, RAN2 can confirm that this assumption is correct and only the SSB transmission time in the target node is changed after t-Service.

Finally, it is clear from RAN2’s perspective that RAN1’s last assumption is correct. RAN2 may include or refer to SIB19’s ASN.1 structure in TS 38.331 and explain the contents of the field satSwitchWithResync which includes the SSB time offset, the ephemeris, and the start coverage time of the target satellite. Additionally, RAN2 can confirm that it is up to network implementation to secure that the information is not outdated when the switch happens.

Confirm the second and the third assumptions from RAN1’s LS.

## 2.2	RAN 4 LS

Under following assumptions,

- Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point;

- The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service];

RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

RAN4’s has concluded that maintaining DL synchronization is possible when there is a temporary shift of SSBs from the target satellite that is reverted upon t-Service. There are two associated conditions: the cell timing is aligned in the UTSRP and the relative offset between SSBs in provided. We consider these two assumptions are already aligned with RAN2’s design based on previous discussions.

- RAN4’s assumptions are aligned with the intended RAN2 design.

RAN4 would like to note that the temporarily SSB pattern shift (i.e., subject to ssb-TimeOffset) during [t-ServiceStart, t-Service] and shifting back after t-Service was not considered during RAN4 requirement definition and it may require additional UE processing for shifting. Also, it may have impact to UE performing initial access during [t-ServiceStart, t-Service]. It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index.

RAN4 also informs there will be associated specification work to adopt this feature and notes potential impact to UE’s performing initial random access. In RAN2, it has been previously discussed and agreed that the soft satellite transition, i.e., the time between t-ServiceStart and t-Service, will last for a short period of time and that the behavior of UE’s in RRC_IDLE/INACTIVE is not specified. Therefore, if this feature is adopted, provided the short transition period, we consider that enhancements to initial access are not a priority, and they should not be pursued as part of Rel-18 maintenance.

UE initial access during soft satellite switch is not optimized in Rel-18 maintenance.

In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset,

- Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point

- Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite

Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2:

1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5.

2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

Finally, RAN4 presents two possible interpretations of the ssb-TimeOffset parameter. Option 1 relates to the intended RAN2 design where the timing of the cell is aligned between the two satellites and the SSB offset is the relative distance in subframes between the transmissions in the source and the target satellites at the UTSRP. In contrast, Option 2 refers to a change in the timing of the cell, as stated earlier, this is not in line with the latest RAN2 understanding. Therefore, RAN2 can confirm the intended design corresponds to Option 1.

Confirm that option 1 is the correct understanding of ssb-TimeOffset parameter.

Provided RAN2 selects Option 1, there are two follow-up questions. For the first one, the parameter is defined in number of subframes from 0 to 159 and it is possible to add a restriction in RRC so that it is only provided in multiple values of 5 ms. In our understanding, if it not provided in multiple values of 5 ms, there will be RAN1 specification impact.

From RAN2 perspective, it is feasible to configure ssb-TimeOffset with values multiple of 5ms.

Regarding the last question, we think that this aspect touches upon a RAN1 procedure described in clause 4.1 of TS 38.213, which restricts where an SSB can be placed within a half-frame. Thus, the answer to this question needs to wait for RAN1 to discuss this and other aspects, including the feasibility of supporting this feature accounting for the specification impacts.

The question on the half-frame indication and SFN indication bits touches upon a RAN1 procedure which restricts where an SSB can be placed within a half-frame. Thus, this question can be answered once RAN1 has finalized their discussions on this and other aspects, including the feasibility of supporting this feature accounting for the specification impacts.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN4’s assumptions are aligned with the intended RAN2 design.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm there might be specification work in RAN1 when SSBs are shifted back upon t-Service for the soft satellite switch with resync.

Proposal 2	Confirm the second and the third assumptions from RAN1’s LS.

Proposal 3	UE initial access during soft satellite switch is not optimized in Rel-18 maintenance.

Proposal 4	Confirm that option 1 is the correct understanding of ssb-TimeOffset parameter.

Proposal 5	From RAN2 perspective, it is feasible to configure ssb-TimeOffset with values multiple of 5ms.

Proposal 6	The question on the half-frame indication and SFN indication bits touches upon a RAN1 procedure which restricts where an SSB can be placed within a half-frame. Thus, this question can be answered once RAN1 has finalized their discussions on this and other aspects, including the feasibility of supporting this feature accounting for the specification impacts.

# 4	References

[1]	R1-2501606

[2]	R4-2502701, reply LS on soft satellite switch with re-sync, Huawei, RAN4#114, February 2025.