3GPP TSG-RAN WG2 Meeting #129	R2-2502723

Athens, Greece, Feb. 17th – 21st, 2025

# Agenda item:	8.4.4

# Source:	CMCC

# Title:	Discussion on LP-WUS operation in CONNECTED mode

# Document for:	Discussion, Decision

- Introduction

In the previous meetings, RAN2 discussed LP-WUS operations in RRC_CONNECTED mode, and mainly focused on Option1-1 and Option1-2, following is the related agreements:

RAN2#126 meeting agreements [1]:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

RAN2#127 meeting agreements [2]:

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

RAN2#127bis meeting agreements [3]:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

RAN2#128 meeting agreements [4]:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

RAN2#129meeting agreements [5]:

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

In this contribution, we will discuss the potential RAN2 impacts with LP-WUS/WUR in RRC_CONNECTED state.

- Discussion

  - LP-WUS configuration

LP-WUS & short DRX cycle

In last meeting, RAN2 discussed the issue in RAN1’s LS on whether LP-WUS can be applied to the UE configured with short DRX cycle.  For Option 1-1, though the UE may get extra power saving gain if LP-WUS is applied with short DRX, it should be noticed that short DRX cycle is mainly for the services with high latency requirements. In this case, we think the possibility of no PDCCH is low, therefore Option 1-1 is not as useful as in long DRX cycle. Besides, RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE, while DCP is only applicate in long DRX cycle. Therefore, it is proposed that Option 1-1 is not applied for short DRX cycle.

As to Option 1-2, it is LP-WUS monitoring is performed at least outside legacy C-DRX Active Time, and the drx-onDurationTimer is not started with Option 1-2 LP-WUS. That to say, UE monitor LP-WUS outside legacy C-DRX Active Time, no matter short DRX cycle or long DRX cycle  is used. Therefore, there’s no need to specify Option 1-2 with short DRX.

Proposal 1: LP-WUS especially  Option 1-1 is not applied for short DRX cycle.

LP-WUS in CA/DC

Following are the RAN1 agreements related to LP-WUS configuration in CA/DC case made in last meeting [6]:

Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell - LP-WUS indication is applicable to all serving cells - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell or can be RRC configured cell other than PCell. Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell.

One open issue in CA/DC case with only one DRX group is whether SCell(s) other than PCell/PSCell can be configured for LP-WUS monitoring. Though in current cross carrier scheduling is supported and there’s no limitation to schedule PCell by SCell,  the scheduling of control information is on PCell while cross carrier scheduling is mainly used for the scheduling of traffic data. When it comes to LP-WUS, it is a kind of control information. And considering the potential deployment, it’s more possible to deploy LP-WUS in low frequencies like 2.6GHz to ensure the coverage, Therefore, we prefer that LP-WUS monitoring is only configured on the SpCell (PCell or PSCell) per cell group, and LP-WUS indication is applicable to all serving cells in the cell-group.

Proposal 2: If there’s only one DRX group in DC/CA scenario, LP-WUS monitoring is only configured on the SpCell (PCell or PSCell) per cell group, and LP-WUS is applicable to all serving cells in the cell-group.

If dual DRX group is configured for the UE, LP-WUS monitoring can be configured per DRX group, and any serving cell of a specific DRX group can be configured for LP-WUS monitoring, and LP-WUS indication is applicable to all serving cells of the DRX group.

Proposal 3: If dual DRX group is configured for the UE, LP-WUS monitoring is per DRX group configured. In this case, any serving cell of one DRX group can be configured for LP-WUS monitoring, and LP-WUS indication is applicable to all serving cells of the DRX group.

LP-WUS with Cell DTX

RAN1 discussed the coexistence of  LP-WUS and cell DTX in RAN1#120 meeting:

Agreement [6] For RRC CONNECTED mode, LP-WUS can be configured with Cell DTX. - FFS During Cell DTX inactive time, whether the UE is expected to monitor LP-WUS or not.

Based on [7], when cell DTX is configured and activated for the concerned cell, the UE may not monitor PDCCH in selected cases or does not monitor SPS occasions during cell DTX non-active duration. Since there’s no PDCCH for UE, therefore there’s no need for the UE to monitor LP-WUS during DTX non-active time.

Proposal 4: The UE is not expect to monitor LP-WUS during cell DRX non-active time.

  - Option 1-3

Both Option 1-1 and Option 1-2 are used to trigger the MR into active time for PDCCH monitoring,  which means that with only Option 1-1 or Option 1-2, once MR is activated, the UE need to always monitor all PDCCH occasions. While Option 1-3 works inside UE’s active time, with LP-WUS indication, UE can skip some PDCCH occasions monitoring for further power saving. Based on the discussion in SI phase, Option 1-3 main focuses on XR service, which the service is periodic, but there is jitter for each packet, to reduce the effects introduced by jitter, the C-DRX on-duration may be configured to cover the jitters. But it results in UE’s extra power assumption for PDCCU monitoring. With Option 1-3, UE can start PDCCH monitoring when the traffic arrives and achieve energy saving.

Figure 1: Example for Option 1-3

Based on RAN1’s definition/description of Option 1-3, LP-WUS indication is received in UE’s active time. Currently, the active time includes the following cases TS 38.321 [8]:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

It’s not clear whether Option 1-3 LP-WUS indication is applied to the timers mentioned above. In our understanding, since Option 1-3 is to solve the jitter issue of XR service, and C-DRX cycle  and on-duration configuration has taken XR service periodicity and jitter into account, Option 1-3 applied to drx-onDurationTimer is sufficient.

Proposal 5: UE only monitors the Option 1-3 LP-WUS indication within the legacy drx-onDurationTimer duration.

Besides, considering that Option 1-3 and Option 1-1/1-2 focuses on different phases of PDCCH monitoring, therefore, If Option 1-3 is supported, it can be configured with Option 1-1 or Option 1-2.

Proposal 6: If Option 1-3 is supported, it can be configured together with Option 1-1 or Option 1-2.

Support of LP-WUS use options

Following is the working assumption made in RAN1#118 meeting [9]:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

As to Option 1-3, though it can be used with Option 1-1 or Option 1-2, but the power saving gain is not as large as the other two options.  Considering RAN1’s working assumption, it can be deprioritized.

Proposal 7: RAN2 confirms RAN1’s working assumption that Option 1-3 is deprioritized for RRC_CONNECTED UE.

  - Configuration and (de)activation of LP-WUS monitoring

In RAN1#116 meeting [10], gNB RRC signalling based LP-WUS enable/disable is agreed and whether additional options on top of it is for further study.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Considering the motivation of using LP-WUS in CONNECTED mode is to reduce the PDCCH monitoring power consumption while guarantee the latency performance, it is not necessary to adapt the on/off of LP-WUS monitoring frequently based on fast L1/L2 enable/disable signalling. Besides, UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring. For example, when a UE is in lower battery status, UE can report to gNB that it prefers to activate the LP-WUS monitoring function. As to the condition based activation/deactivation mechanism, similar performance can be achieved by proper network configuration based on UE reported information, including UE assistant information and SR/BSR. Therefore, in our understanding, Option 3 and Option 4 can be achieved by network implementation.

Proposal 8: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 9: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 10: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 11: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

  - Co-existence with other power saving mechanisms

Co-existence with DCP

In last meeting, RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE, while Option 1-2 and Option 1-3 are more flexible to trigger PDCCH monitoring in drx off period or within DRX active time, therefore, there’s no need to configure DCP and LP-WUS for RRC_CONNECTED PDCCH monitoring simultaneously.

Proposal 12: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

- Conclusions

In this contribution, we discuss the LP-WUS operation in RRC_CONNECTED mode, and the following proposals are made.

Proposals:

Proposal 1: LP-WUS especially  Option 1-1 is not applied for short DRX cycle.

Proposal 2: If there’s only one DRX group in DC/CA scenario, LP-WUS monitoring is only configured on the SpCell (PCell or PSCell) per cell group, and LP-WUS is applicable to all serving cells in the cell-group.

Proposal 3: If dual DRX group is configured for the UE, LP-WUS monitoring is per DRX group configured. In this case, any serving cell of one DRX group can be configured for LP-WUS monitoring, and LP-WUS indication is applicable to all serving cells of the DRX group.

Proposal 4: The UE is not expect to monitor LP-WUS during cell DRX non-active time.

Proposal 5: UE only monitors the Option 1-3 LP-WUS indication within the legacy drx-onDurationTimer duration.

Proposal 6: If Option 1-3 is supported, it can be configured together with Option 1-1 or Option 1-2.

Proposal 7: RAN2 confirms RAN1’s working assumption that Option 1-3 is deprioritized for RRC_CONNECTED UE.

Proposal 8: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 9: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 10: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 11: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

Proposal 12: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

- References

- Report of 3GPP TSG RAN WG2 meeting #126;

- Report of 3GPP TSG RAN WG2 meeting #127;

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- Report of 3GPP TSG RAN WG2 meeting #128;

- Report of 3GPP TSG RAN WG2 meeting #129;

- Draft Report of 3GPP TSG RAN WG1 #120;

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 18);

- TS 38.321; Medium Access Control (MAC) protocol specification; (Release 18);

- Report of 3GPP TSG RAN WG1 #118

- Report of 3GPP TSG RAN WG1 #116.