3GPP TSG-RAN WG2 Meeting #129bis  											                         R2-2502770

Wuhan, China, April 7th – 11th, 2025                                                        Revision of R2-2501161

Agenda item:	8.9.4

Source:	MediaTek Inc.

Title:	Discussion on supporting PWS for NB-IoT

Document for:	Discussion and Decision

# Introduction

New objective was added to Rel-19 IoT NTN work item in RAN#105 [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

This work item has been discussed in RAN2#127bis and RAN2#128 meeting, and the following agreements have been reached.

RAN2#127bis Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

RAN2#128 Agreement

- We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

RAN2#129 Agreement

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

- Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

- Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

- We don’t introduce a new SIB for carrying geographical information for emergency areas

In this contribution, we continue discussing the rest of the issues.

# Discussion

SIB1-NB acquisition

In the last RAN2 meeting, it has been agreed that we will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed.

It is clear that the SIB1-NB contains the schedulling information of system information block. UE cannot acquire SIB10/11/12 without this schedulling information.

The schedulling periodicity of SIB1-NB is 2560ms. The least NPDSCH repetition number in the 2560ms is 4, which means the UE can get a SIB1-NB at least in 640ms. The latency for receving the PWS message is acceptable.

Therefore, we think SIB1-NB acquisition after receiving the PWS notification is needed.

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

PWS in connected mode

RAN2 had send LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19. It has been concluded in RAN#107 that RAN1 will not specify anything for this.

RAN#107 conclusion: conclusion: no extra TUs will be allocated for RAN1, we will do nothing.

Therefore, when the PWS is enabled in the RRC connected mode, the network will have to release the UE to RRC idle mode to acquire PWS. However, the UE still needs time to receive the paging message after cell selection. The PWS is sensitive to promptness. It will be beneficial to know the connection release is due to the PWS notification. The UE can directly acquire the PWS message after cell selection, skipping the paging message reception and possible re-attempting connection establishment.

Proposal 2: The network will release the UE in connected mode to acquire PWS.

Proposal 3: New release cause PWS notification is used.

# Conclusion

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

Proposal 2: The network will release the UE in connected mode to acquire PWS.

Proposal 3: New release cause PWS notification is used.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)