3GPP TSG-RAN WG2 Meeting #129bis	R2-2502778

Wuhan, China, Apr. 7th- 11th, 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	Consideration on CP issues for multi-hop SL relay

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the control plane procedure for the multi-hop SL relay was extensively discussed with various agreements on remote UE RRC connection establishment, system information acquisition and delivery, paging message acquisition and delivery, etc.  In this contribution, we will share our further view on CP related procedures.

Discussion

Baseline procedure

Issue 1: Accessing latency

As the baseline procedure, the intermediate relay UE has to establish RRC connection first before relaying the packets from its child node. The benefit of such procedure is that the design of Rel-17 single hop SL relay can be almost reused. However, such baseline procedure results in large accessing latency of the remote UE; moreover, such accessing latency will be increased with the number of hops. Thus, the baseline procedure may result in the connection establishment/re-establishment/resume failure at the remote UE. For example, during RRC connection setup procedure, T300 is started to monitor whether the RRC connection setup is failed or not. The largest value of T300 is 2000ms. In multi-hop SL relay, the remote UE may not be able to finish RRC connection setup procedure within 2000ms due to the RRC connection establishment of first/intermediate/last relay UEs.  Thus, RAN2 needs discuss how to avoid the connection establishment failure caused by the connection setup of first/intermediate/last relay UE. One possible way is to set the T300 according to different number of hops, i.e., the T300 can be increased with the number of hops.

Proposal 1-1: RAN2 is kindly asked to discuss how to avoid the connection establishment failure caused by the connection setup of first/intermediate/last relay UE (e.g., hop number based T300/T301/T319 setting).

Issue 2: reduction of accessing latency

On the other hand, RAN2 can discuss potential enhancement to reduce the latency. As mentioned above, the latency of baseline procedure is caused by the hop-by-hop connection establishment, i.e., the connection establishment/resume procedure of each intermediate relay UE can be performed only after its parent relay UE completes the connection establishment/resume. To reduce the latency, one possible way is to allow the parallel connection establishment/resume of intermediate relay UEs. Specifically, whenever an intermediate relay UE receives the first SRB0 RRC message (e.g., RRC Setup/resume request) from its child node, it can performs the forwarding of the received SRB0 RRC message and its own RRC connection establishment/resume procedure. Since the SRB0 RRC message forwarding may be carried out among RRC idle/inactive intermediate relay UE, the authorization would become a problem. That is to say, it is better to allow the authorized relay UE to perform such forwarding. To achieve this purpose, the intermediate relay UE in RRC idle state may not be suitable since the core network does not perform any authorization to it. Instead, the relay UE in RRC inactive state may be applicable since its context is still stored at the network side, and the authorization can be considered as valid.

Proposal1-2: to reduce the accessing latency of remote UE, RAN2 is kindly asked to agree that the intermediate relay UE in RRC inactive state can perform the SRB0 RRC message forwarding of its child nodes, and its own RRC resume procedure in parallel.

To illustrate the parallel procedures, Fig. 1 gives one example flow chart, with three stages for parallel procedures:

- 1st RRC message (SRB0) forwarding

In this stage, each intermediate relay UE performs its own RRC resume procedure and the forwarding of SRB0 message of its child node(s) at the same time. The forwarding can be performed via the SL-RLC0. In order to identify the source node of each forwarded SRB0 message, the intermediate relay UE can assign a local UE ID to its immediate child node; then, it can add the local ID of the source node and the bearer ID in the SRAP header when forwarding the SRB0 message to its parent node.

The local ID can be pre-configured to the intermediate relay UE when releasing it to Inactive state. Then, the intermediate relay UE can select one of pre-configured local ID and assign it to its immediate child node.

- 2nd RRC message (SRB0/1) forwarding

After receiving the 1st SRB0 message from relay/remote UE, the gNB can response with SRB0 message (for remote UE) and SRB1 messages (for intermediate relay UEs) via Uu RLC channel. The SRAP header with local UE ID and bearer ID is added to each SRB0/SRB1 message.  Then, the last relay UE can forward those message via SL-RLC0 (for SRB0 message towards remote UE) and SL-RLC1 (for SRB1 message towards intermediate relay UE). With the local ID in the SRAP header, each SRB message can be forwarded to the right destination.

- 3rd RRC message (SRB1) forwarding

The forwarding of the 3rd RRC message can be carried out via SL-RLC1.

After forwarding of 3rd RRC message, the connection setup of remote UE and the connection resume of intermediate UE(s) are completed. Since the PC5 RLC channels for SRBs/DRBs are not configured between neighboring intermediate relay UEs, each node continue using SL-RLC1 to forward the RRC message (e.g., SUI, RRC Reconfiguration) till the PC5 RLC channels are configured.

The above procedure requires the following changes to the specification:

- gNB pre-configures the local ID to the intermediate relay UE

- The intermediate relay UE assigns local ID when receiving 1st SRB0 message from its immediate child node, and forward such message by adding SRAP header with the assigned local ID and bearer ID

- The intermediate relay UE uses SL-RLC0/SL-RLC1 to forward the SRB0/SRB1 message before the PC5 RLC channel for SRBs is configured.

Proposal 1-3: To support the parallel procedures, RAN2 is kindly asked to agree the following operations:

- gNB pre-configures the local ID(s) at the intermediate relay UE, which can be assigned to its child node

- The intermediate relay UE assigns local ID when receiving 1st SRB0 message from its immediate child node, and forward such message by adding SRAP header with the assigned local ID and bearer ID

- The intermediate relay UE uses SL-RLC0/SL-RLC1 to forward the SRB0/SRB1 message before the PC5 RLC channel for SRBs is configured.

Fig. 1 Parallel connection establishment/resume procedures

After connection establishment, the gNB may release the remote UE to the IDLE/INACTIVE state, or gNB may perform the path switch to another direct or indirect path. This requires to release configurations (e.g., mapping configuration, routing configuration) along the routing path for the remote UE. In order to reduce the packet loss, the gNB should know whether the packets from the remote UE have been completely received or not before releasing the configuration.

Proposal 1-4: RAN2 is kindly asked to discuss how to handle the remote UE’s packets undertaking forwarding in the routing path before releasing the configuration for the remote UE at the intermediate relay UE.

Notification message design

The agreements in last RAN2 meeting on triggering of relay (re)selection at remote UE and intermediate UE can be summarized as the following table:

|  | Remote UE | Intermediate relay UE |
| --- | --- | --- |
| Relay selection | Reuse R17 - Direct Uu signal strength of current serving cell is below a configured threshold - Indicated by upper layer | Reuse R17 for remote UE - FFS upper layer conditions |
| Relay reselection | Reuse R17 - PC5 signal strength < threshold - Receive PC5-S release message from 1st relay UE - Detect PC5 RLF - Indicated by upper layer - Upon PC5-RRC signalling (remote UE in idle/inactive) indicating: - Cell reselection, handover, Uu RLF, Uu RRC connection establishment/resume failure between the last relay UE and NW - Release of RRC connection between an intermediate relay UE and NW - PC5 RLF or PC5-S connection release between the multi-hop U2N relay UE(s) - FFS: link quality degradation - FFS: if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE) - FFS: on the signalling contents of the PC5-RRC indication from the first relay UE - FFS: applicability to L3 |  |
| Behaviour after trigger relay (re)selection | N.A | Notify its child node - FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) - FFS if the notification is required in all cases - FFS the content in the notification - FFS the child node behaviour after receiving this notification - FFS other condition/timing that the intermediate relay UE notifies its child node |

According to the above summary, the remote UE may receive the notification message to indicate the state of different hops along the routing path:

- Between last relay UE and NW: cell reselection, handover, Uu RLF, Uu RRC connection establishment/resume failure

- Between an intermediate relay UE and NW: RRC connection release

- Between multi-hop U2N relay UE(s): PC5 RLF or PC5-S connection release

- Issue 1: transmission of the notification message

Each node may trigger its own notification message independently. After the initiation, the transmission of the notification message can be carried out by considering the following two options:

  - Option 1: transparent delivery

This option is the simplest one, i.e., each relay UE independently generates the content of the notification message, and each message is forwarded hop-by-hop transparently. This requires to include the notification message of each node as container of PC5-RRC message.

  - Option 2: non-transparent delivery

This option allows each intermediate relay UE to regenerate the content of the notification message, i.e., if the intermediate relay UE wants to add the state information of itself, it can add its own state in the new generated notification message, and forward it to its child node.

Proposal 2-1: RAN2 is kindly asked to discuss whether the notification message initiated by last/intermediate relay UE is forwarded transparently to the remote UE or not.

- Issue 2: content of the notification message

If the notification message is triggered by the last relay UE, the content of the notification message can clearly indicate that it is from the last relay UE since it is about Uu link. However, if the notification message is initiated due to the intermediate relay UE, one issue is whether the notification message should indicate the intermediate relay UE corresponding to the indication type (e.g., PC5 RLF, RRC connection release) . This issue depends on whether the remote UE has different behaviours if the corresponding indication type (e.g., PC5 RLF, RRC connection release) occurs at different intermediate relay UEs. In our understanding, remote UE does not have different behaviours, i.e., once a notification message is received by the remote UE, the remote UE can perform either relay reselection or RRC reestablishment procedure.

Proposal 2-2: RAN2 is kindly asked to discuss whether the notification message should indicate the intermediate relay UE corresponding to the indication type (e.g., PC5 RLF, RRC connection release).

In last meeting, one FFS is “FFS: on the signalling contents of the PC5-RRC indication from the first relay UE”. The first relay UE may encounter the PC5 RLF, RRC connection release, and PC5-S connection release. Those will cause the RRC reestablishment at the remote UE. Thus, if the notification message is sent to the remote UE, the indication type can indicate PC5 RLF, RRC release, PC5-S release.

Proposal 2-3: RAN2 is kindly asked to agree that the notification message from the first relay UE can indicate PC5 RLF, RRC release and PC5-S release.

- Issue 3: Notification message triggering

In legacy, the notification message is triggered by U2N relay UE upon: 1) Uu RLF, 2) handover, 3) cell reselection, 4) RRC connection failure. The same triggering condition can be applied for the last relay UE.

Proposal 2-4: RAN2 is kindly asked to agree that Rel17 triggering condition of notification message is reused by the last relay UE.

For intermediate relay UE, it may receive the notification message from its parent node. Also, it may encounter that link state change, e.g., PC5 RLF, PC5-S connection release, RRC release, by itself. For the former case, the intermediate relay UE can forward it to its child node (by either transparent way or non-transparent way). For the latter case, the intermediate relay UE can carry out its own recovery first. For example, as shown in the following figure, Relay 3 triggers the relay reselection after PC5 RLF with Relay1. Since the reselected relay, i.e., relay2, is connecting to the same gNB/cell, Relay3 itself recovers the link for the remote UE, and it needn’t trigger the notification message to the remote UE.

Proposal 2-5: RAN2 is kindly asked to agree that the notification message from the last relay UE can trigger the intermediate relay UE forwarding it.

Proposal 2-6: RAN2 is kindly asked to agree that the intermediate relay UE can trigger its own notification message when the following condition is satisfied:

- The occurrence of PC5 RLF, PC-S connection release, RRC release

- The reselected relay is served by a different cell from the one before PC5 RLF or PC5-S connection release

Fig.2 Fast recovery

Conclusion

Baseline procedure

Proposal 1-1: RAN2 is kindly asked to discuss how to avoid the connection establishment failure caused by the connection setup of first/intermediate/last relay UE (e.g., hop number based T300/T301/T319 setting).

Proposal1-2: to reduce the accessing latency of remote UE, RAN2 is kindly asked to agree that the intermediate relay UE in RRC inactive state can perform the SRB0 RRC message forwarding of its child nodes, and its own RRC resume procedure in parallel.

Proposal 1-3: To support the parallel procedures, RAN2 is kindly asked to agree the following operations:

- gNB pre-configures the local ID(s) at the intermediate relay UE, which can be assigned to its child node

- The intermediate relay UE assigns local ID when receiving 1st SRB0 message from its immediate child node, and forward such message by adding SRAP header with the assigned local ID and bearer ID

- The intermediate relay UE uses SL-RLC0/SL-RLC1 to forward the SRB0/SRB1 message before the PC5 RLC channel for SRBs is configured.

Proposal 1-4: RAN2 is kindly asked to discuss how to handle the remote UE’s packets undertaking forwarding in the routing path before releasing the configuration for the remote UE at the intermediate relay UE.

Notification message design

Proposal 2-1: RAN2 is kindly asked to discuss whether the notification message initiated by last/intermediate relay UE is forwarded transparently to the remote UE or not.

Proposal 2-2: RAN2 is kindly asked to discuss whether the notification message should indicate the intermediate relay UE corresponding to the indication type (e.g., PC5 RLF, RRC connection release).

Proposal 2-3: RAN2 is kindly asked to agree that the notification message from the first relay UE can indicate PC5 RLF, RRC release and PC5-S release.

Proposal 2-4: RAN2 is kindly asked to agree that Rel17 triggering condition of notification message is reused by the last relay UE.

Proposal 2-5: RAN2 is kindly asked to agree that the notification message from the last relay UE can trigger the intermediate relay UE forwarding it.

Proposal 2-6: RAN2 is kindly asked to agree that the intermediate relay UE can trigger its own notification message when the following condition is satisfied:

- The occurrence of PC5 RLF，PC5-S connection release, RRC release

- The reselected relay is served by a different cell from the one before PC5 RLF or PC5-S connection release

References