3GPP TSG-RAN WG2 Meeting #129bis	R2-2502857

Wuhan, China, Apr 7th – 11th, 2024

Agenda item:		7.0.2.16

Source:		Samsung

Title:			Corrections on PDCP SN Gap Reporting

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we discuss two issues that are relevant to the agreed approach of bringing alignment between the triggering and the reporting of PDCP SN gap report in RAN2#129 [1]

# 2 Discussion

## 2.1 Ambiguity with Discard Bitmap

In RAN2#129, an alignment is agreed between the triggering and the reporting SDUs of a PDCP SN gap report, which is reflected in “5.16.1 Transmit operation”. While, in “6.3.16 Discard Bitmap”, the field descriptions of bits 0 and 1 remain the same as before, such that the discrepency in the meaning of the bits between:

- In clause 5.16.1: bit 1 (0) indicates SDUs satisfying (not satisfying) the triggering conditions of the PDCP SN gap report, and

- In clause 6.3.16: bit 1 (0) indicates which SDUs are discarded (not discarded) in the transmitting PDCP entity,

could result in further unnecessary ambiguity in Discard Bitmap setting, and should be avoided.

To address this concern, it is required in clause 6.3.16, to avoid stating the actual status (discarded/not discarded) of the SDUs, and instead, specifying that Discard Bitmap indicates which SDUs are “reported as discarded” and which SDUs are “reported as not discarded” in the transmitting PDCP entity.

Proposal 1: Specify that Discard Bitmap indicates which SDUs are reported as discarded and which SDUs are reported as not discarded in the transmitting PDCP entity. (Adopt TP1)

## 2.2 Consideration for discard status at receiving PDCP entity

The below-mentioned steps are relevant to clause 5.2.2.1, clause 5.2.2.2 and clause 5.16.2:

…. - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV; …

Another issue concerns the receiving PDCP entity behaviour for considering the discard status for PDCP SDU. Particularly, the receiver may need to react to a situation where two different discard statuses for the same PDCP SDU is sequentially received in two different PDCP SN gap reports. For example, PDCP SDU X is reported as bit set to 1 in a PDCP SN gap report A and later PDCP SDU X is reported as bit set to 0 in a PDCP SN gap report B. It is required that the receiving PDCP entity considers the “actual non-discarded status” for the PDCP SDU X in both the PDCP SN gap reports and not just from the reported non-discarded status (bit set to 0) in the latest PDCP SN gap report. Hence, a clarification is needed to make specification umabiguous.

Proposal 2: Specify the consideration for discard status at the receiving PDCP entity as: (Adopt TP2)

- deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

  - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which have been considered as discarded as specified in clause 5.16.2;

- update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and has not been considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV”.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

Proposal 1: Specify that Discard Bitmap indicates which SDUs are reported as discarded and which SDUs are reported as not discarded in the transmitting PDCP entity. (Adopt TP1)

Proposal 2: Specify the consideration for discard status at the receiving PDCP entity as: (Adopt TP2)

- deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

  - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which have been considered as discarded as specified in clause 5.16.2;

- update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and has not been considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV”.

# 4 References

[1] R2-2501506: CR 0147 to TS 38.323

# 5 Annex – Text Proposals

## TP1:

### 6.3.16 Discard Bitmap Length: Variable. The length of the discard bitmap field can be 0. This field indicates which SDUs are reported as discarded and which SDUs are reported as not discarded in the transmitting PDCP entity. The bit position of the Nth bit in the Discard Bitmap is N, i.e., the bit position of the first bit in the Discard Bitmap is 1. Table 6.3.16-1 Discard Bitmap Bit; Description. 0; PDCP SDU with COUNT = (FDC + bit position) modulo 232 is reported as not discarded.. 1; PDCP SDU with COUNT = (FDC + bit position) modulo 232 is reported as discarded..

## TP2:

#### 5.2.2.1 Actions when a PDCP Data PDU is received from lower layers In this clause, following definitions are used: - HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable; - SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable; - RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header; - RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity; - RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN]. At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows: - if RCVD_SN < SN(RX_DELIV) – Window_Size: - RCVD_HFN = HFN(RX_DELIV) + 1. - else if RCVD_SN >= SN(RX_DELIV) + Window_Size: - RCVD_HFN = HFN(RX_DELIV) – 1. - else: - RCVD_HFN = HFN(RX_DELIV); - RCVD_COUNT = [RCVD_HFN, RCVD_SN]. After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU and consider it as not received; - if RCVD_COUNT < RX_DELIV; or - if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before: - discard the PDCP Data PDU; If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall: - store the resulting PDCP SDU in the reception buffer; - if RCVD_COUNT >= RX_NEXT: - update RX_NEXT to RCVD_COUNT + 1. - if outOfOrderDelivery is configured: - deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC. - if RCVD_COUNT = RX_DELIV: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which have been considered as discarded as specified in clause 5.16.2; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and has not been considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV; - if t-Reordering is running, and if RX_DELIV >= RX_REORD: - stop and reset t-Reordering. - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering. #### 5.2.2.2 Actions when a t-Reordering expires When t-Reordering expires, the receiving PDCP entity shall: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before: - all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which have been considered as discarded as specified in clause 5.16.2; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and has not been considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD; - if RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering. …… ### 5.16.2 Receive operation At reception of a PDCP SN gap report from lower layers, the receiving PDCP entity shall: - consider each PDCP SDU, if any, with the bit in the discard bitmap set to '1', or with the associated COUNT value equal to the value of FDC field as discarded; - if RX_DELIV is less than or equal to the largest COUNT value associated with the discarded PDCP SDUs: - if RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report: - update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1; - if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s): - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before: - all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which have been considered as discarded; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and has not been considered as discarded, with COUNT value > RX_DELIV; - if t-Reordering is running, and if RX_DELIV >= RX_REORD: - stop and reset t-Reordering; - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering.