3GPP TSG RAN WG2 Meeting #129-bis														R2-2502900

Wuhan, China, April 7th – 11th, 2025

Agenda item:		8.5.3

Title:						Discussion on On-demand SIB1 for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The Release 19 WI for Network Energy Saving (NES) was revised in RAN#105 [1], where the specification of the studied case 2 for on-demand SIB1 was agreed.

This contribution discusses some of the relevant aspects concerning on-demand SIB1 transmissions for UEs in idle/inactive mode, necessary for a successful feature.

# Initial Cell Search with On-Demand SIB1

During the study phase there was not enough discussion on the impact of on-demand SIB1 on Initial Cell Search. This section discusses how a suitable on-demand SIB1 design is of paramount importance to avoid performance issues on the Initial Cell Search.

The Initial Cell Search is performed when the UE has no information about the RF environment, for example, if the UE is turned on for the first time or if the UE enters a different country. The order of search is UE implementation specific and there are thousands of potential frequencies (GSCNs) to be scanned. As soon as the UE finds one suitable cell, information about other cells can be obtained from the SI. Due to the implementation specific order, there is no guarantee that the UE will find a Cell A before finding an NES cell. The more popular on-demand SIB1 gets, the more often UEs will find NES cells first. In the absence of SIB1 the UE has little information about that cell. The UE cannot even evaluate whether the cell is suitable or not, and the UE does not know if the cell is from its own operator.

- Observation 1: Often, in Initial Cell Search, UEs will find NES cells before a Cell A. In the absence of SIB1 the UE has little information about the NES cell. The UE cannot even evaluate whether the cell is suitable or not and the UE does not know if the cell is from its own operator.

In current operation with regularly broadcast SIB1 the UE can immediately evaluate if a cell is from its own PLMN or at least some allowable PLMN. Once on-demand SIB1 is broadly used, this evaluation from the UE will be hindered, unless a solution is designed to provide the UE with the respective information. For this purpose of broadcasting the PLMN, a list of PLMNs or a list of NPNs would be prohibitive and negate the energy saving benefits of omitting SIB1 on the NES cell. Instead, a compressed information can be sent for the sake of identifying (at least) the main PLMN which that cell belongs to. Namely, the cell can apply a hashing function to its PLMN ID and broadcast this PLMN hash. Then, the UE can also apply the same hashing function to its PLMN and compare the hash result with what is broadcast by the NES cell. In this way, the same Initial Cell Search performance of today can be kept: the UE can promptly evaluate whether a cell belongs to its PLMN or not. This will enable to attain similar performance on Initial Cell Search as today, while saving energy.

- Proposal 1: The NES cell broadcasts a PLMN hash, so that the UE can identify if the NES cell belongs to the desired PLMN or not.

# Reduction of UL WUS Configuration Overhead

In the case chosen for standardization (case 2), a Cell A will carry UL WUS configuration for many NES cells. Therefore, even a moderate content per NES cell (e.g. 30 octets) can quickly grow large to an extent of spending too much energy for the broadcast on Cell A.  This can make the whole feature less attractive and negate the energy saving on NES cells by adding energy expenditure on the Cell A. For this reason, it is very important to reduce the UL WUS configuration overhead.

Observation 2: It is very important to reduce UL WUS configuration overhead in on-demand SIB1.

To obtain the UL WUS configuration of NES cells the UE obtains SIB1 of Cell A. This enables the following approach for reducing the overhead drastically: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration needs to be sent and the UE uses the values from Cell A SIB1 for elements not sent.

Proposal 2: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration needs to be sent and the UE uses the values from Cell A SIB1 for elements not sent.

# Conclusions

In this contribution, we discussed some aspects of on-demand SIB1 for NES. The following observations and proposals have been made:

- Observation 1: Often, in Initial Cell Search, UEs will find NES cells before a Cell A. In the absence of SIB1 the UE has little information about the NES cell. The UE cannot even evaluate whether the cell is suitable or not and the UE does not even know if the cell is from its own operator.

Observation 2: It is very important to reduce UL WUS configuration overhead in on-demand SIB1.

- Proposal 1: The NES cell broadcasts a PLMN hash, so that the UE can identify if the NES cell belongs to the desired PLMN or not.

Proposal 2: In the UL WUS configuration all elements are made optional. Only the ones which differ from Cell A SIB1/PRACH/RAR configuration needs to be sent and the UE uses the values from Cell A SIB1 for elements not sent.

# References

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #105	 Melbourne, Australia, September 2024.