3GPP TSG-RAN2 Meeting #129bis	R2-2502970

Wuhan, China, Apr. 7th – 11th, 2025

Source: 			ZTE Corporation

Title: 	Discussion on specification impact for AI mobility

Agenda item:	8.3.5

Document for: 	Discussion and Decision

- Introduction

Based on the SID, the potential AI mobility specification enhancement needs to be studied, as follows:

• Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID • Potential specification impacts of AI/ML aided mobility [RAN2]

And in the last meeting, we have agreed to consider temporal domain prediction, frequency domain prediction and spatial domain prediction in the spec impact study procedure.

Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frequency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

In this contribution, we’d like to share our view on the potential specification impact for AI mobility with UE side model and network side model, separately.

- Discussion

## 2.1 Specification impact with UE side model

### 2.1.1 Applicability reporting

For the functionality-based LCM for UE-sided model in AI-mobility, the basic procedure in the Rel-19 AI-phy shall be reused as much as possible, which is as follows:

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. 3) The inference configuration/parameter. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

Proposal 1: The applicability reporting procedure in AI-phy is reused in the AI-mobility study.

For UE capability reporting (step 1~step 2), currently, no sufficient progress is made in AI-phy. It’s unclear the content and/or granularity of the AI-related UE capability. So, in order to avoid meaningless repetitive discussions in AI-phy and AI-mob, we suggest to postpone the discussion about the AI mobility specific UE capability until enough progress is made in AI-phy.

Proposal 2: For applicability reporting study in the AI-mobility, postpone the AI mobility specific UE capability study until sufficient progress is made in AI-phy.

For step 3~5, the network configures network-sided additional condition, inference configuration to the UE, and the UE reports the applicability based on the configurations and other information (e.g. whether the model is available, or UE side additional condition).

Network additional condition

The network additional condition refers to any aspects in the network that are used for the training of the model but are not known by the UE. So far, AI-phy study did not have agreements on the content of NW-side additional condition, but agreed to use associated Id to ensure the consistency of NW-side additional condition across training and inference. The similar solution can be reused in the AI-mobility.

Observation 1: Associated Id is used in the AI-phy to ensure the consistency of NW-side additional condition across training and inference.

Proposal 3: Use associated id to ensure the consistency of network-side additional condition across training and inference.

However, since AI-based beam management only focuses on the beams within one serving cell but AI-based mobility also needs to consider neighbor cell(s), some updates for the associated id may be needed. Specifically, in AI-phy, the associated id is per cell level, which means that the same associated id refers different network additional condition in different cells. For example, associated ID 1 in the cell 1 refers to the network additional condition (e.g. beam pattern) 1, while associated ID 1 in the cell 2 refers to the network additional condition 2. It is workable in the AI-phy, since all the beams are from one cell. However, in AI based mobility, the network additional condition of neighbor cell(s) also needs to be considered. If per cell level granularity is reused, the UE may be confused about the network additional condition referred by the associated id. Taking the figure 1(a) as an example: cell 1~3 are intra-freq cells with different network additional condition, but their associated id may be the same, which brings difficulty for the UE to identity the consistency between training and inference. Because the UE may need additional information (e.g. CellIdentity) to understanding / identity the consistency, which brings difficulty for the UE memory, since the UE needs to remember each cell’s id and the coresponding associated id.

Observation 2: Per-cell level associated Id concept is not applicable to AI-Mob, because AI-Mob involves multiple cells (not only serving cell).

Besides per cell level, in our view, other granularity of the associated id can be considered, which includes：

- Option 1: Per area level associated id;

- Option 2: Per scenario level associated id;

The per area level associated id means the referred network additional condition with the same associated id are the same in different cells that belonging to the same area. Taking the figure 1(b) as an example, the network additional condition in the cell 1 and cell 2 are the same, so their associated id are the same. This can help the UE to better identity the consistency. While, the shortage is that the coordination between different base station / vendor to allocate the associated id may be difficult. The per scenario level associated id refers the scenario / case that to predict the results with the network additional condition B based on the results with the network additional condition A. In the figure 1(c), associated id 1 can indicate that to predict the results with network additional condition 2 based on the network additional conditional 1. In this way, an associated id can indicate the network additional condition of both cell(s) for input and cell(s) for output. RAN2 can further discuss the meaning and the granularity of the associated id in the AI-based mobility.

Figure 1: Example for associated Id

Proposal 4: For the granularity for the AI mobility specific associated id, RAN2 to discuss the following options:

Option 1: per area level associated id;

Option 2: per scenario level associated id.

Inference configuration

The inference configuration is the configurations that are used to facilitate the AI/ML model inference. In the AI-phy, both full inference configuration (i.e. option a) and partial inference configuration (i.e. option b) are agreed, as follows:

- In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

For AI-based mobility, both cell results (e.g. RRM sub use case 2) and beam results (e.g. RRM sub use case 1) can be used as model input and output. So, the inference configuration includes cell(s)/beam(s) info whose results are used as model input/output. While, in the existing L3 measurement, the measurement is configured per measId level, the cell info may not explicitly include in the configurations. So, it worth to discuss whether and how to explicitly indicate the cell info/beam info. Similar to AI-phy, the associated id may be also included in the inference configuration, to help the UE to determine whether the functionality is applicable. After prediction, the UE needs to report the inference outcome to the network. In our view, the reporting configuration mainly includes triggering configuration (e.g. periodical or event triggered) and reporting content configuration (e.g. RSRP/RSRQ/SINR). Besides, in the AI-Phy BM-Case 2, it is agreed that the inference configuration include time gap between two consecutive future time instances and the number of future time instances. For temporal domain prediction in the AI-mobility, the same parameters can be reused.

Proposal 5: For applicability reporting study in AI-mobility, the inference configuration may include:

  - The cell(s)/beam(s) info whose results are used in the AI model input;

  - The cell(s)/beam(s) info whose results are used in the AI model output;

    - how to indicate the cell/beam info.

  - Report configuration:

    - The triggering configuration (e.g. measurement event configuration and/or periodical reporting configuration)

    - The configuration related to the content of the report (e.g. reportQuantity, maxReportCells, includeBeamMeasurements…);

  - Associated id;

  - For temporal domain prediction:

    - The time instance related information for measurement;

    - The time instance related information for prediction;

      - The time gap between two consecutive future time instances;

      - The number of future time instances;

For the signaling design, the inference configuration in AI-phy takes current CSI framework as starting point. For full inference configuration, CSI-ReportConfig are used as inference configuration, and the beam set(s) for input and output are indicated via CSI-ResourceConfigId. For partial inference configuration, the IE in the CSI-ReportConfig are taken as starting point. Similarly, for inference configuration in AI-mobility, the current L3 measurement framework can be taken as baseline, and details can be further studied.

Proposal 6: For signaling design of inference configuration in AI-mobility, RAN2 to take current L3 measurement framework as starting point.

Applicability reporting triggering

In the AI-phy, the applicability reporting triggering condition are triggered upon:

- being configured to provide applicable functionality (i.e. initial reporting)

- applicability change:

  - A configured functionality becomes from non-applicable to applicable;

  - A configured functionality becomes from applicable to non-applicable;

The same triggering condition can be reused in the AI-mobility.

Proposal 7: The applicability reporting triggering condition in the AI-phy can be reused in the AI-mobility.

For the content of the applicable functionality reporting

In the AI-phy, the UE reports applicability based on the full inference configuration or partial inference configuration. Specifically, for full inference configuration, the UE can report one or more CSI-ReportConfig configured by the network for AI, e.g. via CSI-ReportConfigId. Similarly, for AI mobility, the content of the applicable functionality reporting also depends on the inference configuration, so we suggest to postpone the content of the applicable functionality reporting study in the AI-mob until enough progress is made in the AI mobility specific inference configuration.

Proposal 8: Postpone the study of applicable functionality reporting content in the AI-mob until enough progress is made in the AI mobility specific inference configuration.

### 2.1.2 Measurement reporting:

- Measurement reporting triggering

The current event-triggered measurement and periodic measurement reporting are supported in the L3 measurement framework. Since measurement event prediction is supported, it’s natural to support measurement event triggered reporting. The periodical reporting can help the network to monitor the channel quality fluctuation. And in the case that only RRM measurement prediction is supported (measurement event prediction is not predicted), the UE can know the future measurement results via periodical reporting. So, for inference outcome reporting, both event-triggered measurement reporting and periodic measurement reporting shall be supported.

Proposal 9: For inference outcome reporting in AI-mob, support event-triggered measurement reporting and periodical measurement reporting.

- The content of the event-triggered measurement report

In the legacy event-triggered measurement report, the measurement report is sent when the event is triggered and the measurement report includes the latest measurement results of serving cell(s) and optional neighbor cell(s). For AI-assisted measurement, measurement event prediction with temporal domain prediction may be used. In this case, the time instance that measurement report is triggered (@t0) may be earlier than the time instance that measurement event is triggered (@t2), as shown in the figure 2. Since the measurement reporting time and event triggered time may be different in the AI-mobility, the UE needs to indicate the event-triggered time to the network. Besides, with AI-assisted measurement, the measurement results for multiple time instances are available, it worth to study that the measurement results of which time instances are included in the measurement report. In our understanding, the following can be considered:

- The measurement results at current time instance (@t0).

This can help the network know the actual channel quality. While, at this time instance, the entry condition of the event may not be satisfied (as shown in the figure 2(b)), the source cell can not identity which cell triggers the measurement event based on the measurement results at t0, and can not initiate the handover preparation procedure towards target cell.

- The measurement results at event-triggered time instance (@t2).

This follows legacy rule. Based on the measurement results, the network can identity which cell triggers the measurement event. But in the case that the handover command are sent before event-triggered time, the target cell may be not able to configure suitable RRM configurations based on the measurement results at event-triggered time instance, e.g. for CFRA resource configuration, the best SSB at the t2 and t1 may be different.

Figure 2: measurement event triggering and reporting for (a) legacy event; (2) measurement event with temporal prediction

For event-triggered measurement report with spatial domain prediction or frequency domain prediction, there is no future measurement results available, the UE reports the latest measurement results (from prediction or actual measurement) to the network, as legacy.

Proposal 10: For the content of event-triggered measurement report in AI-assisted measurement, RAN2 to consider the followings:

- If temporal domain prediction is performed:

  - The time occasion that measurement event is predicted to occur

  - The measurement results at the time instance that the event is predicted;

  - The measurement results at predicted event-triggered time instance.

- If frequency or spatial domain prediction is performed:

  - The latest measurement results (includes predicted results and/or actual measured results).

- The content of the periodical measurement report

In the legacy periodical measurement report, the measurement report is sent upon the expiry of the report interval. For temporal domain prediction case A, the current actual measurement results and future predicted results are available, it is worth to discuss the measurement results of which time instance need to be reported to the network. Since the motivation of the temporal domain case A is to improve handover performance, in our understanding, it is benefitable for the network to know the measurement results at future time instance.

For temporal domain prediction case B, depending on the time instance that measurement report is initiated, the latest historical measurement result can be from actual measurement or prediction; and the future measurement results can be available or not. For example, if the time instance that the measurement report is initiated is within observation window of the temporal domain case B, the future measurement result is not available, the UE can only report latest actual measurement results.

For spatial domain prediction and frequency domain prediction, the UE can report the latest measurement results (from prediction or actual measurement) to the network, as legacy.

Proposal 11: For the content of the periodical measurement report in the AI-assisted measurement, consider the followings:

- If temporal domain prediction is performed:

  - The latest results from actual measurement or prediction;

  - One or more available future measurement results;

- If spatial domain prediction / frequency domain prediction is performed:

- The latest results from actual measurement or prediction;

    - Performance monitoring

In the AI-phy, the network can initiate performance monitoring and make management decisions based on the performance monitoring results. While, up to now, no sufficient progress is made in the AI-phy study, specifically for UE-assisted performance monitoring. Our suggestion is to postpone the performance monitoring discussion in the AI-mob until enough progress is made in the AI-phy.

Proposal 12: Postpone the performance monitoring study in the AI-mob until enough progress is made in the AI-phy.

    - Data collection for training data

In the AI-phy, the training data collection configuration procedure is shown as follows:

Step 1 (optional): The network can configure whether UE is allowed to initiate request for data collection.

Step 2 (optional): For data collection configuration UE-side model training, the UE can send a request for data collection. The content of the request is unclear so far.

Step 3: The network can provide the data collection configuration (at any point in time), with or without UE request. While, the content of the data collection configuration is still unclear so far.

Step 4/6: The network can decide when to start/stop/release the data collection

The basic procedure of training data collection configuration can be reused in the AI-mobility, while AI-phy is still discussing the content or the details of each step. So, for AI-mobility, we can wait for more progress in the AI-phy.

Proposal 13: The basis procedure of training data collection configuration is reused in the AI-mobility, the details wait for more progress in AI-phy.

Although extensive discussions are made, there is no conclusion about how to transfer the training data. So, in the AI mobility study, we suggest not to discuss collected training data transfer to avoid repetitive discussion.

Proposal 14: Not discuss training data transfer in the AI mobility study.

## 2.2 Specification impact with network side model

In the AI-phy, for the network side model, RAN2 focus on the data collection procedure. Similarly, in AI-mobility, for the specification impact with the network side model, the study can focus on the data collection first. The network needs to collect the data for training model and/or for performing inference from the UE. For the sake of simplicity, we suggest to define a unified data collection framework, which can be used for the model training and inference data collection for the NW-sided model.

Proposal 15: Aim to define unified data collection framework for model training and inference for NW-sided model.

In the AI-phy, RAN2 confirms that the UE will not be informed any gNB/LMF-sided model/functionality management decision, and will not be involved in any management decision making expect being configured to provide the required measurement/data. Similarly, in the AI-mobility, for the network side model, the network is responsible for the functionality management. For the UE, upon receiving measurement configuration, the UE performs measurement and reports the measurement results to the network based on the received network configuration. In our view, the usage of the reported measurement results (e.g. for training model or performing inference) is transparent to the UE.

Proposal 16: Whether the measurement reporting is for NW-sided model training or inference can be transparent to UE.

In the existing measurement framework, the network can inform the UE which type of the measurement results need to be reported to the network and when to trigger the measurement report via RRM measurement configuration. Specifically, the network can indicate the reporting quantity (RSRP, RSRQ, or SINR) via MeasReportQuantity; and indicate the measurement type (e.g. periodical reporting or measurement event triggered reporting) via reportType.

Observation 3: In existing RRM measurement configuration, the network can already indicate the reporting quantity (RSRP/RSRQ/SINR) and measurement type (e.g. periodical).

For the measurement configuration for AI-assist measurement, the existing measurement configuration can be taken as baseline. And since three RRM sub use cases are considered for cell-level RRM measurement prediction, the network needs to collect different types of measurement results, e.g. L1 filtered beam level measurement results (e.g. for sub use case 1) or L3 filtered/unfiltered cell level measurement result(s) (e.g. for sub use case 1) for different types of AI model. So, the network can configure the UE which type of the measurement results need to be reported. Besides, the network may indicate the UE the cell(s)/beam(s) info whose measurement results needs to report. In this way, the UE can only report the data that required by the network and reduce the signaling overhead.

For temporal domain prediction, in the current simulation, the measurement interval and/or prediction interval are fixed. However, in the current specification, when to perform measurement is up to UE implementation, the UE measurement only needs to satisfy the measurement periodicity requirements provided in the TS 38.133. While, for the model training and inference, in order to obtain better prediction accuracy, it’s expected to have measurement results with similar measurement interval. So, in our view, the network can configure an expected L1/L3 measurement interval to the UE, the UE is expected to perform measurement with configured interval.

For temporal domain prediction case B, the UE needs to report measurement results at multiple time instance to the network (in order to obtain the measurement results within observation window). If using existing measurement reporting mechanism, the measurement report only includes the measurement results at only one time instance. Thus, multiple measurement reports are needed. In order to reduce signaling overhead, for AI-mobility, the network can indicate the number of the continuous measurement to the UE, and then the UE reports the measurement results of the continuous measurement to the network via one measurement report. So, the logging of the measurement results shall be supported for data collection for NW-sided model.

Proposal 17: On top of existing RRM measurement and reporting framework, the network can configure the following information.

  - The type of measurement results (e.g. L1 filtered beam results, L3 filtered cell results)

  - The cell(s)/beams whose measurement results needs to report;

  - The expected L1/L3 measurement interval;

  - The number of continuous measurements;

Proposal 18: Measurement logging and reporting is supported for data collection for NW-sided model.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

For UE side model:

Applicability reporting

Proposal 1: The applicability reporting procedure in AI-phy is reused in the AI-mobility study.

Proposal 2: For applicability reporting study in the AI-mobility, postpone the AI mobility specific UE capability study until sufficient progress is made in AI-phy.

Proposal 3: Use associated id to ensure the consistency of network-side additional condition across training and inference.

Proposal 4: For the granularity for the AI mobility specific associated id, RAN2 to discuss the following options:

Option 1: per area level associated id;

Option 2: per scenario level associated id.

Proposal 5: For applicability reporting study in AI-mobility, the inference configuration may include:

  - The cell(s)/beam(s) info whose results are used in the AI model input;

  - The cell(s)/beam(s) info whose results are used in the AI model output;

    - how to indicate the cell/beam info.

  - Report configuration:

    - The triggering configuration (e.g. measurement event configuration and/or periodical reporting configuration)

    - The configuration related to the content of the report (e.g. reportQuantity, maxReportCells, includeBeamMeasurements…);

  - Associated id;

  - For temporal domain prediction:

    - The time instance related information for measurement;

    - The time instance related information for prediction;

      - The time gap between two consecutive future time instances;

      - The number of future time instances;

Proposal 6: For signaling design of inference configuration in AI-mobility, RAN2 to take current L3 measurement framework as starting point.

Proposal 7: The applicability reporting triggering condition in the AI-phy can be reused in the AI-mobility.

Proposal 8: Postpone the study of applicable functionality reporting content in the AI-mob until enough progress is made in the AI mobility specific inference configuration.

Measurement reporting

Proposal 9: For inference outcome reporting in AI-mob, support event-triggered measurement reporting and periodical measurement reporting.

Proposal 10: For the content of event-triggered measurement report in AI-assisted measurement, RAN2 to consider the followings:

- If temporal domain prediction is performed:

  - The time occasion that measurement event is predicted to occur

  - The measurement results at the time instance that the event is predicted;

  - The measurement results at predicted event-triggered time instance.

- If frequency or spatial domain prediction is performed:

  - The latest measurement results (includes predicted results and/or actual measured results).

Proposal 11: For the content of the periodical measurement report in the AI-assisted measurement, consider the followings:

- If temporal domain prediction is performed:

  - The latest results from actual measurement or prediction;

  - One or more available future measurement results;

- If spatial domain prediction / frequency domain prediction is performed:

- The latest results from actual measurement or prediction;

Performance monitoring

Proposal 12: Postpone the performance monitoring study in the AI-mob until enough progress is made in the AI-phy.

Data collection for training data

Proposal 13: The basis procedure of training data collection configuration is reused in the AI-mobility, the details wait for more progress in AI-phy.

Proposal 14: Not discuss training data transfer in the AI mobility study.

For NW side model:

Observation 3: In existing RRM measurement configuration, the network can already indicate the reporting quantity (RSRP/RSRQ/SINR) and measurement type (e.g. periodical).

Proposal 15: Aim to define unified data collection framework for model training and inference for NW-sided model.

Proposal 16: Whether the measurement reporting is for NW-sided model training or inference can be transparent to UE.

Proposal 17: On top of existing RRM measurement and reporting framework, the network can configure the following information:

  - The type of measurement results (e.g. L1 filtered beam results, L3 filtered cell results)

  - The cell(s)/beams whose measurement results needs to report;

  - The expected L1/L3 measurement interval;

  - The number of continuous measurements;

Proposal 18: Measurement logging and reporting is supported for data collection for NW-sided model.

- Reference

- Report of RAN2 129 meeting.