3GPP TSG-RAN WG2 Meeting #129bis	R2-2502985

Wuhan, China, Apr. 7th – 11th, 2025

Source: 	Session chair (Huawei)

Title:	Report from session on R19 XR

## List of AT-meeting offline discussions

[AT129bis][500][XR] Organizational – Session on R19 XR (Session chair)

Scope:

      - Share plans and list of ongoing email discussions for the session on R19 XR

      - Share meeting notes and agreements for review and endorsement

## List of POST-meeting offline discussions

[POST129bis][501][XR] Stage-2 running CR (Nokia)

Scope: Update and review the CR

Intended outcome: Running CR for endorsement in the next meeting

Deadline:  Long

[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][503][XR] RRC running CR and open issues (Huawei)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][504][XR] PDPC running CR and open issues (LGE)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][505][XR] RLC running CR and open issues (vivo)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

[POST129bis][506][XR] UE capabilities CRs (Xiaomi)

Scope: Update and review the CRs

Intended outcome: Running CRs for endorsement in the next meeting

Deadline:  Long

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-18 and earlier maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

- NOTE: the tdoc limit applies to all CRs (i.e. WI spec rapporteurs are NO longer expected to submit individual contributions).  They can submit a company CR where they also include miscellaneous corrections that have been sent to them.

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	RAN1/RAN4 NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and agreed as individual CRs

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning.  The work plan is not expected to be updated/submitted every meeting, unless needed.   It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Postponed CRs still count towards tdoc limit unless 3 or more companies are co-sourcing it.

Tdoc request/submission for RAN2#129bis deadlines:

- Tdoc Submission deadline: Mar. 28th, 1000 UTC

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243318)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.7.1	Organizational

LS, Rapporteur input, workplan, running CRs etc.Including initial input/CRs from R19 XR UE capabilities rapporteur.

WI rapporteur input

R2-2502275	Rapporteur Inputs	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Incoming LS(es)

R2-2501754	LS reply on multi-modality awareness (S2-2502465; contact: CMCC)	SA2	LS in	Rel-19	XRM_Ph2, NR_XR_Ph3-Core	To:RAN2, RAN3, SA	Cc:SA4

Noted

R2-2501757	Reply to LS on Application-Layer FEC Awareness at RAN (S4-250252; contact: Qualcomm)	SA4	LS in	Rel-19	NR_XR_Ph3-Core, FS_5G_RTP_Ph2, FS_XRM_Ph2	To:RAN2, SA2	Cc:RAN3

Noted

Running CRs

R2-2501761	Introduction to R19 XR enhancements	Qualcomm Incorporated	draftCR	Rel-19	38.321	18.5.0	NR_XR_Ph3-Core

R2-2501950	PDCP running CR for R19 XR	LG Electronics Inc. (Rapporteur)	draftCR	Rel-19	38.323	18.5.0	NR_XR_Ph3-Core

R2-2502089	Draft runnnig RRC CR for R19 XR	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_XR_Ph3-Core

R2-2502162	RLC Running CR for XR	vivo	draftCR	Rel-19	38.322	18.2.0	B	NR_XR_Ph3-Core

R2-2502276	Draft Stage 2 CR for XR	Nokia (Rapporteur)	draftCR	Rel-19	38.300	18.5.0	B	NR_XR_Ph3-Core

All the CRs above are endorsed as baseline for further changes

Discussion on running CRs

R2-2501951	Discussion of [POST129][511][XR] PDCP running CR	LG Electronics Inc. (Rapporteur)	report	Rel-19	NR_XR_Ph3-Core

Noted

Proposal: Discuss the following issues in the RAN2#129bis meeting.

1.	Whether the RAN2 agreement “non-delay critical data ahead of delay critical data” applies to non-delay-reporting data ahead of delay-reporting data.

2.	Whether to use COUNT to describe the non-delay-critical data ahead of delay-critical data.

3.	Whether to use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set.

4.	Whether to keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the PDCP specification.

5.	Whether to specify delay-reporting indication in the PDCP specification or leave it up to UE implementation.

6.	Whether to specify update of delay-reporting indication in the PDCP specification or leave it up to UE implementation.

- LGE clarifies issue 1 and 2 can be discussed based on Tdocs. We can discuss point 3-6 now.

On bullet 3:

- Xiaomi thinks the definition is not needed.

On bullet 4:

- OPPO thinks this should be captured in RRC. Sharp, Samsung agrees, as otherwise some impact on UE cannot be avoided.

- Huawei thinks does not think we need such restriction in RRC as long as it is used properly in PDCP.

- Vivo supports capturing this in PDCP, no strong view on RRC, but see no strong motivation.

- Nokia is also OK to keep this in PDCP, but we also need some more clarifications on how data is calculated.

- Huawei wonders if UE will reject configuration if they are not ordered properly.

- Nokia thinks that if we capture in RRC, then in PDCP we can just capture that UE calculates from lowest threshold.

On bullet 5:

- CMCC agrees to have this specified.

Ob bullet 6:

- Apple thinks this should be ensured by the UE implementation, no need to specify.

- LGE thinks that we should at least specify that the UE makes sure that the data that is indicated is up to date.

- ZTE thinks that it would be good to specify this at least for DL, e.g. for CU-DU split. Lenovo indicates this is for DSR so only UL.

- Huawei wonders why we have an initial indication but no update? Lenovo thinks it is obvious it needs to be up to date.

- Apple thinks that is already specified in MAC that the data refers to PUSCH start time. LGE thinks that with multiple thresholds some description is needed in PDCP.

Agreements related to PDCP CR - In PDCP specifications, use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set. - Keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the RRC specification. - Specify delay-reporting indication in the PDCP specification. - Do not specify explicitly an update of delay-reporting indication in the PDCP. FFS (when discussing the CR) whether some explanation/note can be added to clarify that the UE should ensure the indicated data volume is up to date.

R2-2502091	Summary for [POST129][510][XR] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Potentially easy to agree:

Proposal3: Introduce RRC configuration to enable/disable the fallback to default priority in the 2nd stage of LCP

Proposal4: Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

Proposal5: The maximum number of entries in the dsr-ReportingThresList is 4.

Proposal6: For the available data rate query MAC CE, confirm on the following:

	Introduce a prohibit timer for the UL transmission of the data rate query MAC CE

	Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration

Proposal7: The granularity of the bit rate query prohibit timer (QoS flow level or DRB level) should be the same as that of bit rate recommendation. FFS based on further progress on bit rate recommendation.

Need further discussion:

Proposal1: Discuss whether the entries of drs-ReportingThresholdList need to be configured in ascending order.

Proposal2: Determine the name of new R19 DSR from two options have been proposed currently: (a) multiple entry DSR; (b) enhanced DSR.

- QCM asks whether there will be different UE behaviour depending on the configuration? Nokia thinks we have a rule that no capability should be defined without network configuration.

- Vivo thinks we should revisit P4 after we make the decision for the priority fallback. Xiaomi thinks this is different as for this one we can already agree.

Agreements related to RRC CR - Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR - The maximum number of entries in the dsr-ReportingThresList is 4. - For the available data rate query MAC CE, confirm on the following: - Introduce a prohibit timer for the UL transmission of the data rate query MAC CE - Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration - FFS the details of prohibit timer, e.g. granularity

UE capabilities

R2-2501880	UE capabilities for XR	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: RAN2 to discuss whether to define separate UE capabilities for different XR features (as in Rel-18 XR), or to define a generic Rel-19 XR capability which can include multiple features.

Proposal 2: RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19.

Proposal 3: An optional UE capability (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. FFS whether a UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 4: RAN2 to discuss the capability handling regarding fallback to default priority in the 2nd round of LCP, e.g. whether it is a capability with or without signalling.

Proposal 5: An optional UE capability (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. A UE supporting this feature shall also indicate support of delayStatusReport-r18.

Proposal 6: An optional UE capability (e.g. delayStatusReportNonCriticalData-r19) is introduced to indicate the support of including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. A UE supporting this feature shall also indicate support of enhanced delay status report.

Proposal 7: An optional UE capability (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites.

Proposal 8: An optional UE capability (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites.

Proposal 9: Separate UE capabilities are introduced for UE Tx and Rx operations for unnecessary retransmission avoidance.

Proposal 10: An optional UE capability (e.g. supportOfStopRetxObsoleteSDU-r19) is introduced to indicate the support of stopping retransmission of obsolete SDUs. The capability does not have pre-requisites.

Proposal 11: An optional UE capability (e.g. rlc-Rx-DiscardTimer-r19) is introduced to indicate the support of RLC receiver timer to abandon obsolete SDUs. The capability does not have pre-requisites.

Proposal 12: An optional UE capability (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites.

Proposal 13: If working assumption is confirmed, an optional UE capability (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE.

Proposal 14: Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

DISCUSSION on P1:

- OPPO, QCM, Ericsson thinks we should have separate capabilities per feature, not combined one.

- Nokia asks if we can now have a R19 XR UE which does not support any XR feature. Nokia thinks there should be a minimum set of UE capabilities that XR UE supports.

- Ericsson indicates we have the same situation in Rel-18 and we can follow this.

- Ofinno thinks we only need to define relations between capabilities at a later stage (after we define capabilities).

- Nokia would like to move away from how we define capabilities so far, if not now then we will have to re-discuss in 6G. CMCC share the concern.

- ZTE understands the concern from Nokia, but thinks that whether to have a minimum set should be discussed at plenary.

- LGE indicates that we specify rather independent features and it is hard to define a minimum set.

- Huawei wonders what “XR UE” means, is it another UE category?

- Sharp also believes we should have separate capabilities.

- Ericsson agrees with LGE and Huawei. Ericsson does not see any specific requirements for XR UEs.

- vivo thinks it is too late to discuss this, we should have done this from Rel-18.

- QCM thinks we only define UE type if we have mandatory behaviours for the UE which is not the case for XR.

- Nokia thinks that we risk that the feature will never be deployed if we do not specify anything. The problem will not go away if we do not address it in 3GPP, then other bodies will have to do it.

DISCUSSION on P3:

- QCM does not think R19 LCP needs to be dependent on R18 DSR. Lenovo, Samsung, OPPO, Ericsson agrees.

On P5:

- LGE, QCM thinks we do not need to couple R18 and R19 DSR. Vivo thinks reasonable UE will implement R18 if implements R19, but does not want to couple in specs.

- Huawei does not think it is possible to support R19 without R18

On P12:

- QCM thinks we should have separate capabilities for the UL and DL MAC CEs.

On P14:

- QCM thinks that DSR capability in DC configurations can be band combination dependent.

Agreements on UE capabilities - RAN2 assumes that we define separate UE capabilities for different XR features (as in Rel-18 XR). - RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19. - An optional UE capability with signalling (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. No dependency on support of delayStatusReport-r18. - An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18. - An optional UE capability with signalling (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites. - An optional UE capability with signalling (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites. - An optional UE capability with signalling (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites. FFS whether there is a separate UE capability for UL rate query. - For the above Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

### 8.7.2	Multi-modality support

No contributions are expected for this AI for RAN2#129bis

### 8.7.3	RRM measurement gaps/restrictions related enhancements

No contributions are expected for this AI for RAN2#129bis

R2-2501803	Discussion on Measurement Gap enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.4	Scheduling enhancements

#### 8.7.4.1	LCP enhancements

Further details of handling of the additional priority for LCH, e.g. configuration details, impact on Bj, impact on SR priority determination during intra-UE prioritization.

Bj enhancement

R2-2501838	Discussion on LCP enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: Do not support an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

R2-2502934	LCP enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1	RAN2 to agree on the FFS that all LCH priority adjusted data can be transmitted from a LCH, despite Bj being negative, before moving to the next LCH in the LCP procedure.

Proposal 2	The behaviour in Proposal 1 should be controlled by network configuration, as the wanted behaviour depends on the traffic served and LCP parameters configured.

DISCUSSION:

- QCM thinks Bj enhancement is not needed and priority adjustment is sufficient.

- LGE supports Ericsson proposal. It may happen that due to big burst of data Bj may become negative and if this data is urgent, it should be sent. Fairness over long-term period will be met anyway.

- Huawei agrees with LGE and Ericsson. In current LCP we can already include data if Bj is negative in 2nd phase.

- OPPO does not think this is fundamental, but is OK with considering Bj negative case.

- Xiaomi agrees with QCM, they think this is equivalent to PBR enhancement. We already apply additional priority in 2nd round, so there should be no issue.

- Apple thinks we should either do nothing or have proper solution. Negative Bj will not solve the issue. E.g. we if an LCH has small positive Bj, then they will be in worse situation.

- Nokia agrees there is no need to do anything. Network can conifgure PBR properly. Sharp agrees.

- Sharp thinks there is an issue with fairness.

- vivo agrees with LGE on fairness, so there is no technical concern with Bj enhancement.

- IDT agrees with LGE and Ericsson.

- Fujitsu also agrees with Ericsson proposal. This is simpler than PBR adjustment and the specificaitons impact is very small.

- Samsung thinks this will happen very often, so the optimization is not needed.

- Lenovo also thinks this is an optimization, can follow majority.

Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

Intra-UE prioritization – SR priority

R2-2502163	Remaining issues on LCP enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2	Additional LCH priority does not impact SR priority determination.

R2-2502202	LCP enhancements for XR	ZTE Corporation, Sanechips	discussion

Proposal 3: For “LCH priority-adjusted data”, the additional LCP priority is also used for SR priority determination.

Noted

DISCUSSION:

- Samsung agrees with ZTE, Samsung clarifies that we just using additional priority, but it is not identifying a specific channel.

- OPPO supports vivo’s proposal as additional priority is determined during LCP, so it is not known for SR. Think there are many additional topics to discuss, e.g. whether we use the same threshold.

- LGE indicates we already agreed that we do not consider additional priority for triggering SR, so we do not have to use it in this case as well.

- CATT supports ZTE proposal.

- Xiaomi agrees that there are additional spec impacts and prefers not to consider add priority for SR.

- IDT also supports vivo proposal, it is OK to just use it for LCP and no need to enhance URLLC.

- Lenovo also supports vivo view, the gain is unclear.

- QCM thinks triggering and priority determination for intra-UE prioritization is different thing. Thinks that it is useful to adjust the priority of SR so that UE has bigger chance to get a grant from the NW.

- Huawei supports ZTE and thinks the spec impact is small.

- Ericsson, Nokia agrees with Lenovo, it is hard to understand the gain.

- Sharp agrees with ZTE. We agreed to use it for grant, so it is straightforward to reuse it for SR priority determination.

- CMCC is afraid that we might need to study it for BSR as a next step.

- Apple thinks that for SR vs grant conflict, the UE can include urgent data in this grant. For SR vs SR conflict, this does not seem to be a big problem.

- Huawei indicates that such grant may not be used because we have LCH restrictions and in this case urgent data cannot use it. In those cases SR should be prioritized. ZTE and Samsung agree.

- Sharp thinks that intra-UE prioritization happens per MAC entity, what Huawei mentioned is not a rare.

- LGE thinks there is additional aspect of how to determine the time point of SR priority determination.

We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

Intra-UE prioritization – others

R2-2502579	remaining details on Intra-UE prioritization / LCP enhancements	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: RAN2 to agree that it should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant.

Proposal 2: RAN2 to agree that UE should use for the retransmission case the additional priority of a LCH for determining the priority of a corresponding UL grant only if the additional priority was used during the complete LCP, i.e. first and second round of LCP.

Proposal 3:  RAN2 to agree that UE should use for the initial transmission case the priority that will be used for a LCH during first round of LCP for determining the priority of the corresponding UL grant.

Proposal 4: RAN2 to agree that UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

DISCUSSION:

- Lenovo clarifies P4 is for shared spectrum, but it can be easily supported.

- Fujitsu supports P4, we should not exclude shared spectrum usage.

- OPPO is not sure whether we need to support shared spectrum for XR, but they are OK, if companies want it.

It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP.

For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

2nd round priority fallback capability

R2-2502265	LCP Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5: RAN2 to choose from one of the following options:

-	Option #1: No capability

-	Option #1a: the additional LCH priority is always applied to both the 1st and 2nd round of the LCP procedure,

-	Option #1b: all UEs support fallback to default priority in the 2nd round of the LCP procedure

-	Option #2: capability without signaling

-	It is left for UE implementation whether to fallback to default priority in the 2nd round of LCP procedure

-	Option #3: capability with signaling

-	The NW is enabled to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure

- QCM would like to have capability with signalling, but no configuration is needed. QCM think we can have a configuration which is always enabled.

- Ofinno thinks that it can be enabled by default but the network should eb able to disable it.

- Vivo still wonders why we need NW configuration if anyway it is always enabled. Xiaomi agrees.

- LGE thinks we should have NW configurability and no need to restrict NW behavior.

- CMCC thinks it should be up to NW to configure or not. Lenovo agrees. Lenovo thinks we need the NW to be able to enable/disable the feature. ZTE agrees the NW should contril this, the UE should not reject the configuration if the NW does not indicate to use this.

After offline discussion on the following: “FFS capability with signaling and the NW is allowed to configure whether the UE shall fallback to default priority in the 2nd round of the LCP procedure (offline- Nokia)”:

- Nokia reports that it seems all companies are OK with supporting option 1b from their paper, i.e. all UEs support fallback to default priority in the 2nd round of the LCP procedure.

All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

R2-2501762	Discussion on LCP enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501815	Discussion on LCH priority adjustment for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501870	Consideration on LCP Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501946	Discussion on LCP Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502008	Discussion on LCP enhancements of XR traffic	Xiaomi Communications	discussion

R2-2502034	Discussions on enhancements for LCH priority-adjusted data	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502180	Discussions on Delay-based LCP Enhancements	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502295	Considerations on LCP enhancements for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502300	Discussion on additional priority based LCP enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502398	Discussion on enhanced LCP for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2502562	Considerations on LCP Enhancements	China Telecom	discussion

R2-2502583	LCP enhancement for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502718	Further consideration on LCP enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502848	Remaining issues on LCP enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502948	Finalising LCP design for XR Ph3	Samsung R&D Institute UK	discussion

#### 8.7.4.2	DSR enhancements

Further details of enhanced DSR configuration/procedure, data volume calculation etc..

Empty DSR issue

R2-2502007	Discussion on DSR enhancements of XR traffic	Xiaomi Communications	discussion

Noted

Proposal 3 To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold.

R2-2502932	DSR enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1	A trigger threshold can be configured with larger value than the range of the reporting thresholds.

Proposal 2	When a DSR is triggered but no data resides inside the reporting threshold range the multi threshold DSR can indicate the data volume above the reporting threshold range.

DISCUSSION:

- Nokia thinks sensible network implementation will ensure that there is data to report when the DSR is triggered.

- Samsung agrees and thinks this can be achieved with Xiaomi’s proposal. OPPO also agrees.

- ZTE also agrees with Xiaomi proposal, but not sure whether we need to capture it in field description.

To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

C-PDU and retransmitted PDU

R2-2502181	On Data Volume Calculations for Rel-19 DSR	Apple, CATT, LG Electronics, OPPO, Samsung, Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold, which is the i-th dsr-ReportingThreshold with i=1 (i.e. Option 2).

Proposal 2: For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, RAN2 selects one of the following alternatives to set the remaining time field for this reporting threshold in the DSR MAC CE:

o	Alt. 1: Set the remaining time field in DSR MAC CE as 0;

o	Alt. 2: Set the remaining time field in DSR MAC CE as the i-th dsr-ReportingThreshold with i=1 (i.e. the smallest reporting threshold);

o	Alt. 3: Leave it to UE implementation.

R2-2501947	Discussion on DSR Enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1	Consider Control PDU and PDUs/SDUs to be retransmitted as the data volume of the smallest reported dsr-ReportingThreshold for the PDCP entity.

Proposal 2	If there is no delay-reporting PDCP SDUs in a PDCP entity, Control PDU is not considered as PDCP data volume for DSR.

Proposal 3	If there is no delay-reporting PDCP SDUs in a PDCP entity, PDUs/SDUs to be retransmitted are not considered as PDCP data volume for DSR.

Proposal 4	RLC data volume calculation and PDCP data volume calculation on Control PDU and PDUs/SDUs to be retransmitted are consistent with each other.

DISCUSSION on first configured vs first reported threshold:

- QCM has no strong preference, but prefers a simpler option, with less conditions to check. We can use some special value to report and NW decides how to treat this data.

- LGE prefers Apple way. LGE think that with the proposal from Sharp the calculation of delay critical data will differ from R18 calculation and they prefer to have aligned method. Also, it will cause cross-layer interactions which should be avoided.

- Lenovo thinks we should not treat retransmitted PDUs differently from initially transmitted PDUs. It makes more sense to report them according to the remaining time. Apple clarifies that this is to align with R18 DSR where retransmitted data is always treated as delay critical data.

- IDT thinks that we do not specify anything special for retransmitted data and we should consider remaining time for this data.

- LGE clarifies that with the current PDCP specifications, retransmitted data is always delay critical data.

- CMCC agrees with Lenovo, even for retransmission we should consider remaining time.

- Xiaomi thinks that remaining time indicated in DSR only considers remaining time of new data.

- Huawei agrees with Lenovo view. Previously there was one threshold, so this is new case.

- OPPO thinks that considering the retransmission will complicate the UE behaviour a lot as the UE needs to recheck the remaining time.

- Nokia agrees with Lenovo, the point is to indicate the network what is the time left.

- Ericsson thinks that this is relate also with non-delay critical ahead of delay critical. Now we would also put the retransmitted data in the wrong threshold.

- Apple is mainly concerned about UE complexity, DSR is already very complex with multiple thresholds.

- QCM thinks that always configured works in all cases, but smallest reported does not. Nokia clarifies the DSR is triggered only if there is data to report.

During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

Non-delay critical data related clarifications

R2-2502849	Discussion on DSR enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1. Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

Proposal 2. Use COUNT value to determine non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation.

On P1:

- Xiaomi wonders whether we need to define non-delay reporting data? Vivo thinks we have a definition of delay-reporting data so all other data is non-delay reporting data.

On P2:

- Xiaomi thinks we can just say “data ahead of”. LGE thinks it is hard to specify this and interpret when implementing, so using COUNT is clearer.

- QCM thinks this proposal assumes a specific UE implementation, e.g. depends on when the UE assigns SN.

- Samsung thinks COUNT value is usable for this purpose. Since the data is delay critical, it should have SNs assigned already.

- Fujitsu does not think COUNT value is suitable. COUNT may not reflect an order in the buffer. Also some channels with higher priority should be treated as “ahead of delay reported data).

- IDT thinks maybe we over-specify this, while details can be left to UE implementation.

- Nokia thinks using COUNT value seems most straightforward.

- Mediatek has strong concerns on an impact on chipset implementation with this. Thinks it should be left to UE.

- LGE thinks it is hard to specify in PDCP specifications.

- Samsung thinks that with different implementations there is a fairness issue which is a big concern.

- Nokia also worries that the UE may not report it if it is left to UE implementation.

Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review)

We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

Coexistence with Rel-18 DSR

R2-2502751	Remaining Issues on DSR enhancements in Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2. RAN2 to confirm that enhanced DSR MAC CE is used if at least one LCG is configured with dsr-ReportingThresList.

Proposal 3. RAN2 to agree on the name of the new DSR MAC CE as Enhanced DSR MAC CE.

Proposal 4. RAN2 to confirm that, enhanced DSR MAC CE inclusively serves the LCG(s) not configured with reporting threshold, but with pending DSR, and its field descriptions should reflect it.

DSR/BSR cancellation issues

R2-2502301	Discussion on DSR enhancements in XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5:	RAN2 to specify that all triggered BSRs can be cancelled if amount of data to be reported in the BSR is already indicated by the DSR included in the same MAC PDU, as given in Appendix.

Proposal 6:	In Dual Connectivity case, a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted.

R2-2501763	Discussion on DSR enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	For Rel-19, define an optional UE capability, which is per band combination, for the support of canceling pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

Proposal 2.	For UEs that support the capability defined in Proposal 1, network configures UE whether it shall cancel pending DSRs in both MAC entities when an associated DSR MAC CE is sent in one of the MAC entities.

R2-2501816	Discussion on DSR enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501839	Discussion on DSR enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501871	Consideration on DSR Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502035	Discussions on DSR enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502060	Remaining issues on DSR enhancements	TCL	discussion

R2-2502164	Remaining issues on DSR enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502204	DSR enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2502266	DSR Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502296	Considerations on DSR enhancements for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502364	Enhanced delay status reporting for XR	Lenovo	discussion	Rel-19

R2-2502478	DSR enhancements for UL scheduling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502563	Remaining Issues on DSR Enhancements	China Telecom	discussion

R2-2502719	Further consideration on DSR enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502877	Discussion on XR DSR enhancements	III	discussion	NR_XR_Ph3-Core

### 8.7.5	RLC enhancements

Further details of autonmous retransmission and enhanced polling mechanisms (e.g. timer to be used) and unnecessary retransmission avoidance.

Unnecessary retransmissions

R2-2502218	On RLC Status report after discarding outdated AMD PDU	Futurewei	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal. The UE shall not autonomously send a Status Report when t-RxDiscard expires.

R2-2501840	Discussion on RLC enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: When the t-RxDiscard expires and all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, the expiration of t-RxDiscard can trigger an SR.

Proposal 2: If it is agreed that the t-RxDiscard expiration can trigger an SR in some cases, RX_Highest_Status should be updated before triggering SR.

Proposal 3: In the case that all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, t-Reassembly should be stopped or restarted.

DISUCSSION:

- Nokia agrees with FTW, there is no need for SR to be triggered.

- Sharp also agrees with FTW and the side effect can be that we trigger unnecessary retransmissions.

- Samsung prefers to trigger the SR and the prohibit timer will prevent unnecessary reTx.

- OPPO also agrees with FTW and Nokia.

- vivo would also like to trigger SR, but there is no need to combine t reassembly and t discard operation.

- Huawei thinks SR should be triggered to advance the window at Tx. T Rx discard should be shorter than t reassembly.

- LGE has similar understanding as Huawei, it is important to send SR to solve the window stalling problem. We should send the SR without additional conditions.

- Fujitsu agrees with Huawei and LGE.

- Ericsson agrees with LGE, but has not strong preference. It could also be solved by proper timer configurations.

- Lenovo also thinks we need SR after timer expiry.

- Apple indicates this is for packets with short PDB. There were previously concerns with Tx window stalling and SR is needed for that.

- QCM thinks SR is needed at least for some UEs.

- MTK thinks SR is not needed.

- IDT wonders what the benefit of not sending the SR is.

- FTW still has concerns with the agreement.

When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

PDCP or RLC timer for polling/retransmission

R2-2502036	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1: for autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP.

R2-2502436	Discussion on timely RLC retransmission(s)	Spreadtrum, UNISOC	discussion	Rel-19

Noted

Proposal 1: Introduce a new timer in RLC layer for autonomous RLC retransmission.

Proposal 2: Discuss the following two options for the value of the new timer:

Option 1: value of new timer = PDCP remaining time

Option 2: value of new timer = PDCP remaining time – threshold

Proposal 3: The new timer for one RLC PDU should start when RLC Tx side receives the RLC SDU from PDCP layer.

Proposal 4: The new timer should terminate after RLC Tx side received positive acknowledgement of the corresponding RLC PDU.

DISCUSSION:

- Nokia thinks there has to be a possibility to use HARQ reTx for a PDU, so the timer needs to be at RLC layer and started when PDU goes to MAC layer.

- QCM, Huawei agrees with Nokia.

- Huawei thinks the retransmission should be triggered only after initial transmission.

- CMCC also supports new timer at RLC layer and the length should be shorter than PDCP discardTimer.

- Lenovo would like to reuse PDCP timer as we do not know when PDCP submits SDU to lower layers.

- Samsung would also like to reuse PDCP timer, retransmission should be decided based on remaining time.

- Apple thinks there is much additional impact due to new timer, e.g. length, stopping etc. Xiaomi agrees.

- OPPO thinks we need to ensure that PDU is sent within PDB so we should reuse PDCP timer.

- LGE thinks it is a valid point that it’s implementation when to submit SDU to lower layers from PDCP. Discard timer expresses correct remaining time.

- Ericsson slightly prefers PDCP timer, the problem with RLC timer is that it can be non-synced with PDCP timer, so we may get unnecessary reTx.

- IDT thinks we need to somehow use discard timer anyway as it indicates how much remaining time there is.

- ZTE would slightly prefer RLC timer.

- Nokia thinks we need to avoid unnecessary retransmissions.

- Lenovo agrees we need some additional conditions to avoid too early reTx.

For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

Further conditions for autonomous retransmission

R2-2502501	Remainning details on RLC AM enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 5: Auto-Rx is triggered only for SDU pending for feedback, i.e., it should be neither pending for retransmission already nor positively acknowledged by a STATUS PDU. RAN2 consider the TP as shown in section 3.2 to merge auto-retransmission into retransmission.

R2-2502479	Discussion on RLC enhancements	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2: 	Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been fully transmitted by lower layers.

Proposal 3: 	Autonomous retransmission is not triggered if the RLC SDU (segment) is already considered for retransmission (e.g., based on receiving negative acknowledgement).

DISCUSSION:

- QCM supports those conditions, but wonders how RLC knows when HARQ process is finished. Perhaps some timer is needed?

- IDT thinks that perhaps we can base on whether it was submitted to lower layers, not fully transmitted.

- Huawei supports first bullet, but second one is related to what we have already agreed. Huawei thinks it should be linked with PDU set.

- LGE would like to use “submitted to lower layers” as a baseline condition.

- LGE thinks the intention is to consider only those SDUs which are not already considered for retransmission.

- Ericsson is not sure for the second one.

Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

After offline discussion on “FFS Autonomous retransmission is not triggered if the RLC SDU (segment) is already considered for retransmission”:

- IDT clarifies that companies seem to agree that we should only have one retransmission (either autonomous or normal). How this impacts specifications can be further checked.

Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

Retransmission count and RLF detection impact

R2-2502972	Remaining issues for XR RLC AM enhancement	MediaTek Inc.	discussion	Rel-19	38.322	NR_XR_enh-Core

Proposal 2: RETX_COUNT should be increased by 1 due to autonomous retransmission.

R2-2501802	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3	RETX_COUNT is not incremented for autonomous retransmission.

Proposal 4	RAN2 to discuss whether/how to solve RLC-based RLF miss-detection issue (i.e., maxRetxThreshold cannot be reached) caused by in-flight PDU discard enhancement.

R2-2502401	Discussion on RLC AM Enhancements 	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Stopping the retransmission of a RLC PDU due to a remaining time condition shall not trigger an RLF.

Misc FFS points from the running CR

R2-2501872	Consideration on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: For the autonomous retransmission and legacy retransmission, they will be transmitted in the order SN of RLC SDU or segment, no further spec impact will be introduced.

Proposal 3:  Only a single enhanced polling will be triggered per RLC SDU.

R2-2502223	Remaining issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4. Even if polling is included based on the remaining time, no special handling to avoid multiple polling is introduced.

Proposal 5. If the remaining time of an RLC SDU falls below a specified threshold, the RLC entity should include a poll even when the RLC SDU is retransmitted. (Related TP is given below)

Status reporting enhancements

R2-2501764	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	When sending a poll, the transmitter includes an indication whether it is a time-critical poll (i.e. triggered due to remaining time below a threshold).

Proposal 2.	Upon receiving a time-critical poll, the receiver triggers a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

Proposal 3.	When the remaining value of t-RxDiscard is below a configured threshold, the receiver sends a time-critical status report without the restriction by t-StatusProhibit (or apply a shorter t-StatusProhibit).

R2-2502491	Timely retransmissions for RLC AM	Sony, Canon	discussion	Rel-19	NR_XR_Ph3

Proposal 1: After Rx side receives polling information, the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running when generating status report.

Proposal 2: For any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/ redefining the corresponding reserved bit as 1 (R = 1).

R2-2502277	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

UE capabilities

R2-2502673	Discussion on UE Capabilities for RLC AM Enhancements	Ericsson, Qualcomm Incorporated, ZTE Corporation, MediaTek Inc.	discussion	Rel-19

Proposal 1	Individual (optional) per-UE capabilities with signalling are defined for the autonomous retransmission and the polling enhancement feature.

Proposal 2	Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

Proposal 3	Define an (optional) per-UE capability with signalling for the Tx-side aspect, conditional on the UE capability for the Rx-side, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP.

R2-2501881	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501948	Discussion on RLC Enhancements in XR	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502165	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502182	Discussions on Fast RLC Retransmission	Apple, Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502183	Views on Avoidance of Unnecessary RLC Retransmissions	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502203	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2502222	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502302	Further discussion on RLC retransmission for XR	Quectel	discussion

R2-2502365	AM RLC enhancement	Lenovo	discussion	Rel-19

R2-2502564	Consideration on RLC AM Enhancements	China Telecom	discussion

R2-2502710	Discussion on the left issue of RLC enhancements	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502853	Discussion on RLC enhancements	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502860	RLC Enhancements for XR	Samsung	discussion	Rel-19

R2-2502872	Remaining issues on timely RLC retransmission	TCL	discussion	Rel-19

### 8.7.6	XR rate control

Including details of per QoS flow indication, e.g. QoS flow ID in MAC CE or in RRC, new bit rate table design, whether multipliers are needed etc.

Rate indication details

R2-2501949	Discussion on Rate Control in XR	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 2	In Rel-19 UL Rate Control MAC CE, the size of bit rate field is 7 bits or 8 bits.

Proposal 3	Bit rate field supports the value range from 100kbps to 40Mbps.

Proposal 4	Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

Proposal 5	The value table of bit rate field uses an exponential distribution.

On P4:

- Nokia is OK with physical if we make it clear. LGE is also OK.

- Meta is not clear why we had PHY layer in legacy but prefers app layer.

- Ericsson thinks gNB measures physical layer, so it has to be physical. QCM agrees and indicates the gNB has no idea how to translate PHY bitrate to application layer bit rate. Vivo agrees.

- MTK thinks it should be app layer.

- Sharp think the situation is the same as in Rel-14, there is some interaction between app layer and lower layers, but MAC indicates PHY layer and we can follow this.

- Huawei think it should be app since it is indicated to and used at app layer.

- Ofinno also indicates that MAC actually clarifies how this is calculated.

R2-2502883	XR rate control	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

Noted

Proposal 1: Multiple tables with distinct ranges and granularities are specified for XR rate control.

R2-2502088	Discussion on XR rate control	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 3: Define a rate control table with a smaller range, e.g. 100kbps ~ 1000kbps, with 1% exponential distribution.

Proposal 4: Use dynamic multipliers to extend the range of the recommended rate indication to cover the whole range of rate from 100kpbs to 40Mbps.

DISCUSSION on single vs. multiple tables vs a table with multiplier:

- Ericsson thinks another solution would be to have one table with linear distribution. Exponential does not work well for gradual increase.

- CATT thinks we should have an exponential distribution.

- LGE indicates that with 10 bits we can get 1% granularity Multiple tables and multiplier require additional bits as well. Single table with more bits is preferred instead.

- QCM would like a single table as we can achieve the same granularity, but we need more discussions on the details.

- Lenovo has similar views with Ericsson and also prefers a single table. Sony agrees.

- Xiaomi thinks the range is already clear and one table is sufficient. HONOR agrees.

- Nokia thinks with one table we can reduce the number of codepoints with multiple tables but is OK with single table. Nokia thinks no multiplier is needed.

- Futurewei thinks signalling needs to be adapted to what codec can use. Do not think 1% granularity is needed.

- Ericsson think we can first decide number of bits and then decide distribution. ZTE thinks we can first choose type of distribution and then decide the exact codepoints.

- Vivo thinks we should decide both together, propose exponential with 8-9 bits.

- Huawei thinks 1% requires 10 bits while 8 bits allows for 5% which should be sufficient. Exponential distribution is preferred.

- LGE agrees 8 bits should be sufficient and also prefers exponential distribution.

- Lenovo thinks SA4 said 10% is sufficient.

- ZTE thinks we should clarify no multiplier will be introduced. Huawei thinks this can impact the overhead, so would not like to exclude multiplier for now.

Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

We specify a single table.

As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered.

Rate query

R2-2502037	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: The working assumption is changed to agreements on rate query MAC CE.

Proposal 4: After the generation or transmission of the rate query MAC CE, the triggered rate query is cancelled.

Proposal 5: The upper layer can cancel the pending rate query.

The working assumption is changed to agreements on rate query MAC CE.

R2-2502166	Discussion on XR rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6	NW configures a prohibit timer for each QoS flow to avoid frequent XR rate control query.

Proposal 7	One codepoint of bit rate (i.e., the maximum value of the recommeneded bit rate) is defined for the case of congestion relief; upon reaching this bit rate, the UE is allowed to send an XR rate control query for a proper bit rate in case that an XR rate control MAC CE has been received previously.

MAC CE granularity

R2-2501765	Discussion on XR rate control	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Noted

Proposal 1.	Per-QoS-flow rate indication by RRC configuration is the baseline. Support for per-QoS-flow rate indication within MAC CE is an optional UE capability.

Proposal 2.	If rate indication for a DRB is based on RRC configuration, network configures UE with a ratio for allocating the indicated rate among QoS flows in the DRB.

Proposal 3.	Send an LS to SA2/SA4/RAN3 to inform them about Proposal 2 and request them to discuss and specify any remaining design details, if applicable.

R2-2502205	Data rate control for XR applications	ZTE Corporation, Sanechips	discussion

Noted

Proposal 1: If the understanding is that both solutions have RAN3 impact, then RAN2 should select the MAC based solution which seems to be the most flexible one (i.e. gNB to indicate the recommended bit rate on a per QoS level basis in the MAC CE)

Proposal 2: RAN2 should send an LS to RAN3 to inform them about the agreed solution and ask RAN3 to define any corresponding F1 signalling (if needed)

DISCUSSION:

- Meta support ZTE’s approach. Since there is similar impact on F1 from both solutions, it is straightforward to use MAC CE, it is up to gNB how to estimate rate.

- Nokia thinks RAN3 has no consensus on exact impact, Nokia thinks we need to wait for further information.

- Samsung supports ZTE’s approach. We can focus on what we want from R2 point of view and R3 can discuss impact.

- OPPO also supports MAC CE approach, both solutions have F1 impact. OPPO thinks we need to be able to adapt multiple flows in a single DRB.

- Xiaomi prefers combined approach. Xiaomi is concerned about the overhead of flow indication.

- CMCC prefers semi-static indication for QoS flow in RRC and then DRB is indicated in MAC CE.

- Lenovo prefers ZTE’s approach. There will be overhead with RRC solution as we may need to send reconfigurations via RRC.

- Ericsson indicates that we agreed that gNB can estimate QoS flow rate, but we did not discuss whether this is DU or CU which needs to be discussed by R3. If R3 decides it needs to be done in CU, then there is more F1 signalling. Would like to go with DRB indication as a baseline.

- Vivo thinks based on current feedback from R3 we should select an approach simpler from R2 side.

- Ofinno also thinks we should select per flow MAC CE, but we can make it conditional on R3 impact and can fallback to RRC solution based. Apple agrees, we can make a working assumption and if R3 concluseds otherwise, we can revisit.

- LGE thinks PDCP is an anchor point, so that is why both solutions require CU decision, so per flow is more straightforward, but we can also wait due to NW concerns.

- CATT supports ZTE’s solution and thinks it will help RAN3 and make more work for them.

- ZTE thinks RAN2 should make the decision and R3 should work on the solution accordingly, they did not indicate sth is not feasible. ZTE is afraid R3 cannot make the decision. We should make a decision on which is better from R2 point of view, but if this is not feasible then we can revert.

- QCM thinks that R3 assumed that there is F1 impact for RRC solution due to rate ratio indication, but this is not the only solution, but there are other solutions.

- Nokia thinks both solutions can control flow, MAC CE is more flexible, but it is not clear why we need such flexibility. Nokia does not see need for too much flexibility.

- LGE thinks the gNB has no possibility to indicate specific flow with RRC based solution.

- Ericsson thinks that RRC solution allows to use information from CN about the flows. Ericsson thinks we need at least LCID indication and then flow ID is on top.

- Ericsson thinks we need to discuss how we indicate QFI in MAC CE, there are different ways. E.g. DRB ID could be indicated and then QFI would be optional.

- Ericsson thinks we do not have to make working assumption and just wait for R3.

- Vivo thinks decision should be made by R2 as this is R2 feature and R3 can feedback us.

- Sharp thinks QoS flow ID indication is enough as one QoS flow maps to one DRB.

- Nokia thinks per flow indication has feasibility issues for UE chipset. Apple and Samsung do not see such issues.

From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this.

FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Single or multiple flow/DRB in MAC CE

R2-2502957	Discussion on Rate Control for XR	China Telecom	discussion

Proposal 2: One rate control MAC CE can comprise multiple flows, and the network can configure the maximum number of QoS flows per MAC CE via RRC.

Proposal 3: New MAC CE should be specified in Rel-19 to support Per-QoS flow granularity recommended bit rate.

Dual-connectivity issue

R2-2502298	Uplink rate control for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: RAN2 to discuss how to apply the indicated recommended bit rate for a QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

R2-2501841	XR rate control	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501873	Discussion on XR Rate Control	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2501882	XR rate control	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502224	Remaining issues on rate control signaling for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502461	XR Rate Control	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502480	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502492	Recommended bit rate based XR rate control	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2502674	Views on XR Rate Control	Ericsson	discussion	Rel-19

R2-2502720	Further consideration on XR rate control	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502750	Discussion on UL rate control for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502762	Discussion on XR rate control	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2502889	Discussion on RAN Awareness and UL Rate Control for XR	Meta	discussion

R2-2502890	Discussion on XR rate control	MediaTek Inc.	discussion	Rel-19	38.321	NR_XR_enh-Core