3GPP TSG-RAN WG2 Meeting #130                                                                R2-2503356

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.9.4

Source:	vivo

Title:	Remaining Issues on PWS Support for NB-IoT

Document for:	Discussion and Decision

- Introduction

In RAN2#128 [1], the following open issues were made by RAN2 on PWS support for NB-IoT.

Agreements: 1. We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. 2. We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB-NB acquisition is needed 3. Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication.

Besides, according to the open issue list provided by the rapporteur [2], it is FFS whether to support the inter-cell PWS reception for NB-IoT NTN.

In the contribution, we will continue to analyse the following issues and give our views.

- FFS to skip reading SIB1-NB for PWS acquisition;

- FFS to allow UE to receive and assemble PWS segments from different cells during mobility.

- Discussion

## 2.1 FFS to skip reading SIB1-NB for PWS acquisition

When we reviewed the spec, we found that the normal procedure for PWS acquisition includes the following steps:

Step 1: UE acquires the PWS notification either via direct indication information or via the paging message;

Step 2: UE acquires the SIB1-NB and decides the PWS scheduling information;

Step 3: UE acquires the corresponding PWS message in the corresponding resource.

Observation 1: PWS acquisition using the current mechanism leads to comparable long latency.

However, considering the urgency of the PWS message, the earlier the UE acquires the PWS message the earlier UE can make the corresponding response. Moreover, due to the long RTT in NTN scenario, the problem will be more serious. Optimizations can be researched to enable the UE to accelerate the PWS acquisition。

The candidate solution can be:

Option 1: NW provides the PWS scheduling information within SIB1-NB even though there is no PWS notification, and UE can avoid reading the SIB1-NB upon acquiring the PWS message.

According to the spec [3], the PWS information change does affect the value tag of the SIB1-NB. Thus for this option, it is totally up to NW implementation, i.e., NW always includes the PWS scheduling information even if there is no PWS notification. From the UE perspective, it can use the stored SIB1-NB directly upon receiving the PWS notification. One may argue there may be some resource consumption, but considering the urgency of the PWS, such a solution is also beneficial. Besides, option 1 can be totally up to implementation, there is no spec impact.

E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite assistance information. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

Option 2: NW provides the PWS scheduling information or the PWS message directly within the paging message.

For this option, providing the PWS scheduling information in the paging message has a similar performance as option 1, while providing the PWS message in the paging message may be faster for PWS acquisition by UE. But the spec impact is larger, and it may impact RAN1, i.e.,  the PWS notification must be included in the paging message since the current LTE spec does not support the paging message and the direct indication information is provided at the same time [4]. Moreover, the signalling overhead is larger due to indicating the scheduling information or the PWS message in the paging message.In the following, we make a comparison of the above options. And it is clear the option 1 is better.

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | UE can avoid reading the SIB1-NB to accelerate the PWS acquisition; No spec impacts. | Resource consumption is reserved for PWS. |
| Option 2 | UE can avoid reading the SIB1-NB to accelerate the PWS acquisition. | Spec impacts are required; RAN1 may be involved; Signaling overhead. |

Besides, according to the following description [3], UE can assume the scheduling information in SIB1(-NB) is unchanged when acquiring the SIB31(-NB), that is, NW can provide the corresponding scheduling information in advance. Such a mechanism is similar to option 1. Thus, we think there is no complicated spec impact for option 1.

NOTE 3: When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

Observation 2: The current spec already supports the NW to provide the SIB31(-NB) scheduling information to UE before the SIB31(-NB) is scheduled.

Proposal 1: To avoid UE reacquire the SIB1-NB upon acquiring the PWS message, the network provides the SIB10-NB/SIB11-NB/SIB12-NB scheduling information within SIB1-NB.

Proposal 2: When acquiring SIB10-NB/SIB11-NB/SIB12-NB, UE may assume that the scheduling is unchanged.

## 2.2 FFS to allow UE to receive and assemble PWS segments from different cells during mobility

During re-selection from the source cell to the target cell, UE may already have received some of the PWS segments from the source cell. However, according to the current spec, the PWS segments from the source cell should be dropped [3].

1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11; 2> when the UE acquires SystemInformationBlockType1 following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> if schedulingInfoList indicates that SystemInformationBlockType10 is present: 4> if the UE is in CE: 5> start acquiring SystemInformationBlockType10; 4> else 5> start acquiring SystemInformationBlockType10 immediately; 3> if schedulingInfoList indicates that SystemInformationBlockType11 is present: 4> start acquiring SystemInformationBlockType11 immediately; NOTE 2: UEs shall start acquiring SystemInformationBlockType10 and SystemInformationBlockType11 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed. 1> if the UE is CMAS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment; 2> when the UE acquires SystemInformationBlockType1 following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment: 3> if schedulingInfoList indicates that SystemInformationBlockType12 is present: 4> acquire SystemInformationBlockType12; NOTE 3: UEs shall start acquiring SystemInformationBlockType12 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

Some companies proposed to support the PWS reception across different cells since the NTN cell is moving, and it is more likely for UE to change the camped cell or serving cell. We agree that it may be beneficial to support the PWS reception for the inter-cell NTN scenario. However. supporting the PWS reception across cells has some specific inpacts. Currently, the PWS message send by the MME is a complete PWS message content along with messageIdentifier and serialNumber. And then up to eNB, i.e., based on the radio link quality, it segments the message and associates with each segment a unique segment number. Supporting the PWS message reception and assembly across different cells requies the coordination of eNBs. Moreover, there are also some RAN2 impacts.  A summary of the impacts to support the functionality can be seen as the following.

RAN2 impacts:

- UE needs to be aware of the received PWS segments from which cells can be kept and combined.

- New UE capability may need to be introduced even for test purposes.

RAN3 impacts:

- The coordination between eNBs is required to ensure the same association of the segment ID and the content of the PWS message;

Observation 3: Both RAN2 and RAN3 impacts are foreseen to support the PWS reception across cells.

However, according to the WID [5], only RAN2 is within the scope, and the PWS support is only for NB-IoT scenario.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

Observation 4: Only RAN2 and NB-IoT NTN are within the scope for the PWS enhancement.

Since there is RAN3 impact, and it is not clear whether the legacy implementation, i.e., OAM, can ensure that different eNBs make the same segments of the received PWS message.  So it is proposed that RAN2 confirms RAN3 before discussing the issue.

Proposal 3: RAN2 to check with RAN3 whether the current RAN3 spec can support UE to receive and assemble PWS segments across different cells.

- Conclusion

In this contribution, we have further discussed the PWS support for NB-IoT. And we make the following observations and proposals:

FFS to accelerate the PWS acquisition:

Observation 1: PWS acquisition using the current mechanism leads to comparable long latency.

Observation 2: The current spec already supports the NW to provide the SIB31(-NB) scheduling information to UE before the SIB31(-NB) is scheduled.

Proposal 1: To avoid UE reacquire the SIB1-NB upon acquiring the PWS message, the network provides the SIB10-NB/SIB11-NB/SIB12-NB scheduling information within SIB1-NB.

Proposal 2: When acquiring SIB10-NB/SIB11-NB/SIB12-NB, UE may assume that the scheduling is unchanged.

FFS to support the inter-cell PWS reception:

Observation 3: Both RAN2 and RAN3 impacts are foreseen to support the PWS reception across cells.

Observation 4: Only RAN2 and NB-IoT NTN are within the scope for the PWS enhancement.

Proposal 3: RAN2 to check with RAN3 whether the current RAN3 spec can support UE to receive and assemble PWS segments across different cells.

- Reference

- Draft_RAN2_128_Meeting_Report_v1.

- [Post129bis][310][R19 IoT NTN] RRC CR (Huawei).

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- TS36.212, Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding.

- RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.