3GPP TSG-RAN WG2 Meeting #130		R2-2503371

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	vivo

Title:	Discussion on functionality management for AIML mobility

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

At the RAN2#129-bis meeting, the following agreements were reached regarding applicability reporting and inference configuration:

L3 Beam-level prediction specification impact will be studied Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference - Agreements - For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details - Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP. Agreements - The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework - For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message. - FFS whether association ID is required and should provide motivation on per use case bases - As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework. - Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20. - Model transfer will not be discussed in this study item 7 For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In this contribution, we will further discuss issues related to applicability reporting and inference configuration.

- Discussion

- 2.1 Necessity of associated ID

For beam management use case, NW-sided additional conditions (mainly including associated ID) were discussed in RAN1 for the determination of the applicability of a functionality, which aims to ensure the consistency between model training and inference configuration without exposure of NW implementation of Tx beam. The UE may assume the similar properties of a Tx beam or beam set/list associated with the same associated ID.

Agreement For UE sided model in beam management, introduce support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

At the latest RAN2 meeting, RAN2 discussed the issue and the conclusion was FFS whether association ID is required and should provide motivation on per use case basis. Beam management only focuses on serving cell, and RRM measurement prediction considers both serving and neighbor cells. We think that the underlying logic of RRM measurement prediction and beam management is the same, i.e., prediction accuracy relies on the consistency of DL Tx beam characteristics between training and inference, and the network vendors also have the intention to not expose the network implementation. Therefore, we think the associated ID is also needed for AI/ML-based mobility. Furthermore, as the Tx beam characteristics have an impact on the L1 RSRP, L3 beam-level and L3 cell-level RSRP, the associated ID can be applicable for all the sub-use cases of RRM measurement prediction. And associated ID can be applicable for measurement event prediction as measurement is also based on measurement or measurement prediction.

- Associated ID is needed for UE-sided model in AI/ML-based mobility to ensure the consistency of DL Tx beam properties between training and inference.

- Associated ID scheme can be applicable for all sub-use cases (1-6) and scenarios (temporal/ frequency/ spatial domain prediction) of RRM measurement prediction, and can be applicable for measurement event prediction.

- 2.2 Applicability reporting

For AI/ML-based beam management use case, RAN2 reached the following agreements about applicability reporting:

Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior - Agreements on applicability reporting - Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec - Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. - No prohibit timer is introduced

For AI mobility, the basic procedure for applicability reporting should be similar to beam management. We think that the above general agreements highlighted in yellow (i.e., except for the beam management-specific configuration) can be applicable for AI mobility.

- Take the agreements on applicability reporting of UE-sided model in AI-based beam management as the baseline, including the following aspects:

- Support both Option A and Option B like scheme

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability.

- No prohibit timer is introduced.

- RAN2 assumes UE receives RRCReconfiguration message including one or multiple sets of inference related parameters via OtherConfig for option B.

- 2.3 Inference configuration

In the current AI/ML for mobility study, the following 3 sub-use cases are considered for L3 cell-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Different sub-use case utilizes different types of measurement results as input. However, from the view of the specification, the focus points are the prediction results and how to use the prediction results. One issue is whether to need to configure the input of the model by the network or leave it to UE implementation. We think that the latter is more reasonable.

- For RRM measurement prediction of UE-sided model, the network does not configure the input of inference, and the selection of input is up to UE implementation.

As to the output, in the previous meeting, RAN2 agreed the following two goals:

- Goal 1: measurement reduction

- Goal 2: handover performance improvement

In the whole RRM measurement procedure, there are three phases:

- Phase 1: the UE performs the measurement to obtain the measurement results.

- Phase 2: the UE determines whether to trigger the measurement report based on measurement results.

- Phase 3: if triggered, the UE sets the content of the measurement report and sends the measurement report to the network.

Achievement of Goal 1 depends on measurement reduction during Phase 1. For Goal 1, we think that RAN2 should firstly discuss how to perform the prediction (e.g., prediction granularity) to achieve the measurement reduction goal before discussing the inference configuration.

The main purpose of measurement reduction goal is to save the UE power and achieve no/less measurement gap. There are two main steps when the UE performs the measurement of a frequency in Phase 1:

- Step 1: the UE receives the signal of the frequency via RF receiver.

- Step 2: the UE processes the signal to obtain the measurement result of all the cells in the frequency via baseband.

According to the industry materials, the proportion of power consumption in Step 1 is about 80% and the one in Step 2 is about 20%. In NR, the measurement object is configured per frequency for RRM measurement. It means that the UE needs to measure all the cells for a measured frequency. The difference in power consumption between measuring all the cells of a frequency and measuring one cell of the frequency may be quite small since the main power consumption is receiving the signal of the frequency by the RF receiver. Moreover, if there is any one cell of the measured frequency that needs to be measured with RF retuning, the measurement gap needs to be configured.

Observation 1: The most power consumption of UE measurement is the RF receiver, not the baseband processing. If there is any one cell of a specific frequency that needs to be measured, no gain of measurement gap reduction can be achieved and the gain of power saving is quite limited.

Therefore, we think that for frequency domain prediction, RAN2 should consider the per-frequency granularity prediction (i.e., predict all the cells of a frequency) to achieve the goal of measurement reduction. Similarly, for temporal domain prediction, all the cells of the measured frequency should be predicted in the predicted time instances to save the signal receiving of the RF receiver in the predicted time instances to achieve the measurement reduction effect. In the measurement procedure, the RF receiver needs to receive the signals from multiple different directions for different beams. If some directions can be predicted, the power can be saved. There may be multiple cells for the measured frequency in the predicted directions. Therefore, it is also a type of per-frequency granularity prediction.

- To achieve the goal of measurement reduction, the prediction should be per-frequency granularity, i.e., all the cells in a frequency layer should be predicted, either for temporal domain, frequency domain, or spatial domain prediction for measurement reduction.

Goal 2 is related to Phase 2 and Phase 3. To reduce the power consumption of inference, gNB may provide a list of cells that need to be predicted. Therefore, we propose:

- To achieve the goal of HO performance improvement, the prediction can be per-frequency or per-cell granularity.

Based on the simulation results, the key parameters that have an impact on the performance of RRM measurement prediction are summarized in [1].

Scenario 2 Proposal 2: The key parameters for scenario 2 for graphical illustration are MRRT and UE speed. Proposal 3: Restrict the number of combinations of the key parameters for capturing the simulation results in a graphic illustration in the TR. Proposal 4: For scenario 2 graphic illustration, the UE speed is set to 30 km/h and 90 km/h; MRRT is set to 50%, 66%, 80% and 90%. Scenario 4 Proposal 5: For scenario 4 graphic illustration, the key parameters are [OW:PW] or PW, and UE speed. Proposal 6: For scenario 4 graphic illustration, the UE speed is set to 60 km/h and 120 km/h. Two options will be considered: - OW:PW ratio is set to 1:1, 2:1 and 1:2. The setting of PW and OW will be considered if needed. - Only consider PW value. Scenario 3 Proposal 7: For scenario 3, use the cluster setting for graphic illustration. Scenario 6 Proposal 8: For Scenario 6, the key parameters for graphic illustration are MRRS and UE speed. If necessary, graphic illustrations including clustering can be added later. Proposal 9: For Scenario 6, rely on the rapporteur to illustrate the findings based on the available simulation results in a best-effort manner. We can later discuss whether to capture those findings in the TR.

To ensure the prediction accuracy, the network should be able to configure the key parameters that have an impact on the performance:

- For temporal domain case B prediction, the configuration may include information related to MRRT, e.g., explicit MRRT value or skipping pattern.

- For temporal domain case A prediction, the configuration may include information related to PW length, e.g., explicit PW length value or instance number& instance time gap.

- For spatial domain prediction, the configuration may include information related to MRRS, e.g., explicit MRRS value or input/output beam pattern.

Therefore, we propose:

- For RRM measurement prediction of UE-sided model, the network may configure the following parameters in inference configuration:

      - Temporal domain case B prediction: information related to MRRT, e.g., explicit MRRT value or skipping pattern;

      - Temporal domain case A prediction: information related to PW length, e.g., explicit PW length value or prediction instance number& instance time gap;

      - Spatial domain prediction: information related to MRRS, e.g., explicit MRRS value or input/output beam pattern.

As to the UE speed, it was concluded that a higher UE speed will result in decreased prediction accuracy. However, the NW can not control the UE speed, while can estimate UE speed based on the UE history information and RRM measurement. The NW can send appropriate inference configurations to UE at certain speeds. Besides, if the prediction accuracy of a high-speed UE exceeds the accuracy requirement, the NW can release the inference configuration based on the performance monitoring result. Similar solution is also applicable to NW-side model.

Therefore, we propose:

- For RRM measurement prediction of UE-sided model, no need to include information related to UE speed in inference configuration.

- 2.4 When to perform inference?

At the RAN2#129bis meeting, RAN2 agreed that the UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC and it is FFS when the UE can perform inference. Regarding the FFS issue, as known, there are two goals of measurement reduction and mobility performance improvement. For different goals, the time of performing inference should be different. In general, for measurement reduction, the UE should perform the inference in the center of serving cell, not in the edge of serving cell, and for mobility performance improvement the UE should perform the inference in the edge of serving cell, not in the center of serving cell. Therefore, we think that event-triggered inference should be supported to avoid unnecessary inference.

- Support event-triggered inference for both RRM measurement prediction and measurement event prediction.

- 2.5 Handover model options

For the system level simulation, RAN2 discussed 2 handover model options for measurement event prediction based on intra-frequency temporal domain case A, as below (extranced from [2]):

The handover model is defined to facilitate SLS, where measurement event is predicted based on either intra-frequency temporal domain case A or intra-frequency temporal domain case B. For both cases, network starts with 40ms handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After handover command, 40ms execution duration is assumed. If measurement event is predicted based on intra-frequency temporal domain case A, there are two options w.r.t. how to decide on the time point to transmit handover command: Option 1: Relying on legacy measurement event Option 2: Relying on predicted measurement event Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

We understand that the introduction of handover model is only for the purpose of facilitating the system-level simulation. From the view of specification impacts, the selection of handover model is up to NW implementation. To be specific, for temporal domain prediction case A, the UE needs to report the predicted timing of the measurement event to NW, and it’s up to NW to make HO decision based on either the predicted measurement event or wait for the legacy measurement event report.

- For measurement event prediction based on temporal domain case A, the UE reports the predicted timing of the measurement event to the network.

- The selection of handover model (e.g., option 1 or 2) of temporal domain case A is up to network implementation, i.e., the network can make HO decision based on either the predicted measurement event or the legacy measurement event.

- Conclusion

In the contribution, we have the following observations and proposals:

Necessity of associated ID

- Associated ID is needed for UE-sided model in AI/ML-based mobility to ensure the consistency of DL Tx beam properties between training and inference.

- Associated ID scheme can be applicable for all sub-use cases (1-6) and scenarios (temporal/ frequency/ spatial domain prediction) of RRM measurement prediction, and can be applicable for measurement event prediction.

Applicability reporting

- Take the agreements on applicability reporting of UE-sided model in AI-based beam management as the baseline, including the following aspects:

- Support both Option A and Option B like scheme

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability.

- No prohibit timer is introduced.

- RAN2 assumes UE receives RRCReconfiguration message including one or multiple sets of inference related parameters via OtherConfig for option B.

Inference configuration

- For RRM measurement prediction of UE-sided model, the network does not configure the input of inference, and the selection of input is up to UE implementation.

Observation 1: The most power consumption of UE measurement is the RF receiver, not the baseband processing. If there is any one cell of a specific frequency that needs to be measured, no gain of measurement gap reduction can be achieved and the gain of power saving is quite limited.

- To achieve the goal of measurement reduction, the prediction should be per-frequency granularity, i.e., all the cells in a frequency layer should be predicted, either for temporal domain, frequency domain, or spatial domain prediction for measurement reduction.

- To achieve the goal of HO performance improvement, the prediction can be per-frequency or per-cell granularity.

- For RRM measurement prediction of UE-sided model, the network may configure the following parameters in inference configuration:

      - Temporal domain case B prediction: information related to MRRT, e.g., explicit MRRT value or skipping pattern;

      - Temporal domain case A prediction: information related to PW length, e.g., explicit PW length value or predicion instance number& instance time gap;

      - Spatial domain prediction: information related to MRRS, e.g., explicit MRRS value or input/output beam pattern.

- For RRM measurement prediction of UE-sided model, no need to include information related to UE speed in inference configuration.

When to perform inference?

- Support event-triggered inference for both RRM measurement prediction and measurement event prediction.

Handover model options

- For measurement event prediction based on temporal domain case A, the UE reports the predicted timing of the measurement event to the network.

- The selection of handover model (e.g., option 1 or 2) of temporal domain case A is up to network implementation, i.e., the network can make HO decision based on either the predicted measurement event or the legacy measurement event.

- Reference

- R2-2501557 	[AT129][022][AI Mob] Simulations (Mediatek)	Mediatek Inc.

- R2-2501822	Text proposal of TR 38.744 V0.0.7.