3GPP TSG RAN WG2 Meeting #130	R2-2503380

St. Julian’s, Malta 19-23 May 2025	(resubmission of R2-2502443)

Agenda Item:	8.1.2.3 LCM for Positioning use case

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Association of measurements and ground truth labels for positioning use-cases

Document for:	Discussion & Decision

# Introduction

The WID [1] , lists, among others, the following as objective for RAN2:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In RAN2#127bis, we reached the following agreements:

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In RAN2#129, we reached the following agreements:

Agreements: 1: Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation, 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

We have so far not discussed the ground truth labels (or “true” position estimates) that are needed for (i) training an ML model (ii) monitoring performance of a deployed AI/ML model.

In this contribution, we discuss the importance of acquiring accurate ground truth labels for AI/ML model to function, and why the labels acquired using the currently supported positioning methods do not provide enough high-quality labels for training and monitoring AI/ML models.

For an AI/ML model to provide accurate inference results, it is essential to collect a sufficient number of high-quality labels, even in challenging scenarios. It would not be practical to deploy an extremely high number of PRUs, which would have enabled collection of a large number of high-quality labels and measurements. Therefore, we need to rely also on UEs that are not deployed as PRUs to provide labels.

GNSS is typically unavailable indoors, and in scenarios dominated by NLOS and heavy multipath, labels from legacy RAT-dependent positioning methods are often inaccurate. As a result, high-quality labels are primarily collected in areas where GNSS or RAT-dependent methods are already sufficiently accurate. This means, we do not have enough high-quality data labels in the areas where AI/ML is envisaged to improve positioning performance. Training AI/ML models with channel measurements linked to inaccurate labels may result in poor inference accuracy when deployed.
Thus, using an external source of ground truth to acquire labels and associating them with the measurements reported in ProvideLocationInformation becomes essential.

To achieve this goal, the ProvideAssistanceData information may be used to provide assistance information to the UE to assist the UE in identifying sources of ground truth, which can be used to determine ground truth labels in scenarios where existing positioning methods cannot be relied upon to provide good quality labels.

The labels may be collected from UEs, subject to their capabilities and consent, by detecting and processing external sources of information such as QR codes, RFID tags, and landmarks.

Observation 1: Legacy 3GPP positioning methods provide inaccurate labels in training data, particularly in the NLOS-heavy and multipath-dominated scenarios where AI/ML is expected to outperform traditional methods..

Proposal 1: UE location for the purpose of ground truth shall be determined using secondary sources (e.g. camera-based), which different from GNSS or RAT dependent methods for UEs that are not provisioned as PRUs (“non-PRU UEs”).

Proposal 2: RAN2 shall discuss association between channel measurement and ground truth for each of the positioning use case, including indication of the method used to obtain the ground truth label.

# LCM for UE-based positioning with UE-side model, direct AI/ML positioning (Case 1)

## Ground truth label

RAN1 agreed in RAN1#116bis the following regarding data collection, separating the data collection into two components – Part A (which pertains to radio measurements) and Part B (which pertains to ground truth).

RAN1 agreements:

For training data collection of AI/ML based positioning, the collected data sample can include the following components:

Part A:

•	channel measurement

•	quality indicator of channel measurement

•	time stamp of channel measurement

Part B:

•	ground truth label (or its approximation)

•	quality indicator of label

•	time stamp of label

Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification.

Note: contents in Part A and Part B may or may not be generated by different entities.

Note: Part A and/or Part B, and their contents may or may not apply for each case

FFS: detailed definition of channel measurement

For Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning), this means that:

- Training is based on channel measurements and UE position (label) both generated by the UE.

- Training is based on channel measurements generated by the UE and the UE position (label) generated by the LMF.

Both training an AI/ML model and monitoring the performance of the AI/ML model requires high quality ground truth labels associated with it.

Especially in indoor scenarios, it is challenging to get accurate ground truth labels, because GNSS does not work indoors, and RAT dependent and RAT-independent methods currently specified cannot deliver high quality labels in scenarios dominated by heavy multipath.

The location of a UE is well known when it is commissioned as a PRU in a fixed position. However, this restricts training and monitoring data to a subset of stationary UEs deployed at specific locations.

Observation 2: Use of PRU type UEs limit the data collected to static UEs and spatially limited to where the PRUs are provisioned.

Use of non-PRU type UEs enable data collection with variety of UE speeds and UE locations.

As described above, the UE position obtained by UE-based or LMF-based methods may be available for a non-PRU UE. However, the labels thus obtained may not necessarily be reliable. Therefore, it may be beneficial to utilize other sensor information available at the UE.

Supporting landmarks in the 5G AI/ML framework can be crucial for enabling data collection and monitoring, particularly in indoor scenarios where landmarks can be the only mechanism to obtain labeled data. In this context, these landmarks enable determination of true location of UE by alternative means which are not affected by radio propagation and thus provide high quality labels that can be associated with specific SRS transmissions or PRS measurements.

Figure 1- A landmark at Frankfurt Airport's wall can serve as a reference point for such DL PRS data collection or monitoring.

Observation 3: In indoor scenarios, landmarks can be the only mechanism to obtain labeled data, enabling the Network/UE to identify and trigger the need for labels in specific areas.

Based on UE capabilities, the UE may be equipped with additional sensors, such as Lidar, camera, or radar sensors. The UE may be configured to search for certain features containing the ground truth labels, such as landmarks, which may be QR codes, visible markers, orientation markers and so on. For example, an autonomous guided vehicle in an indoor factory scenario may be configured to detect QR codes or NFC tags as it moves along a path. Detection of such information could provide a ground truth label in such scenario.

An example of enhancement in ProvideCapabilities is depicted below:

-- ASN1START

NR-AIML-ProvideCapabilities-r19 ::= SEQUENCE {

nr-AIML-Mode-r19						AIML-PositioningModes,

nr-GTL-Support-r19						BIT STRING {	UEbasedRATDepPos (0),

GNSS		     (1),

UEB-Hybrid		 (2),

QRCodeScan		 (3),

RFID-Detection	 (4),

BT-Beacons		 (5),

LandmarkDetection(7)

}	(SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

Observation 4: UEs may be equipped with additional sensors (such as camera and radars) and may have applications that are able to detect and provide or enhance ground truth labels.

Additionally, the detection accuracy of landmarks can benefit from device sensors and user usage patterns. The network can identify or indicate areas where labeled data is needed, and trigger action associated with these landmarks accordingly.

Proposal 3: RAN2 shall discuss what additional information from the UE can be provided to the LMF or OTT server which trains the UE-sided model to ascertain the ground truth. The following information could be transferred from the UE to obtain ground truth labels:

      - Information obtained from on-board sensors, such as camera and radars

      - Information obtained from processing tags, such as QR codes, NFC tags

      - Information obtained from other positioning systems deployed in parallel to 3GPP system.

Although a UE may have capability to acquire ground truth using alternative means, blind search for such information may be computationally prohibitive at the UE. The network may provide assistance data corresponding to the deployment scenario to help UE acquire such ground truth labels.

Proposal 4: Based on UE capabilities, the network may provide assistance data to the UE to help determine GTL using alternative means or the network may provide ground truth labels to the UE if it has accurate GTL available at NW side.

An example of update to ProvideAssistanceData to acquire ground truth labels could be:

-- ASN1START

NR-AIML-ProvideAssistanceData-r19::= SEQUENCE {

nr-DL-PRS-AssistanceData-r19		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r19		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r19

NR-PositionCalculationAssistance-r16

OPTIONAL,	--

assistanceDataValidityArea-r17	AreaID-CellList-r17					OPTIONAL	-- Need ON

]],

[[

nr-PeriodicGTL-r19		NR-PeriodicGTL-r19		OPTIONAL	-- Cond GTLMonitoring

ad-GTLQR-r19			AD-GTL-QR-r19			OPTIONAL	-- Need ON

ad-GTRFID-r19			AD-GTL-RFID-r19			OPTIONAL	-- Need ON

ad-GTAIoT-r19			AD-GTL-AIoT-r19			OPTIONAL	-- Need ON

ad-Landmark-r19			AD-GTL-Landmark-r19		OPTIONAL	-- Need ON

ad-BTBeacons-r19		AD-GTL-BTBeacons-r19	OPTIONAL	-- Need ON

]]

}

As an example, the UE may be provided information about the QR code as follows:

AD-GTL-QR-r19::= SEQUENCE {

qr-Identifier				    INTEGER(0:65535),

qr-locationCoordinates-r19		LocationCoordinates,   OPTIONAL     -- Need ON

qr-checksum-r19					OCTET STRING	       OPTIONAL     -- Need ON

}

The network provides assistance data to the UE containing information about the QR code tags that are deployed by the network in the area to enable data collection. This ensures that the QR codes detected by the UE are indeed the valid QR codes to be used as ground truth label for UE position.

Similar assistance data can be provided to the UE to enable the UE to detect landmarks (e.g. using camera-based object detection).

Proposal 5: If the UE needs assistance from NW to perform monitoring, then the UE shall send a request to NW, indicating the type of assistance data needed.

This may be indicated in the RequestAssistanceData.

-- ASN1START

NR-AIML-RequestAssistanceData-r16 ::= SEQUENCE {

nr-AdType-r16					BIT STRING {	dl-prs	   (0),

posCalc    (1),

adUEBGTL   (2),

adNWBGTL   (3)

} (SIZE (1..8)),

...,

[[

nr-GTLAssistanceRequest-r17	    BIT STRING {	UEbasedRATDepPos (0),

QRCodeScan		 (1),

RFID-Detection	 (2),

BT-Beacons		 (3),

LandmarkDetection(4)

}	(SIZE (1..8))		-- Cond adUEBGTL,

-- ASN1STOP

# LCM for NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning (Case 3b)

For the Case 3b, ground truth from the UE to enable the model monitoring and management functionalities need to be supported. Other LCM components for NW-sided model are outside the scope of RAN2 specifications.

## Ground truth label

For Case 3b (NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning), this means that:

- Training is based on channel measurements provided by the NG-RAN node and the label generated by the LMF.

- Training is based on channel measurements provided by the NG-RAN node and the label provided by the UE (PRU or non-PRU).

In both cases, the training data and label are generated by different entities.

Observation 5: Training data and labels are generated by different entities for Case 3b, thereby requiring alignment between training data and the label.

However, UE-based and LMF-based positioning methods continue to suffer from reduced accuracy in environments with significant multipath and frequent NLOS conditions. These are exactly the scenarios where classical positioning methods show performance degradation. Therefore, it may be beneficial to utilize other sensor information available at the UE.

As discussed earlier for Case 1, the UE shall provide additional information to provide ground truth labels to the NW.

Proposal 6: UE shall report its capability to provide ground truth labels or its ability to provide additional information to improve the ground truth labels to the NW for LCM of NW-based AI/ML models.

Proposal 7:  The UE shall provide ground truth labels or additional information to improve the ground truth labels, when configured by the network.

Observation 6: The LMF may need to time-synchronize the input and ground truth label for training and monitoring purposes.

Proposal 8: The UE shall provide a timestamp synchronized with the network time to ensure proper alignment between ground truth labels and network measurements.

Proposal 9: The UE shall, subject to its capabilities and privacy, be configured to report its ground truth (i.e., UE position at one or more point in time), together with uncertainties to the LMF for assisting training for Case 3a.

# Conclusions

In this contribution, we have made the following observations:

Observation 1: Legacy 3GPP positioning methods provide inaccurate labels in training data, particularly in the NLOS-heavy and multipath-dominated scenarios where AI/ML is expected to outperform traditional methods.

Observation 2: Use of PRU type UEs limit the data collected to static UEs and spatially limited to where the PRUs are provisioned.

Observation 3: In indoor scenarios, landmarks can be the only mechanism to obtain labeled data, enabling the Network/UE to identify and trigger the need for labels in specific areas.

Observation 4: UEs may be equipped with additional sensors (such as camera and radars) and may have applications that are able to detect and provide or enhance ground truth labels.

Observation 5: Training data and labels are generated by different entities for Case 3b, thereby requiring alignment between training data and the label.

Observation 6: The LMF may need to time-synchronize the input and ground truth label for training and monitoring purposes.

Proposal 1: UE location for the purpose of ground truth shall be determined using secondary sources (e.g. camera-based), which different from GNSS or RAT dependent methods for UEs that are not provisioned as PRUs (“non-PRU UEs”).

Proposal 2: RAN2 shall discuss association between channel measurement and ground truth for each of the positioning use case, including indication of the method used to obtain the ground truth label.

Proposal 3: RAN2 shall discuss what additional information from the UE can be provided to the LMF or OTT server which trains the UE-sided model to ascertain the ground truth. The following information could be transferred from the UE to obtain ground truth labels:

      - Information obtained from on-board sensors, such as camera and radars

      - Information obtained from processing tags, such as QR codes, NFC tags

      - Information obtained from other positioning systems deployed in parallel to 3GPP system.

Proposal 4: Based on UE capabilities, the network may provide assistance data to the UE to help determine GTL using alternative means or the network may provide ground truth labels to the UE if it has accurate GTL available at NW side.

Proposal 5: If the UE needs assistance from NW to perform monitoring, then the UE shall send a request to NW, indicating the type of assistance data needed.

Proposal 6: UE shall report its capability to provide ground truth labels or its ability to provide additional information to improve the ground truth labels to the NW for LCM of NW-based AI/ML models.

Proposal 7:  The UE shall provide ground truth labels or additional information to improve the ground truth labels, when configured by the network.

Observation 7: The LMF may need to time-synchronize the input and ground truth label for training and monitoring purposes.

Proposal 8: The UE shall provide a timestamp synchronized with the network time to ensure proper alignment between ground truth labels and network measurements.

Proposal 9: The UE shall, subject to its capabilities and privacy, be configured to report its ground truth (i.e., UE position at one or more point in time), together with uncertainties to the LMF for assisting training for Case 3a.

# References

| [1] | RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," 3GPP, Online, 2023-12. |
| --- | --- |