3GPP TSG-RAN WG2 Meeting #130	R2-2503387

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.13.3

Source:	NEC

Title:	Remaining Issues of Control Plane aspects

Document for:	Discussion, Decision

# Introduction

In RAN2#129bis meeting, the following agreements were reached for the control plane design and the support of fast RRC message forwarding at intermediate relay for Multi-hop Relay.

Agreements:

Control plane design considers only approach 1 (along with the fast forwarding enhancement for SRB0 if agreed), and approach 2 is not further pursued in Rel-19.

Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1.

TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged).

Other approaches are not precluded (contribution-driven) but should be shown at a mature stage considering the time left.

Strive to avoid additional RAN3 impact specific to fast forwarding.

FFS if applicable to DL.

FFS what level of gNB awareness of the path information would be needed.

FFS if fast forwarding is optional/mandatory for UEs to support.

This paper discusses the open issues for fast RRC message forwarding. In addition, this paper also discusses the remaining issues for SI forwarding following the corresponding email discussion ([Post129bis][412][Relay]).

# Discussion

## Fast/parallel SRB0 message forwarding

In RAN2#129bis, there was a discussion if the Fast/parallel SRB0 message forwarding only applies to UL SRB0 or applies to both UL and DL SRB0 message.

In our understanding, if UL SRB0 Fast/parallel forwarding is supported, the DL SRB0 message Fast/parallel forwarding may come almost for free. In addition, the support of DL SRB0 message Fast/parallel forwarding can help to reduce the delay of connection setup for Remote UE. The reason is without the support of DL SRB0 message Fast/parallel forwarding, the RRCSetup message can not be sent back to Remote UE before the Relay UEs enter into RRC_CONNECTED along the relaying path.

From specification impact perspective, we see minimum specification change needed to support DL SRB0 message Fast/parallel forwarding on top of the support UL SRB0 Fast/parallel forwarding.

Proposal 1: Fast/parallel SRB0 message forwarding is supported for both UL SRB0 message and DL SRB0 message.

## Remaining issues of SI acquisition for Remote UE

During RAN2#129 meeting, lots of agreements reached for the system information acquisition for Remote UE for the case of multiple hop based sidelink relay. However there is one FFS as below:

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

During the email discussion ([Post129bis][412][Relay]), the need of including a “remote UE ID” in the SI request/response message (i.e., within the RemoteUEInformationSidelink and UuMessageTransferSidelink) was also discussed.

It should be noted that the first Relay UE receiving the on demand SIB request cannot request the SIB transmission from the network by itself. The first Relay UE needs to forward the on demand SIB request to its upstream relay node, and the upstream relay node needs to keep forwarding the on demand SIB request until reaching the last Relay UE for the Remote UE. The forwarding is not based on SRAP protocol, but on the PC5 RRC message. In case of one hop based forwarding for SI request (i.e., Rel-17), it does not need to indicate the source of the SIB request message, since the Relay UE will keep the record of Remote UE, and in case of SI response, Relay UE will forward the SI message to the Remote UE following its record. However, in case of multiple hop based forwarding(i.e., Rel-19), for SI response, the Relay UE along the relay path may need to know the destination of such message to make correct message forwarding over PC5 RRC. The destination can be expressed by the identity of the Remote UE. This means during SI request, the Remote UE ID needs to be populated along the relay path until the last Relay UE.

In addition, the intermediate Relay UE may need to record the routing information for the Remote UE, since the intermediate Relay UE may need to rely on this routing information to forward back the request SIBs when it receives the request SIBs from its parent Relay UE.

Proposal 2:	The source of on demand SIB request message (i.e., Remote UE ID) is included in the on demand SIB request message over PC5 RRC (i.e., within the RemoteUEInformationSidelink) along the relay path until the last Relay UE.

Proposal 3:	The destination of requested SIB message (i.e., Remote UE ID) is included in the on forwarded SIB message over PC5 RRC (i.e., within the UuMessageTransferSidelink) along the relay path until the first Relay UE.

During RAN2#129 meeting, for the system information acquisition for Remote UE for the case of multiple hop based sidelink relay, there is another FFS as below:

The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

The email discussion ([Post129bis][412][Relay]) only discussed this issue for RRC Idle/Innactive state Relay UE, but we think the discussion should extend to RRC Connected Relay UE also. There may be Relay UE(s) in the relaying path that may temporally store a valid version of the SIB(s) requested by the Remote UE. We think that allowing the intermediate Relay UE to provide the available SIBs to Remote UE per its request can reduce signaling overhead over the air and also reduce the relay for the Remote UE to acquire the SIBs. But if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE, it may not terminate the SIB Request from Remote UE and continue to further forward the SIB request message to its parent Relay UE.

Proposal 4:	The intermediate Relay UE is allowed to provide the request SIBs to the Remote UE if there is a valid version of the request SIBs.

Proposal 5:	The intermediate Relay UE continues to further forward the SIB request message to its parent Relay UE if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE.

# Conclusion and Proposal

We have the following proposals:

Proposal 1: Fast/parallel SRB0 message forwarding is supported for both UL SRB0 message and DL SRB0 message.

Proposal 2:	The source of on demand SIB request message (i.e., Remote UE ID) is included in the on demand SIB request message over PC5 RRC (i.e., within the RemoteUEInformationSidelink) along the relay path until the last Relay UE.

Proposal 3:	The destination of requested SIB message (i.e., Remote UE ID) is included in the on forwarded SIB message over PC5 RRC (i.e., within the UuMessageTransferSidelink) along the relay path until the first Relay UE.

Proposal 4:	The intermediate Relay UE is allowed to provide the request SIBs to the Remote UE if there is a valid version of the request SIBs.

Proposal 5:	The intermediate Relay UE continues to further forward the SIB request message to its parent Relay UE if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE.