3GPP TSG-RAN WG2 Meeting #130	R2-2503390

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.18.2

Source:	NEC

Title:	Configuration to support LTE-based 5G Broadcast

Document for:	Discussion, Decision

# Introduction

The work item LTE-based 5G Broadcast Phase 2 was approved in RP-250794.

This paper intends to discuss how to support the configuration for time interleaved transmission, following the recent RAN1 progress.

# Discussion

## Configuration for time interleaved transmission

In the last RAN1 meeting, the following agreement was reached for the time interleaver configuration:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

- Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

- Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

In our understanding, a configuration signalling through MSI MAC CE is not needed, since the TFI configuration for MBMS dedicated cell is not expected to change very often. The Time interleaver configuration can be seen as a semi-static attribute of MBSFN transmission at physical layer. Hence we think it is better to use RRC signalling for such configuration.

Proposal 1: RRC signalling is employed for configuring time interleaved transmission (i.e., the values of M and N).

In addition, according to RAN1 discussion, the configuration for time interleaved transmission can be either configured per PMCH configuration or per MBS session. In the current design, one PMCH configuration may host multiple MBS sessions. If the configuration is performed per PMCH configuration, all MBMS sessions in PMCH configuration need to share the same parameters for time interleaving, which will limit the flexibility of  the configuration for different MBMS sessions. In practice, different MBMS sessions may have different requirement for time interleaved transmission. Hence we support to enable per MBS session based configuration for time interleaved transmission.

Proposal 2: Per MBMS session is supported for configuring time interleaved transmission (i.e., the values of M and N).

# Conclusion and Proposal

We have the following proposals:

Proposal 1: RRC signalling is employed for configuring time interleaved transmission (i.e., the values of M and N).

Proposal 2: Per MBMS session is supported for configuring time interleaved transmission (i.e., the values of M and N).