3GPP TSG-RAN WG2 Meeting #130					R2-2503393

St.Julians, Malta, May 19th – 23rd , 2025

Agenda item:	8.5.4

Source:	Samsung

Title:	Adaptation of common signal transmissions

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the RAN2 aspects of Paging and PRACH adaptation in time domain.

# Discussion

## 2.1. Paging adaptation

RAN2 has agreed that to configure the following new parameters for paging adaptation,

- N’ and Ns’ for indicating number of paging frames and number of POs for paging adaptation.

- PF_Offset’ for flexible allocation of PF for paging adaptation.

In legacy firstPDCCH-MonitoringOccasionOfPO is optionally signalled to flexibly allocate the PDCCH monitoring occasions for each PO. It indicates the starting PDCCH monitoring occasion number for each PO.

Since the number of POs are separately signalled for paging adaptation, firstPDCCH-MonitoringOccasionOfPO’ should also be separately signalled for paging adaptation. As in legacy, firstPDCCH-MonitoringOccasionOfPO’ for paging adaptation should be optionally signalled.

Proposal 1: If cell supports paging adaptation,

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is configured separately for paging adaptation.

- firstPDCCH-MonitoringOccasionOfPO’ for paging adaptation is optional.

- if firstPDCCH-MonitoringOccasionOfPO’ for paging adaptation is configured, PO for paging adaptation is determined based on starting PDCCH monitoring occasion number, otherwise not.

## 2.2. PRACH adaptation in time domain

## 2.2.1 Configuration aspects

For network energy savings, legacy RACH resources can be configured sparsely and additional RACH resources can be configured dynamically as and when needed. RAN1 has discussed configuration of additional RACH resources in time and frequency domain and agreed the following:

RAN1#118b Agreements

- For configuration of additional PRACH resources, support both of the following:

  - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

  - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

RAN1#119 Agreements

- Frequency domain configuration:

  - At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

RAN2#120bis Agreements

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

  - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Discussion

Based on the above agreements,  following configuration are allowed for random access configuration/random access resource set

- Separate PRACH configuration index for the additional PRACH resources and legacy RACH resources + msg1-FrequencyStart is same for legacy RACH resources and additional PRACH resources

- Separate PRACH configuration index for the additional PRACH resources and legacy RACH resources + separate msg1-FrequencyStart for the legacy RACH resources and additional PRACH resources

- Same PRACH configuration index for the additional PRACH resources and legacy RACH resources + msg1-FrequencyStart is same for legacy RACH resources and additional PRACH resources

- Same PRACH configuration index for the additional PRACH resources and legacy RACH resources + separate msg1-FrequencyStart for the legacy RACH resources and additional PRACH resources

prach-ConfigurationIndex and msg1-FrequencyStart are signalled in RACH-ConfigGeneric IE. So, it’s straightforward to include additional RACH resources configuration in RACH-ConfigGeneric IE. An example is shown below.

An alternate would be to signal a seperate RACH-ConfigCommon for additional RACH resources. However, this is unnecessary overhead as it would lead to duplicate signalling of several random access parameters. This would also lead to configuration of several RACH-ConfigCommon for additional ROs as we have agreed that additional ROs can be applicable for feature and feature combinations.

RACH-ConfigGeneric information element

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

...,

[[

prach-ConfigurationPeriodScaling-IAB-r16    ENUMERATED {scf1,scf2,scf4,scf8,scf16,scf32,scf64}              OPTIONAL,   -- Need R

prach-ConfigurationFrameOffset-IAB-r16      INTEGER (0..63)                                                 OPTIONAL,   -- Need R

prach-ConfigurationSOffset-IAB-r16          INTEGER (0..39)                                                 OPTIONAL,   -- Need R

ra-ResponseWindow-v1610                     ENUMERATED { sl60, sl160}                                       OPTIONAL,   -- Need R

prach-ConfigurationIndex-v1610              INTEGER (256..262)                                              OPTIONAL    -- Need R

]],

[[

ra-ResponseWindow-v1700                     ENUMERATED {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560} OPTIONAL    -- Need R

]],

[[

additionalROConfig			 SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255), OPTIONAL    -- Need R

msg1-FrequencyStart    INTEGER (0..maxNrofPhysicalResourceBlocks-1), OPTIONAL    -- Need R

// other parameters (such as mask index and a value of K (number of association pattern periods), if any

} OPTIONAL    -- Need R

]]

}

Proposal 2: Introduce prach-ConfigurationIndex, msg1-FrequencyStart, mask index and a value of K (number of association pattern periods) for additional rach resources in RACH-ConfigGeneric IE.

PRACH adpatation in time domain for NUL or SUL or both

RAN1 is discussing a general design for PRACH adpatation in time domain. There is no difference in random access configuration for SUL and NUL. So there is no reason to exclude PRACH adpatation in time domain for either carrier. Network can configure additional RACH resources in random access resource configuration of SUL or random access resource configuration of NUL or both.

Proposal 3: PRACH adpatation in time domain is supported for both NUL and SUL.

## 2.2.2 RACH adaptation activation in RRC_CONNECTED

RAN1 has agreed that for DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. This DCI is paging DCI and transmitted in PO and addressed to P-RNTI.

Inorder to receive this, UE needs to monitor PO in RRC_CONNECTED state. However, PO is monitored only if paging search space is configured in active DL BWP. In case paging search space is not configured in active DL BWP, UE cannot receive the DCI format 1_0. So other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources may be needed.

Proposal 4: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

If paging search space is configured in active DL BWP, UE can monitor PO in RRC_CONNECTED for DCI-based adaptation for additional PRACH resources. Currently, UE monitors any PO at least once every modification period for SI change notification and UE monitors any PO at least once every defaultPagingCycle for emergency notification. Upon receiving the paging adaptation indication, UE applies it from current modification period. It would be beneficial for UE to receive the paging adaptation indication as early as possible. So, monitoring PO every defaultPagingCycle seems better than monitoring PO every modification period.

Proposal 5: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle.

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period.

## 2.2.3 FDMed ROs

RAN1 Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Given the RAN1 agreement already limited the FDMed valid RO is no more than 8, e.g., msg1_FDM_legacy + msg1_FDM_additional <=8, thus the calculation of RA-RNTI only needs to ensure the starting is counting from the first RO from legacy RO then to the additional RO. By doing this, there is no impact on the RA-RNTI calculation and also no impact on the legacy UE calculating the RA-RNTI. As shown in following figure, no matter what is the relative position of legacy RO and additional RO in F domain, if we start legacy first, their f_id can be kept as aligned with legacy without changing the calculation methods.

Proposal 6: the f_id in the RA-RNTI calculation for additional RO is starting from legacy FDMed RO to additionally FDMed RO, i.e., f_id for additional NES PRACH is

- From 0 to msg1_FDMed_additional if there is no FDMed legacy RO

- From msg1_FDMed_legacy to msg1_FDMed_legacy + msg1_FDMed_additional if there is some FDMed legacy RO.

# Conclusion

Proposal 1: If cell supports paging adaptation,

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is configured separately for paging adaptation.

- firstPDCCH-MonitoringOccasionOfPO’ for paging adaptation is optional.

- if firstPDCCH-MonitoringOccasionOfPO’ for paging adaptation is configured, PO for paging adaptation is determined based on starting PDCCH monitoring occasion number, otherwise not.

Proposal 2: Introduce prach-ConfigurationIndex, msg1-FrequencyStart, mask index and a value of K (number of association pattern periods) for additional rach resources in RACH-ConfigGeneric IE.

Proposal 3: PRACH adpatation in time domain is supported for both NUL and SUL.

Proposal 4: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that  paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

Proposal 5: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period

Proposal 6: the f_id in the RA-RNTI calculation for additional RO is starting from legacy FDMed RO to additionally FDMed RO, i.e., f_id for additional NES PRACH is

- From 0 to msg1_FDMed_additional if there is no FDMed legacy RO

- From msg1_FDMed_legacy to msg1_FDMed_legacy + msg1_FDMed_additional if there is FDMed legacy RO.

# References