3GPP TSG RAN WG2 Meeting #130            	                      R2-2503418

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.10.3

Source:	Xiaomi

Title:	Discussion on other aspects of Rel-19 SON/MDT

Document for:	Discussion and Decision

Introduction

RAN3#127bis meeting made some progress on SON/MDT for NTN and sent and LS to RAN2 in [1]. The main issues that involved with RAN2 are listed as follows:

RAN3 discussed the MRO enhancement for location-based CHO in NTN for earth moving scenario. In order to optimize the distance threshold for location-based CHO, RAN3 thinks it is beneficial to support either option provided below. - Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens. RAN3 would like to check with RAN2 which option to be adopted in the UE RLF report.

This document starts the discussion on MRO for NTN based on RAN3’s LS, and discuss how to support TA list logging for the support of MRO for NTN additionally.

Discussion

SON for NTN

Discussion on RAN3’s LS

Based on RAN3’s LS, Option 1’s advantage lies in that the network can utilize its own algorithms and multiple data sources (such as satellite ephemeris, precise network - side location information, etc.) for comprehensive and accurate distance calculation. Option 2, which introduces the measured distance from the UE to the moving reference location of the serving cell and the candidate cell when RLF/HOF happens, has the advantage of simplifying the network’s workload as it eliminates the need for the network to perform distance calculations. When comparing the overhead when capturing option 1 or capturing option 2, ASN.1 of UTC time and distance information are listed as below for references.

timeInfoUTC INTEGER (0..549755813887) costs 39 bits. distanceThresh-r17 INTEGER(0..65525) costs 16 bits.

Therefore, in order to store and transfer UTC time in RLF, 39 bits of overhead is required, and 2 distance information stored (measured distance from the UE to the moving reference location of the serving cell and the candidate cell) only costs 16bits*2=32bits.Obviously option 2 has less overhead. So, it’s proposed as below:

Proposal 1: RAN2 to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell in RLF report when RLF/HOF happens, based on which the network computes the distance.

Discussion on TA list of NTN

In Rel-17, NTN introduces trackingAreaList, where a cell can be configured with multiple tracking areas for earth moving cell. However, in current SON reports, the CGI information only include one TAC for logging in TN (listed as below), which cannot fulfill the requirement of MRO for NTN. Therefore, the CGI logging information should be extended to report trackingAreaList to support MRO for NTN.

CGI-Info-Logging-r16 ::=     SEQUENCE {

plmn-Identity-r16                    PLMN-Identity,

cellIdentity-r16                     CellIdentity,

trackingAreaCode-r16                 TrackingAreaCode               OPTIONAL

}

Consider the situation that CGI-Info-Logging-r16 has been used everywhere for CGI information report, and its field is not extendable. It’s suggested to introduce a new CGI-Info-Logging-r19 IE to support trackingAreaList logging in CGI information.

Proposal 2: RAN2 to define a new CGI-Info-Logging-r19 IE to log trackingAreaList information for the support of MRO for NTN.

MDT for NTN

MDT for NTN

In the previous meeting, there is an FFS left that “FFS if, when the UE is in IDLE/INACTIVE, the UE logs or not if the UE cannot obtain its location”. Companies proposed two options for this issue. Option 1 is to continue MDT logging without checking area scope information. Option 2 is not performing MDT logging while no location is obtained by UE itself. Due to that RAN2 has agreed only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT. Thus, no location information obtained means no area scope checking, and the SON reports logged for NTN may be unreliable or unneeded for NW, thus it’s not needed to continue the MDT logging while no location is obtained by UE itself.

Proposal 3: When the UE is in IDLE/INACTIVE and the UE cannot obtain its location, the UE stops MDT logging for NTN.

Conclusion

In this contribution, we propose:

Proposal 1: RAN2 to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell in RLF report when RLF/HOF happens, based on which the network computes the distance.

Proposal 2: RAN2 to define a new CGI-Info-Logging-r19 IE to log trackingAreaList information for the support of MRO for NTN.

Proposal 3: When the UE is in IDLE/INACTIVE and the UE cannot obtain its location, the UE stops MDT logging for NTN.

Reference

[1] R2-2503317 LS on SONMDT for NTN (R3-252382; contact: CMCC)