3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503425

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on RRM Measurement Gaps/Restrictions Related Enhancements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, the following agreements were achieved on RRM measurement gaps/restrictions related enhancements.

Agreements on RRM measurement gaps impacts 1. From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion. 2. “Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC.RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. 3. RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1.

In this contribution, we give our further considerations on other open issues.

Discussion

[RRC-5,RRC-6]UE assistant information

In RAN4#114bis meeting[1], RAN4 discussed UE assistance information for enabling TX/RX during RRM measurements. And the following agreement was made:

Agreement Regarding UE assistance information, the following is agreed: - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

The remaining issue is when to trigger UAI. Since the channel quality change can impact measurement performance, it’s reasonable to trigger the skipped MG UAI based on the changes of channel quality. One way is to follow measurement report triggering, that UE checks the changes of channel quality since previous UAI report, if it’s above a threshold, new UAI is triggered.

Observation 1: One way is to follow measurement report triggering, that UE checks the changes of channel quality since previous UAI report, if it’s above a threshold, new UAI is triggered.

Proposal 1: (RRC-5) UE triggers skipped MG UAI based on the changes of channel quality.

Besides the event triggering, periodic triggering can also be considered with some enhancements. For example, UE can decide to report if current ratio is different from the previous one and not to report if current ratio is the same with the previous one. And from the network side, the network can indicate to UE whether current ratio can fulfill the delay critical data transmission requirement. After UE receives the indication, UE can decide whether to report this UAI when the ratio changes. For example, the network can evaluate buffered delay critical data status and how many skipped MGs is needed to transmit these data, if current ratio can fulfill the requirement, the network can indicate it to UE. Then for the UE side, if the new calculate ratio is above the network indicated ratio, UE can decide not to report this UAI when even the ratio changes.

Proposal 2: (RRC-5) Once the skipped MG UAI is triggered, UE can skip the MG UAI report if the ratio does not change compared to the previous report ratio.

Proposal 3: (RRC-5) The network can indicate to UE what ratio can fulfill the delay critical data transmission requirement. And UE can decide whether to report this UAI when the ratio changes.

The network can configure UE to enable/disable the skipped MG UAI report as network requirement.

Proposal 4: (RRC-6) The network configures UE to enable/disable the skipped MG UAI report.

To avoid frequency reporting, a new Timer should be introduced following the legacy UAI procedure.

Proposal 5: (RRC-5) Define a new prohibit timer dedicated to skipped MG UAI.

Conclusion

According to the analysis in section 2, the following observations and proposals were made:

Observation 1: One way is to follow measurement report triggering, that UE checks the changes of channel quality since previous UAI report, if it’s above a threshold, new UAI is triggered.

Proposal 1: (RRC-5) UE triggers skipped MG UAI based on the changes of channel quality.

Proposal 2: (RRC-5) Once the skipped MG UAI is triggered, UE can skip the MG UAI report if the ratio does not change compared to the previous report ratio.

Proposal 3: (RRC-5) The network can indicate to UE what ratio can fulfill the delay critical data transmission requirement. And UE can decide whether to report this UAI when the ratio changes.

Proposal 4: (RRC-6) The network configures UE to enable/disable the skipped MG UAI report.

Proposal 5: (RRC-5) Define a new prohibit timer dedicated to skipped MG UAI.

Reference

- R4-2504911, WF on RRM requirements for XR_Ph3，RAN4#114bis