3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503429

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on XR Rate Control

Agenda Item:	8.7.6

Document for:	Discussion and Decision

Introduction

In RAN2#129bis meeting, regarding to XR rate control, the below agreements were agreed [1]:

Rate indication details:

Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

We specify a single table.

As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered.

Rate query:

The working assumption is changed to agreements on rate query MAC CE.

MAC CE granularity:

From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this.

FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Companies to check with their SA4 colleagues whether there are any issues with this

In the current contribution, we further discuss the below four key leftover issues on XR rate control:

- How QFI is indicated.

- Single or multiple flow in MAC CE.

- New MAC CE format.

- Dual-connectivity issue.

Discussion

[MAC-04] How QFI is indicated

For the solution to introduce per-QoS flow granularity in the recommended bitrate MAC CE in Rel-19, how to indicate per QoS flow level recommended bit rate needs to be further discussed. Since the QoS flow ID is not unique within the UE, the PDU session ID should be considered to avoid the ambiguity. In the legacy Recommended bit rate MAC CE, “LCID” field is used to identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits. To identify specific QoS flow in the new MAC CE, besides the QoS flow ID, the PDU session ID should also be included. According to the RRC specification, the PDU session ID will cost 8 bits (0-255), the QFI will cost 6 bits (0-63), then the whole overhead to indicate the QoS flow in the MAC CE will cost 14 bits. We can use it as baseline, and further discuss whether any optimize the overhead is needed. For example, to combine the QoS flow ID and the PDU session ID and indicate the combination using a few bits.

Proposal 1: (MAC-04) RAN2 uses the PDU session ID and QoS Flow ID to indicate per QoS flow level recommended bit rate in Rel-19 as baseline.

Proposal 2: (MAC-04) RAN2 further discusses whether any optimize for the overhead is needed.

[MAC-05] Single or multiple flow in MAC CE

For the legacy recommendation bit rate MAC CE, there is no flexibility size supported. There are proponent company that support multiple flow/DRB in the new introduced bit rate MAC CE. For Rel-19 XR service, multiple traffic flows may experience congestion simultaneously. If a single-flow indication in one MAC CE is adopted, the network would need to send multiple MAC CEs to the UE simultaneously, which is inefficient for XR services. Additionally, XR traffic flows exhibit significant differences in data rate requirements, meaning not all flows require rate control. Based on this analysis, one rate control MAC CE can encompass multiple flows. The network can also configure the maximum number of QoS flows per MAC CE via RRC. From our point of view, the difference between single or multiple flow in MAC CE seems not so critical issue. At least from resource consuming point of view, there is no difference. Hence, we prefer not to introduce multiple flow format in the current release.

Proposal 3: (MAC-05) RAN2 follows the legacy design not to introduce the flexibility MAC CE format in Rel-19 XR.

[MAC-07] New MAC CE format

To better pushforward this issue, the potential enhancement for recommendation bit rate MAC CE is discussed. The Figure 6.1.3.20-1 in 38.321[4] shows the format of recommended bit rate MAC CE. It has a fixed size and consists of two octets.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

The related fields are:

1) “LCID” field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits.

2) “UL/DL” field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink.

3) “Bit Rate” field indicates an index to Table 6.1.3.20-1 in 38.321[4]. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate.

4) “X” is Bit rate multiplier, For UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331.

5) “R” is reserved bit. There are two R bits in the current format which are all set to 0.

Based on the current discussion progress, we propose the below new introduced Recommended bit rate MAC CE for Rel-19 XR: The indication to the related QoS flow in the MAC CE will cost 14 bits, “UL/DL” field will cost 1 bit, “Bit Rate” field will cost 8 bits, no multiplier is needed. To further achieve the byte alignment requirement, 1 “R” reserved bit should be further introduced.

Proposal 4: (MAC-07) RAN2 adapts 3 bytes format for the new Recommended bit rate MAC CE for Rel-19 XR, including 14 bits QoS flow indication, 1 bit direction, 8 bits rate index and 1 reservation.

[MAC-09] Dual-connectivity issue

In dual-connectivity scenarios with split bearers, data from a single DRB or QoS flow is transmitted across multiple cells (e.g., MCG and SCG). If congestion arises in either MCG, SCG, or both, the UE may receive a recommended bit rate MAC CE with an updated bit rate from at least one of the connected cells—particularly when there is no coordination between MCG and SCG. Due to potential differences in congestion levels across cells, the UE may or may not need to adjust the application-layer bit rate. Alternatively, only the MCG might send an updated recommended bit rate MAC CE, consolidating the latest rate recommendations from both cells. From the UE's perspective, relying on a single MAC CE could simplify the decision-making process regarding whether to modify the application bit rate.

Proposal 5: (MAC-09) RAN2 confirms no additional specification impact to support the UE behaviour on handling multiple XR rate control MAC CEs in CA/DC scenario from RAN2 perspective.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: (MAC-04) RAN2 uses the PDU session ID and QoS Flow ID to indicate per QoS flow level recommended bit rate in Rel-19 as baseline.

Proposal 2: (MAC-04) RAN2 further discusses whether any optimize for the overhead is needed.

Proposal 3: (MAC-05) RAN2 follows the legacy design not to introduce the flexibility MAC CE format in Rel-19 XR.

Proposal 4: (MAC-07) RAN2 adapts 3 bytes format for the new Recommended bit rate MAC CE for Rel-19 XR, including 14 bits QoS flow indication, 1 bit direction, 8 bits rate index and 1 reservation.

Proposal 5: (MAC-09) RAN2 confirms no additional specification impact to support the UE behaviour on handling multiple XR rate control MAC CEs in CA/DC scenario from RAN2 perspective.

Reference

- R2-2502985 Report from session on R19 XR	Session chair (Huawei)	report