3GPP TSG RAN WG2#130	R2-2503455

St Julian’s, Malta, 19th - 23rd May 2025

Agenda Item:	8.6.4

Source:	Lekha Wireless Solutions

Title:	Discussion on Conditional Intra CU LTM

Document for:	Discussion, Decision

Introduction

At last meeting, we discussed Conditional intra CU LTM and made the following agreements [1]:

- Agreements on Conditional intra CU LTM #RAN2 129bis: - Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. - C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer. - RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. - For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

This contribution discussed different aspect to support conditional intra CU mobility design.

Discussion on Conditional intra CU LTM

  - 2.1 Beam selection and valid CG for L3 based RACH-less CLTM determination

Observation 1:

In RAN2#129, agreement was reached that for L1-based C-LTM, the UE can select a beam for RACH-less LTM execution if the beam meets execution conditions. For L3-based C-LTM (e.g., triggered by measurement events like A3 or A5), no agreement has been made on how to perform beam selection.

The main challenge is that L3-based execution is based on consolidated cell-level measurements, whereas RACH-less LTM requires beam-level selection including consideration of Configured Grant (CG) resources. For L3-based Conditional LTM, the target beam cannot be reliably determined due to beam consolidation and L3 filtering. Even without such configurations, ambiguity arises when multiple beams exceed the threshold during different sub-periods of the TTT window.

A clear procedure is needed for the UE to select a suitable beam for RACH-less C-LTM when the execution condition is based on L3 measurements. Without such a procedure, the UE may select an unsuitable beam, leading to failed mobility attempts or suboptimal performance.

In RACH-less intra-CU Conditional LTM with Configured Grant (CG), beam selection plays a crucial role in ensuring seamless mobility and low-latency handover. Since the UE does not rely on the conventional Random Access (RACH) procedure, it must accurately determine and switch to the most suitable beam at the target cell. A robust target beam selection mechanism is essential to maintain connection reliability, reduce failures, and optimize network performance. Here few beam selection options are explained below.

Option 1: Threshold-based Beam Selection (Legacy-aligned)

- UE selects a beam within the target cell where SS-RSRP > rsrp-ThresholdSSB and the beam has an associated CG occasion.

- Cons: UE may not find a suitable beam if strongest beams lack CG.

Option 2: Prioritize Beams with CG Resources

- UE selects the strongest available beam among those with configured CG occasions.

- Cons: UE may not select the strongest possible beam.

- Option 3: Reuse Beam From L3 Trigger

- UE reuses the beam which contributed to satisfying the L3 condition (e.g., best beam at the time of event A3/A5 triggering).

- Cons: Selected beam may lack CG or may be outdated.

- Option 4: Network-Assisted Beam Indication

- Serving gNB provides the preferred beam index(es) for CG access during C-LTM preparation.

- Cons: Adds signaling overhead and reduces UE flexibility.

Proposal 1: UE selects a beam within the target cell where SS-RSRP > rsrp-ThresholdSSB and the beam has an associated CG occasion.

Proposal 2: UE selects the strongest available beam among those with configured CG occasions.

Proposal 3: UE reuses the beam which contributed to satisfying the L3 condition (e.g., best beam at the time of event A3/A5 triggering).

Proposal 4: Serving gNB provides the preferred beam index(es) for CG access during C-LTM preparation.

Proposal 5: For L3 based RACH less C-LTM, Proposal 1 as the baseline due to its alignment with existing procedures. Proposal 2 to 4 as enhancements to provide robustness in diverse deployment scenarios.

By implementing this integrated approach, beam selection robustness can be improved, reducing handover failures, enhancing mobility performance, and ensuring seamless connectivity in RACH-less intra-CU Conditional LTM scenarios.

  - 2.2 In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for LTM target cell, while UE couldn’t obtain valid CG resource, whether UE will fall back to RACH-based LTM?

Observation:

Conditions Leading to RACH-less CLTM Fallback

- TA is Valid:

UE has already derived and stored a valid Timing Advance (TA) for the target cell (from prior synchronization or preparation).

- No Valid CG Resource:

UE cannot find a suitable configured grant occasion:

- No CG occasion mapped to the selected beam

- CG resources exhausted or already used

- UE is not allowed to use any CG resource for that SSB or beam

Fallback Behavior: Switch to RACH-based LTM

RACH-less LTM relies on preconfigured, contention-free uplink resources (CG). If these cannot be used, data cannot be sent. To recover, the UE must initiate a random access procedure (contention-based or non-contention-based RACH) to reestablish uplink scheduling and mobility confirmation.

Mechanism:

UE may:

- Report CG failure to gNB if uplink failure detection is configured

- Wait for backup RACH occasion if pre-configured

- Trigger RACH procedure on the target cell using the selected beam

gNB may:

- Detect CG transmission failure (e.g., no HARQ feedback)

- Configure backup RACH for fallback

- Initiate handover control procedure

Proposal 6: If a UE considers RACH-less Conditional LTM as ongoing but cannot obtain a valid CG resource, the UE should fall back to RACH-based LTM to complete the mobility procedure. This ensures service continuity and aligns with the robustness principles of LTM. Timer- or failure-detection-based triggers can govern this fallback.

  - 2.3 How UE select the CG resource if the selected beam is CSI-RS?

Observation: In Conditional Handover (CHO) or Layer 1/2 (L1/L2) triggered mobility, the UE knows the Configured Grant (CG) resource association with CSI-RS beams of candidate cells through explicit RRC configuration from the serving gNB. Here’s how it works:

- RRC Configuration for CG Resources in Candidate Cells

- Before mobility is triggered, the serving gNB provides the UE with:

- A list of candidate target cells (or beams within a multi-TRP scenario).

- CG configurations for each candidate cell (if configured grant-based UL is supported).

- Beam association rules linking CG resources to specific CSI-RS or SSB beams.

This is done via:

- CHOConfig (for Conditional Handover) or

- L1/L2 Mobility Config (for beam-based mobility).

- UE Procedure for Selecting CG Resource During Mobility

- Measure CSI-RS Beams: The UE measures CSI-RS from candidate cells (configured in `measConfig`).

- Evaluate Trigger Conditions: If a CSI-RS beam meets the CHO/L1/L2 trigger condition (e.g., RSRP > threshold), the UE prepares for mobility.

- Corresponding CG Resource:  The UE checks the pre-configured beam-to-CG mapping (from RRC). It activates the CG resource linked to the selected CSI-RS beam.

- Switch UL Transmission: The UE starts using the new CG resource for UL transmission in the target cell.

How CG Resources Are Mapped to CSI-RS Beams

The association between CG resources and CSI-RS beams is configured in one of the following ways:

Option 1: Explicit Beam-to-CG Mapping in RRC

The gNB configures spatialRelationInfo` for each CG resource, linking it to a specific CSI-RS resource ID or SSB index of the target cell.

Example (simplified RRC structure):

"ConfiguredGrantConfig": {

"frequencyDomainAllocation": "...",

"timeDomainAllocation": "...",

"periodicity": "sf20",

"spatialRelationInfo": {

"csi-RS-Index": 3,  // Associated with CSI-RS beam #3 of target cell

"servingCellId": 2  // Candidate target cell ID

}

}

The UE knows that CG resource X must be used when CSI-RS beam Y is selected.

Option 2: TCI State Association

- The gNB configures Transmission Configuration Indicator (TCI) states for UL (for PUSCH with CG).

- Each TCI state points to a CSI-RS or SSB beam of the target cell.

- UE applies the CG resource linked to the active TCI state after beam selection.

Option 3: Implicit Association via UL Beam Correspondence

- If UL/DL beam correspondence is assumed, the UE selects the CG resource whose spatial filter matches the best DL CSI-RS beam.

- No explicit RRC mapping is needed (used in FR2/mmWave deployments).

Proposal 7: The UE knows the CG ↔ CSI-RS beam association for candidate cells because the serving gNB pre-configures these mappings via RRC (e.g., using `spatialRelationInfo` or TCI states). When the UE selects a new beam during mobility, it automatically switches to the corresponding CG resource for seamless UL transmission.

Proposal 8: UE selects the CG resource whose spatial filter matches the best DL CSI-RS beam under the assumption of UL/DL beam correspondence is valid.

  - 2.4 Coexistence between CLTM and (e)RedCap, CovEnh?

Observation: ·

- (e)RedCap: Reduced Capability UEs (including enhanced RedCap) with simplified processing and power constraints, often targeting use cases like industrial sensors, wearables, etc [2].

- CovEnh: Coverage Enhancement procedures, typically using repetitions and relaxed performance metrics to ensure robust communication at the cell edge or for deep indoor UEs.

- C-LTM (Conditional L1/L2 Triggered Mobility): Enables fast mobility based on pre-configured conditions (e.g., measurements) without requiring RACH in certain scenarios.

·  RAN2 previously agreed that inter-CU LTM in Rel-19 can coexist with both CovEnh and (e)RedCap. The concern arises because CLTM involves tight timing, beam selection, and pre-configured CG — which might be affected by RedCap/CovEnh characteristics.

2.4.1 Coexistence Between Rel-19 CLTM and (e)RedCap

Rationale:

- RedCap supports LTM in Rel-17/18 with constraints (e.g., reduced measurements, simplified beam handling).

- CLTM reuses core LTM procedures, with added optimization from conditional triggering.

- Conditional execution based on simplified triggers can benefit RedCap, as it avoids the need for full RACH.

Why Feasible:

- (e)RedCap UEs can still report L1/L3 events.

- Beam selection strategies (e.g., simplified or pre-indicated beams) can be tailored to RedCap constraints.

Proposal 9: CLTM can be applied to (e)RedCap UEs with minimal tailoring, e.g., using fewer measurement events, relaxed filtering, and pre-indicated beams for CG.

2.4.2 Coexistence Between Rel-19 CLTM and CovEnh

Rationale:

- CovEnh procedures already support LTM through repeated SS/PBCH and PDCCH decoding.

- Conditional mobility can reduce failed handovers in coverage-challenged areas.

Why Feasible:

- CLTM could benefit from CovEnh's robustness, e.g., using repeated SS-RSRP measurements for triggering.

- Conditional triggering allows gNB to predefine handover attempts only when quality is acceptable, reducing RACH overload.

Minor Considerations:

- CG resource availability for CovEnh UEs must match their repetition capabilities.

- CG scheduling must respect CovEnh timing (e.g., extended TTIs or repetitions).Coexistence Between Conditional LTM and Rel-19 Inter-CU LTM

Proposal 10:The LTM procedure's flexibility allows adaptation to RedCap’s reduced complexity and CovEnh's repetition-oriented structure. Conditional triggering reduces control overhead, aligns with power/performance limitations, and offers robust handover in weak coverage scenarios.

Proposal 11: Supporting coexistence between Rel-19 Conditional LTM and both (e)RedCap and CovEnh.

- 3 Conclusion

In this contribution, the following proposals are made:

Beam selection and Valid CG for L3 based RACH Less Conditional LTM:

Proposal 1: UE selects a beam within the target cell where SS-RSRP > rsrp-ThresholdSSB and the beam has an associated CG occasion.

Proposal 2: UE selects the strongest available beam among those with configured CG occasions.

Proposal 3: UE reuses the beam which contributed to satisfying the L3 condition (e.g., best beam at the time of event A3/A5 triggering).

Proposal 4: Serving gNB provides the preferred beam index(es) for CG access during C-LTM preparation.

Proposal 5: For L3 based RACH less C-LTM, Proposal 1 as the baseline due to its alignment with existing procedures. Proposal 2 to 4 as enhancements to provide robustness in diverse deployment scenarios.

Fall back mechanism in Conditional LTM:

Proposal 6: If a UE considers RACH-less Conditional LTM as ongoing but cannot obtain a valid CG resource, the UE should fall back to RACH-based LTM to complete the mobility procedure. This ensures service continuity and aligns with the robustness principles of LTM. Timer- or failure-detection-based triggers can govern this fallback.

How UE select the CG resource if the selected beam is CSI-RS?:

Proposal 7: The UE knows the CG ↔ CSI-RS beam association for candidate cells because the serving gNB pre-configures these mappings via RRC (e.g., using `spatialRelationInfo` or TCI states). When the UE selects a new beam during mobility, it automatically switches to the corresponding CG resource for seamless UL transmission.

Proposal 8: UE selects the CG resource whose spatial filter matches the best DL CSI-RS beam under the assumption of UL/DL beam correspondence is valid.

  - Coexistence between CLTM and (e)RedCap, CovEnh:

Proposal 9: CLTM can be applied to (e)RedCap UEs with minimal tailoring, e.g., using fewer measurement events, relaxed filtering, and pre-indicated beams for CG.

Proposal 10:The LTM procedure's flexibility allows adaptation to RedCap’s reduced complexity and CovEnh's repetition-oriented structure. Conditional triggering reduces control overhead, aligns with power/performance limitations, and offers robust handover in weak coverage scenarios.

Proposal 11: Supporting coexistence between Rel-19 Conditional LTM and both (e)RedCap and CovEnh.

- 4 References

- R2-2502981 “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung)

- 38.300, NR and NG-RAN Overall Description; Stage 2, Release 18