3GPP TSG-RAN WG2 Meeting #130	 R2-2503465

St Julian’s, Malta, May 19-23, 2025

Agenda Item:8.3.5

Source: NEC

Title: Discussion on AIML mobility Performance monitoring

Document for: Discussion and Decision

# 1 Introduction

In last RAN2#129b meeting, the following agreements are made:

=> The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects:

- Data collection for model training

- UE capability

- Applicability reporting

- Inference configuration and reporting

- Performance monitoring and management

= > Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

=> The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.  FFS when the UE can perform inference

=> For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.  FFS details

=> For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

=> Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

=> The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

=> For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

FFS whether association ID is required and should provide motivation on per use case bases

=> As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

=> Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

=> Model transfer will not be discussed in this study item.

=> For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In this contribution, we further discuss AI/ML based mobility performance monitoring for both UE and Network sided models.

# 2 Discussion

In last RAN2#129b meeting, RAN2 has agreed that for the UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case. Performance monitoring involves analyzing the metrics for a particular use case and then taking some decision for improving the efficiency of prediction. This involves configuring the UE to perform the prediction using the model at the UE end and then report the metrics to the network. Otherwise, analyzing the metrics by it’s own and taking some decision for improving the efficiency.

In the case of UE-side model-based performance monitoring for different use cases (for example, in measurement event prediction (e.g., A3/A5), the network configures the UE with necessary parameters to perform metric evaluation over a specified time window. The network provides configuration such as Maximum Event Timing Difference (ETDmax) based on frequency range (e.g., 20ms for FR2, 40ms for FR1), Evaluation Time Window (ETW), reporting mode (periodic or trigger-induced), type of event(s) to evaluate, model switching rules, and cluster information (single or multiple gNBs and associated cells). The UE calculates relevant metrics (e.g., F1 score) and performs direct or indirect prediction analysis as instructed, supporting performance tracking and potential model adaptation decisions.

Proposal 1: Network may configure the UE with metric calculation configuration with ETDmax and ETW in the RRC configuration message for performance monitoring.

Proposal 2: Network may configure the reporting type (periodic or event based reporting) of the performance metric for performance monitoring.

Proposal 3: The network may configure some model switching rules (condition for model change) to improve the overall handover performance for AI/ML based mobility management.

# 3 Conclusions

We have the following proposals for performance monitoring of UE and network sided model.

Proposal 1: Network may configure the UE with metric calculation configuration with ETDmax and ETW in the RRC configuration message for performance monitoring.

Proposal 2: Network may configure the reporting type (periodic or event based reporting) of the performance metric for performance monitoring.

Proposal 3: The network may configure some model switching rules (condition for model change) to improve the overall handover performance for AI/ML based mobility management.