3GPP TSG-RAN WG2 Meeting #130	R2-2503470

St.Julians, Malta, 19th – 23rd May 2025

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Further discussion on Conditional LTM

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining details for intra-CU conditional LTM are discussed.

Discussion

Leftover issues for target beam selection in L1 based RACH-less CLTM

In last meetings, RAN2 agreed that the target beam selection for L1 based CLTM is mainly based on the beam which triggered the CLTM execution condition:

For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

However, simply assuming that the UE will choose the trigger beam as the target beam in all scenarios may have some potential issues, and the UE behavior still remain unclear in some scenarios.

Let’s assume single beam fulfills C-LTM execution condition first.

Triggered beam without associated CG resources

In our understanding, for CG-based LTM/C-LTM, the network pre-configures certain beams and corresponding resources of each candidate cell for a UE in the LTM configuration. If the triggered beam is NOT associated with any CG resources, it means the network does not reserve resources in that beam for this UE, and those resources may already be allocated for other uses. In this scenario, the UE's behavior is still unclear.

Please note that this is not a corner case as the CG resources depends on network scheduling policy and resource allocation, while the triggered beam is related to channel condition, which is changed dynamically.

Observation 1: It is unclear the UE behavior if the triggered beam is NOT associated with any CG resource.

Potential UE behaviors if the triggered beam is NOT associated with any CG resources:

- Option 1: The target beam is always the triggered beam. If the triggered beam is not associated with any CG resources, the UE performs CBRA using the triggered beam to send a preamble to the network.

- Option2: Target beam may not be triggered beam. If the triggered beam is not associated with any CG resources and UE has valid TA, UE selects a good beam with CG resource to send the first UL message without RACH.

- Option3: If the triggered beam is not associated with any CG resources and UE has valid TA, UE does not perform C-LTM cell switch and wait for the next triggered beam with CG resource. (i.e., C-LTM event evaluation only focus on beams associated with CG resources)

Analyzation of these potential solutions:

Option 1 aligns with the previous RAN2 agreement but may introduce more complexity to network scheduling. The intention of the UE sending a RACH preamble via the triggered beam is to request the resources that are not pre-allocated to the UE. From the network's perspective, those resources should already be reserved or in use by other UEs. In this case, the contention resolution would be needed, and it is probably that extra interruption will be introduced due to preamble retransmission, or even handover failure. Moreover, since the UE has obtained a TA for the target cell, performing RACH again to request the use of one specific beam seems wasteful and is not aligned with the legacy handover RACH procedure, which is mainly based on CFRA and UE-specific RACH resources.

Observation 2: Option1 will introduce extra mobility interruption and even handover failure rate due to the resource conflict. It is also inefficient for the UE to perform RACH if it already has a valid TA, and it is unfriendly to network resource scheduling.

Option 2 and Option 3 differ in whether the UE still triggers a conditional LTM cell switch if the triggered beam has no associated CG resources. This depends on the specific scenario. Sometimes the quality of sub-optimal beams is still good enough, and performance remains acceptable even using sub-optimal beams. Defining such threshold may depend on different deployment, UE speed, and frequency.

In option2, the threshold of “good beam” can be configured by network and realize option3 if the threshold is set very high. So option3 can be a sub-case of option2. Therefore, option2 is slight preferred from our side. RAN2 can further discuss these options.

Proposal 1: RAN2 discuss the potential solutions if the triggered beam is NOT associated with any CG resources:

- Option 1: The target beam is always the triggered beam. If the triggered beam is not associated with any CG resources, the UE performs CBRA using the triggered beam to send a preamble to the network.

- Option2: Target beam may not be triggered beam. If the triggered beam is not associated with any CG resources and UE has valid TA, UE selects a good beam with CG resource to send the first UL message without RACH.

- Option3: If the triggered beam is not associated with any CG resources and UE has valid TA, UE does not perform C-LTM cell switch and wait for the next triggered beam with CG resource.

CG resources allocation

For CG-based LTM/C-LTM, the network pre-configures certain beams and corresponding resources of each candidate cell for a UE in the LTM configuration. In Rel-18 LTM, the LTM cell switch is triggered by the network, and the network does not need to reserve all CG resources throughout the entire LTM and subsequent LTM procedures. For example, when the network indicates the UE to switch to cell A, the CG resources in cells B, C, and D can be temporarily released for use by other UEs.

In another example, if the network pre-configures CG resources associated with beams #1 to #8 for a UE, and then network activates TCI states with QCLed beams #1, #3, and #5 during early DL synchronization (assuming joint TCI state), the remaining beams (#2,4,6,7,8) of that candidate cell can be temporarily re-allocated to other UEs.

Observation 3: In Rel-18 LTM, the cell switch is controlled by the network. The network does not need to reserve all CG resources for all candidate cells for a UE throughout the entire LTM and subsequent LTM procedures. The resources can be dynamically scheduled and allocated by the network.

However, in R19 Conditional LTM, the cell switch procedure is triggered by the UE, and the target cell is unknown to the network before the cell switch is triggered. Also, since the CLTM is triggered by UE, it seems not possible to receive DCI at the moment of the CLTM cell switch is triggered. Therefore, The CG Type2 using DCI for dynamically resource control cannot be supported, which means the CG is configured via RRC and become active immediately after configuration. In this scenario, it is unclear whether the network could reserve all pre-configured resources and beams of all candidate cells throughout the entire LTM and subsequent LTM procedures. From our understanding, this appears to result in a waste of network resources.

Observation 4: In R19 Conditional LTM, the network does not know the UE-selected target cell in advance, and CG Type2 is not supported. It is unclear whether the network can reserve all pre-configured resources and candidate beams of all candidate cells for a UE throughout the LTM and subsequent LTMs.

Therefore, we would like to check if RAN2 has a common answer for Observation 4. If the answer to Observation 4 is Yes, it means the UE is allowed to use any CG resource and associated target beam of one candidate cell to send the first UL message without RACH, if UE has a valid TA for that cell.

Proposal 2: RAN2 to discuss and confirm whether the UE is allowed to use any CG resource and associated target beam to send the first UL message without RACH, if the TA of the target cell is valid (via early UL sync). If yes, it means the network will reserve all CG resources of all candidate cells for one UE throughout the LTM and subsequent LTMs.

Further thinking on dynamic resource allocation

If the answer to Proposal 2 is No, it indicates that the cost of reserving all CG resources for all candidate cells for a UE during the entire LTM and subsequent LTM is unaffordable for the network. In that case, further dynamic resource control and allocation are needed.

The Early TCI activation/deactivation procedure in R18 LTM is an effective way to provide such control, as it is managed by the network and operates at the beam level. Additionally, it does not require extra specification effort. However, to implement this, the following conditions must be met:

- Early TCI activation/deactivation is mandatory for C-LTM.

- The C-LTM event evaluation should start only after early TCI activation/deactivation procedure.

- UE only selects target beam among active TCI states.

Alternatively, if we want to decouple early TCI state activation from resource allocation, CG resources can be provided more dynamically (something like CG Type2, as now CG Type2 is not supported). The TA MAC CE is a good way to carry such information, as it is used to provide TA for early UL synchronization, and the network should also reserve some UL grant at that moment. However, this solution requires more spec impact and support on inter-node signaling. And all discussions in this section are based on the answer on proposal 2. Therefore, we would like to wait for the answer to Proposal 2 first.

Proposal 3: If the answer to Proposal 2 is No, RAN2 should further discuss potential solutions for dynamic resource allocation and UE target beam selection. The use of the R18 Early TCI activation/deactivation procedure or TA MAC CE with UL grant can be considered.

Target beam selection of L3 based C-LTM

For conditional LTM with L3 measurement configured, besides the potential issue mentioned above, another issue is that it is hard to define the target beam due to the beam consolidation and L3 filtering based on the L3 measurement results.

Even without these configurations, selecting a target beam can still be difficult. For example:

- From TTT=0~50ms, beam 1 (of cell 1) above the threshold.

- From TTT=40~130ms, beam 2 (of cell1) above the threshold.

- From TTT=120ms~160ms, beam 1 (of cell 1) above the threshold again.

When TTT=160ms (TTT length), the event condition is fulfilled and the cell 1 is selected as target cell. In this case, it is hard to say if beam1 or beam2 is the target beam.

Observation 5: For L3 based conditional LTM, the target beam cannot be determined with beam consolidation and L3 filtering. Even without these configurations, it is hard to determine the target beam with the best quality as the best beam may change during TTT.

Therefore, we suggest leaving this decision to UE implementation.

Proposal 4:  For L3 based C-LTM, it is up to UE implementation to select the target beam.

Target beam selection of RACH-based C-LTM

For RACH-based CLTM target beam selection, a straightforward approach might be to consider the target beam as the one used for RACH. However, in R18 LTM, the UE may receive the LTM CSC MAC CE with both target TCI state and the CFRA information for the random access procedure. If these two reference signals are configured differently, the UE is expected to switch to the indicated target TCI state for subsequent UL transmission, rather than using the CFRA SSB. This is specified in the RAN1 specification.

TS 38.213 V18.5.0 << Omit irrelevant parts >> For RACH-based LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is indicated for the candidate cell. For RACH-less LTM cell switch [19, TS 38.300], the UE applies the CandidateTCI-State for receptions on the candidate cell and applies a spatial domain filter corresponding to the CandidateTCI-State or the CandidateTCI-UL-State for transmissions on the candidate cell before a new TCI state is indicated for the candidate cell.

Observation 6: In Rel-18 RACH-based LTM, the UE switches to the indicated target TCI state for the following UL transmission rather than using the CFRA SSB, if configured differently.

In RAN2 #129, it has been agreed that the CFRA is supported for CLTM:

For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed

Therefore, the selection of RS/beam for the RACH procedure and subsequent UL transmission may not be always aligned. We further discuss different cases for CFRA and CBRA.

CFRA

For CFRA, the SSB resource is indicated in the CFRA information in RRC RACH-ConfigDedicated, so the UE will track the indicated CFRA SSB for the RACH procedure first. However, after RACH, there are several possible solutions for target beam selection:

- The UE continues to use the CFRA SSB until the next TCI state activation indication.

- Network indicates the target beam (TCI States Activation) to UE right after RACH procedure is complete.

3a. The UE selects the previous triggered beam (before RACH) as the target beam. (This beam may not be associated with CG resource)

3b. UE selects the current best beam of the new serving cell as the target beam (This beam may not be associated with CG resource)

Proposal 5: For CFRA based C-LTM, RAN2 to discuss the target beam selection after RACH:

Option1: The UE continues to use the CFRA SSB until the next TCI state activation indication.

Option2: Network indicates the target beam (TCI States Activation) to UE right after RACH procedure is complete.

Option 3a: The UE selects the previous triggered beam (before RACH) as the target beam. (This beam may not be associated with CG resource)

Option 3b: UE selects the current best beam of the new serving cell as the target beam (This beam may not be associated with CG resource)

CBRA

If CFRA resource is not included in the candidate configuration, UE perform RACH via CBRA. According to the current specification, the UE can select any SSB above a certain threshold as a higher priority, otherwise selecting any SSB. This behavior does not require enhancement.

However, since the UE may continue to use the same beam for subsequent transmissions after RACH, it is recommended that the UE select the triggered beam for the RACH procedure. This can be added as a note to the corresponding spec.

Proposal 6: For CBRA based C-LTM, UE can select the triggered beam for random access procedure.

Candidate TA maintenance and refresh

In the last meeting, the handling of C-LTM TAT for the target cell, source cell, and other candidate cells was clarified:

Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

The C-LTM TAT timers of candidate cells will continue running regardless of C-LTM cell switch, until they expire.

Since early UL synchronization is important for reducing latency during LTM, enhancements could be considered to refresh the candidate TA if the TA timer expires before the conditional cell switch is triggered. A typical approach is to send another TA MAC CE to update the TA value. However, The TA value is used for synchronizing uplink transmission to ensure that the signalling arrives at the gNB at the correct time. The expiration of the TA may indicate that the UE's position has changed without the network's knowledge. Therefore, another RACH preamble is needed to inform the network of this change.

Since both the UE and the network are synchronized with the TA timer, they should be aware of the TA expiration and can feasibly trigger the update procedure. For example, the network may send a PDCCH order to prompt the UE to send another preamble.

Proposal 7: For PDCCH-ordered early TA acquisition, if the candidate TA timer expires before the conditional LTM cell switch is triggered, it is up to the network to initiate a TA refresh by sending another PDCCH order to the UE.

Co-existence on CLTM and MIMO 2TA

In the last meeting, RAN2 reached the following agreement for co-existence on CLTM and MIMO 2TA:

RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In the current running CR, there is only one TAC field in the LTM candidate TAC MAC CE. It is possible for the UE to have only one valid TA for a 2TA candidate cell when CLTM is triggered. In such cases, only the TCI states associated with the valid TA can be selected for RACH-less CLTM, while other candidate TCI states in that cell can only be chosen for RACH-based CLTM.

Observation 7: If the UE has only one valid TA for a 2TA LTM candidate cell when CLTM is triggered, only the TCI states associated with the valid TA can be selected for RACH-less CLTM, while other candidate TCI states in the target cell can only be chosen for RACH-based CLTM.

Therefore, if the triggered beam does not have a valid TA while other candidate beams do, it is unclear which beam the UE should select. The UE may choose the triggered beam and perform RACH-based CLTM, or select another beam with a valid TA to perform RACH-less CLTM.

Proposal 8: RAN2 to discuss the scenario for a 2TA candidate cell when the target cell have valid TA but the target beam does not：

Option 1: The UE selects the triggered beam as the target beam and performs RACH-based CLTM.

Option2: The UE selects another beam with valid TA to perform RACH-less CLTM (FFS the condition to choose another beam).

Additionally, the two agreements made during the last meeting are all for the early UL sync with PDCCH-ordered early RACH case. For UE-based TA, no TAC MAC CE is sent to the UE. In this scenario, the situation is similar to R18 LTM, and co-existence cannot be supported.

Proposal 9: For CLTM, UE based TA measurement cannot be configured if candidate cell is configured with MIMO 2TA.

CLTM and LTM co-existence with TA acquisition (MAC-14)

In the open issue list, MAC-14 raised a concern regarding the appropriate TA (Timing Advance) to use when a CLTM TAT is running and an LTM CSC MAC CE for the same candidate is received. The issue can be addressed in the following scenarios:

- Case 1: If valid TA is included in the (Enhanced) LTM CSC MAC CE, UE uses the latest TA value indicated in the (Enhanced) LTM CSC MAC CE. the UE should use the latest TA value indicated in the (Enhanced) LTM CSC MAC CE. The reason is that the validity of the new TA is longer than the TA previously received in the TA MAC CE.

- Case 2: If valid TA is NOT included in the (Enhanced) LTM CSC MAC CE, UE uses the valid TA from the TA MAC CE and perform RACH-less LTM, if the corresponding CLTM TAT is still running.

- Case 3: If valid TA is NOT included in the (Enhanced) LTM CSC MAC CE and UE does not have valid TA for the indicated cell, UE performs RACH-based LTM.

Proposal 10: When the UE receives the LTM CSC MAC CE while the event evaluation for CLTM is ongoing, the UE should prioritize TA acquisition as follows:

- The new TA indicated in the LTM CSC MAC CE.

- The valid TA with a running CLTM TAT.

- RACH-based LTM.

UE capability issue (ltm-RACH-LessCG-r18)

In the last meeting, the following agreement was made to reuse the R18 RACH-less capability for supporting RACH-less Conditional LTM.

- When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

However, this agreement does not consider a scenario where UE support R18 RACH-less LTM+ R19 CLTM, but does not support R19 RACH-less CLTM. A UE only support R18 RACH-less LTM and R19 RACH-based CLTM cannot find a proper way to indicate its capability. It is wired as R19 RACH-less CLTM introduced new features like early TA(s) acquisition and TA maintenance. A UE should be able to report this difference.

Observation 8: For a UE support R18 RACH-less LTM, R19 basic CLTM, but not support R19 RACH-less CLTM cannot find a proper way to indicates its capability.

Therefore, we suggest to simply revise the previous agreement and field description in the running CR to solve this issue:

When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, and at last one of cltm-EarlyTA-Indication-r19 and ue-TA-Measurement-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant.

The field description of the latest UE capability running CR can be simply updated as follow to solve the issue:

| ltm-RACH-LessCG-r18 Indicates whether the UE supports RACH-less LTM with configured grant for MCG LTM if the UE indicates support of ltm-MCG-IntraFreq-r18 or for SCG LTM if the UE indicates support of ltm-SCG-IntraFreq-r18 respectively. UE indicating support for this feature shall also indicate support of either ltm-BeamIndicationJointTCI-r18 or ltm-BeamIndicationSeparateTCI-r18 for at least one band and either ta-IndicationCellSwitch-r18 or ue-TA-Measurement-r18. If the UE indicates support of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19, and at least one of cltm-EarlyTA-Indication-r19 and ue-TA-Measurement-r18, this feature also indicates whether the UE supports RACH-less conditional LTM with configured grant for MCG LTM. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Proposal 11: Add the condition “at last support one of cltm-EarlyTA-Indication-r19 or ue-TA-Measurement-r18” to determine whether UE support RACH-less conditional LTM. The TP to the UE capability running CR is adopted.

Conclusion

In this contribution, the following proposals are made:

2.1 Leftover issues for target beam selection in L1 based RACH-less CLTM

Proposal 1: RAN2 discuss the potential solutions if the triggered beam is NOT associated with any CG resources:

- Option 1: The target beam is always the triggered beam. If the triggered beam is not associated with any CG resources, the UE performs CBRA using the triggered beam to send a preamble to the network.

- Option2: Target beam may not be triggered beam. If the triggered beam is not associated with any CG resources and UE has valid TA, UE selects a good beam with CG resource to send the first UL message without RACH.

- Option3: If the triggered beam is not associated with any CG resources and UE has valid TA, UE does not perform C-LTM cell switch and wait for the next triggered beam with CG resource.

Proposal 2: RAN2 to discuss and confirm whether the UE is allowed to use any CG resource and associated target beam to send the first UL message without RACH, if the TA of the target cell is valid (via early UL sync). If yes, it means the network will reserve all CG resources of all candidate cells for one UE throughout the LTM and subsequent LTMs.

Proposal 3: If the answer to Proposal 2 is No, RAN2 should further discuss potential solutions for dynamic resource allocation and UE target beam selection. The use of the R18 Early TCI activation/deactivation procedure or TA MAC CE with UL grant can be considered.

2.2 Target beam selection of L3 based C-LTM

Proposal 4:  For L3 based C-LTM, it is up to UE implementation to select the target beam.

2.3 Target beam selection of RACH-based C-LTM

Proposal 5: For CFRA based C-LTM, RAN2 to discuss the target beam selection after RACH:

- Option1: The UE continues to use the CFRA SSB until the next TCI state activation indication.

- Option2: Network indicates the target beam (TCI States Activation) to UE right after RACH procedure is complete.

- Option 3a: The UE selects the previous triggered beam (before RACH) as the target beam. (This beam may not be associated with CG resource)

- Option 3b: UE selects the current best beam of the new serving cell as the target beam (This beam may not be associated with CG resource)

Proposal 6: For CBRA based C-LTM, UE can select the triggered beam for random access procedure.

2.4 Candidate TA maintenance and refresh

Proposal 7: For PDCCH-ordered early TA acquisition, if the candidate TA timer expires before the conditional LTM cell switch is triggered, it is up to the network to initiate a TA refresh by sending another PDCCH order to the UE.

2.5 Co-existence on CLTM and MIMO 2TA

Proposal 8: RAN2 to discuss the scenario for a 2TA candidate cell when the target cell have valid TA but the target beam does not：

Option 1: The UE selects the triggered beam as the target beam and performs RACH-based CLTM.

Option2: The UE selects another beam with valid TA to perform RACH-less CLTM (FFS the condition to choose another beam).

Proposal 9: For CLTM, UE based TA measurement cannot be configured if candidate cell is configured with MIMO 2TA.

2.6 CLTM and LTM co-existence with TA acquisition (MAC-14)

Proposal 10: When the UE receives the LTM CSC MAC CE while the event evaluation for CLTM is ongoing, the UE should prioritize TA acquisition as follows:

- The new TA indicated in the LTM CSC MAC CE.

- The valid TA with a running CLTM TAT.

- RACH-based LTM.

2.7 UE capability issue (ltm-RACH-LessCG-r18)

Proposal 11: Add the condition “at last support one of cltm-EarlyTA-Indication-r19 or ue-TA-Measurement-r18” to determine whether UE support RACH-less conditional LTM. The TP to the UE capability running CR is adopted.

References

- R2_129bis_R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM