3GPP TSG-RAN2 Meeting #130	R2- 2503475

# St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.7.4.1 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Remaining issues on LCP enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to define additional Logical channel priority handling based on the delay/deadline information.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

During the post-meeting discussion, following open issues [2] are remained for LCP enhancements.

- [MAC-01] Impact of priority adjustment on SR priority determination - [MAC-02] Impact of congestion (i.e. PSI-based SDU discard) on priority adjustment - [MAC-13] Priority fallback with consideration of PDU Set integrated handling

This document discusses remaining open issues on LCP enhancement for XR.

2.	Discussion

On SR priority determination for intra-UE prioritization with additional priority

In RAN2#129bis meeting [3], for SR transmission, it was agreed to further discuss whether additional priority can be applied for SR PUCCH transmission during intra-UE prioritization. However, during the CR review, it was still remained as an FFS due to no sufficient feedbacks on this issue:

- We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

In detail, during the MAC CR discussion, in order to apply additional priority for SR during the intra-UE prioritization, it was proposed to determine the priority of SR based on the time point of SR PUCCH occasion.

During the discussion on SR priority for intra-UE prioritization in last meeting, it was pointed out that the priority of SR is determined by the priority of the logical channel that “triggered” the SR and it was also agreed that the additional priority is not applied for triggering SR in RAN2#129 meeting[4]. In addition, it was also pointed out that LCP procedure is not performed during the SR transmission.

(…omitted) 2> if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

In this sense, in order to apply additional priority for SR priority based on the remaining time of the triggering LCH for intra-UE prioritization, additional steps to determine ‘the priority of the logical channel that triggered the SR’ should be defined since no LCP procedure is actually performed for SR transmission. It is different case from priority determination of uplink grant, since the applied priority is determined during the LCP procedure, if additionalPriority is configured for the logical channel.

Here, the detailed procedure could be similar as priority determination during the LCP procedure, e.g.,:

- For triggered SR, if the corresponding logical channel is configured with priorityAdjustmentThreshold and if the smallest remaining value of the running PDCP discardTimers among its data available for this transmission, evaluated at the time of the first symbol of SR PUCCH transmission, is below or equal to its priorityAdjustmentThreshold:

  - The priority of the logical channel triggering an SR is considered as additionalPriority;

- Otherwise,

  - The priority of the logical channel triggering an SR is considered as priority;

In addition, given that ‘the priority of the logical channel that triggered the SR’ at time of SR PUCCH transmission, the remaining time for a LCH should be re-checked in each SR PUCCH transmission in the same SR procedure. In other words, it is possible that the applied priority is legacy priority (i.e., priority) for initial SR PUCCH transmission, while the applied priority for retransmission is additional priority (i.e., additionalPriority) as the remaining time decreases. In our understanding, re-determination of priority for each SR PUCCH transmission is not simple enough and inconsistent with priority determination of UL grant, since the priority of UL grant is not changed for retransmission case.

On the other hand, prioritization of SR PUCCH occasion may not be essential in many cases. For BSR procedure, since the SR is triggered based on the legacy priority, it is not usual case to trigger regular BSR based on LCH with delay-critical data. In addition, the prioritization of SR transmission may not be helpful to transmit the corresponding UL data, as the actual transmission for the corresponding data will be performed later, i.e., after receiving the UL grant after the SR PUCCH transmission. If the UL grant is prioritized over the SR in this case, the corresponding data or BSR/DSR MAC CE will be transmitted prior to the UL data from LCH with non-delay critical data, so the benefit of prioritizing SR will be limited only in special cases.

In this sense, it would be better to not further consider further enhancement of priority adjustment for SR transmission, given that new procedure to determine ‘the priority of the logical channel that triggered the SR’ is needed and causes inconsistency with the intra-UE prioritization for UL grant.

Proposal 1. For priority determination of SR, always apply the legacy priority, i.e., no need to apply the additional priority.

Consideration on PSI based discard priority for priority adjustment

In MAC open issue list, it was added as an open issue on whether any further consideration on PSI-based discard is needed for LCP enhancement for XR.

In our understanding, if the PSI-based discard is activated, low importance data does not affect the LCH priority adjustment, since the LCP priority is determined by remaining time until PDCP discard timer expiry, while low importance data is discarded by separated discard timer, i.e., discardTimerForLowImportance. In addition, given that separated discard timer (i.e., discardTimerForLowImportance) would be configured with the shorter value than legacy discard timer (i.e., discardTimer) value, it is less likely that next data is queued in the same buffer and becomes LCH priority-adjusted data, while the low importance data ahead of LCH priority-adjusted data has not been discarded yet. Therefore, it would be corner case when the low importance data causes any impact on existing LCP procedure.

In Rel-18, several enhancements on DSR procedure were proposed considering the congestion scenario, e.g., deactivating DSR procedure when the PSI-based discard is activated, in order to reduce the additional overhead by transmitting DSR MAC CE. However, it was not agreed in order to minimize additional complexity caused by any dependency between the two functionalities.

Therefore, given that there is no critical motivation to couple two functionalities, i.e., PSI-based discard and LCP enhancement, no further enhancements on LCP procedure for PSI-based activation case is needed.

Proposal 2. No further enhancement on LCP procedure is needed based on PSI-based discard activation.

PDU set discard in LCP enhancements

During the running MAC CR review, it was suggested to discuss whether the PDU set discard should be considered when the UE decides whether the logical channel priority should be fallback form additionalPriority to priority. Specifically, when the PDU set discard is configured but if only some of PDCP PDUs in PDU set is included in the first round of LCP procedure, the logical channel priority of this LCH will be fallback from additionalPriority to priority based on PDCP discard timer value of each PDCP PDU, even though the shortest remaining time of the PDU set is less than the priorityAdjustmentThreshold.

It is true that applying the discard timer value of each PDCP PDU is sub-optimal for the LCP enhancement. However, it should be noted that PDU set discard is not considered in other parts of Rel-18 XR and Rel-19 XR functionalities, e.g.,

- In the first round of LCP procedure, priority adjustment is performed based on running PDCP discard timer value for each PDCP SDU, regardless of whether PDU set discard is activated or not.

- In DSR triggering procedure, only running PDCP discard timer value for each PDCP SDU is considered, regardless of whether PDU set discard is activated or not.

In this sense, if RAN2 starts to consider the PDU set discard on priority fallback on 2nd round of LCP procedure, further discussion is needed on whether the PDU set discard should be considered in other XR-related functions (i.e., priority adjustment in the 1st round of LCP procedure or DSR triggering procedure) as well.

On the other hand, due to fairness issue and UE complexity, current LCP procedure does not ensure that all the LCH priority-adjusted data is included prior to other data in all cases, e.g., by no further enhancement to update Bj or PBR value. Therefore, it is not necessary to keep the additionalPriority to ensure the transmission of PDU set, while it causes additional UE complexity to identify the PDU set and the shortest remaining time of the corresponding PDU set.

In this sense, further consideration of PDU set discard is not essential LCP enhancements.

Proposal 3. No further consider PDU set discard in LCP enhancements.

3.	Conclusion

In this contribution, it is discussed for the LCP enhancement for XR. This document includes following proposals.

Proposal 1. For priority determination of SR, always apply the legacy priority, i.e., no need to apply the additional priority.

Proposal 2. No further enhancement on LCP procedure is needed based on PSI-based discard activation.

Proposal 3. No further consider PDU set discard in LCP enhancements.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] R2-2503361 List of open issues in MAC

[3] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7 - 11 April, 2025

[4] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17-21 February, 2025