3GPP TSG-RAN WG2 Meeting #130	R2-2503482

Malta, 19th – 23rd May 2025

Agenda Item:	8.2.3

Source: 	Xiaomi

Title:  	Remaining open issues on access procedure for A-IOT

Document for:	Discussion and decision

# Introduction

In the email discussion [POST129bis][017][AIoT] 38.391 Running CR (Huawei), A-IoT MAC CR Rapporteur listed open issues for Paging, Random access and data transmission.

In this contribution, we discuss the following RACH related open issues:

- Subgroup: R2D trigger message and Msg1 related

  - Issue 2-1: Msg1 resource selection

  - Issue 2-2: Paging&first R2D trigger message

  - Issue 2-3: R2D trigger message byte alignment

-	Subgroup: CBRA procedure related

  - Issue 2-4: CBRA failure detection

-	Subgroup: Msg2 content

  - Issue 2-5: random ID differentiation in Msg2

  - Issue 2-6: number indication of echoed random IDs in Msg2

  - Issue 2-7: present/absent indication of assigned AS ID in Msg2

- Subgroup: CFRA procedure specific

  - Issue 2-8: no re-access for CFRA

  - Issue 2-9: AS ID assignment in multi-device CFRA

- Subgroup: NACK feedback

  - Issue 2-10: NACK before paging or R2D trigger message

  - Issue 2-11: explicit message for NACK

  - Issue 2-12: multiplexing for NACK indication

# Discussion

## 2.1 Subgroup: R2D trigger message and Msg1 related (issue 2-1, 2-2, 2-3)

### 2.1.1 Issue 2-1: Msg1 resource selection

As listed by Rapporteur, the issue is

| Issue 2-1: Msg1 resource selection | Whether/how to specify the device detailed behaviour of randomly selecting the Msg1 resource based on the R2D trigger message. - Relevant agreements: - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. - Status in running CR: captured as Editor’s Note in 5.3.3.1. |
| --- | --- |

In last RAN2 meeting, RAN2 discussed whether RFID based access occasion selection is reused for A-IoT. Based on the online discussion, the access occasion selection is:

1 Paging message indicates the Number of access occasions for a paging round, i.e. between two paging message;

2 New R2D trigger message indicates the start of a set of MSG 1 resources;

3 Based on RAN1 agreements, from time domain, the max time slots between two R2D trigger messages for CBRA Msg 1 transmission is 2. Therefore there are two access occasions between two R2D trigger messages if the X is 2.

4 the device randomly selects the access occasion based on the Number of access occasions indicated in paging message; See the draft CR from A-IoT MAC Rapporteur

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2

The A-IoT MAC entity shall: 1> randomly select an access occasion for transmission of the Random ID message among the access occasions configured in A-IoT Paging message;

The question is how can a device find the access occasion?

Option 1: count down (same as RFID);

Option 2: increase the number until reach the selected number.

From performance perspective, there is no difference between these two options. Therefore we prefer to leave it to device implementation.

Proposal 1 (Issue 2-1): Same as current TP, the A-IoT device randomly selects an access occasion among the access occasions configured in A-IoT Paging message. The details, e.g., count down or increase the number are left to Device implementation. No additional spec impacts. The issue 2-1 can be closed.

### 2.1.2 Issue 2-2: Paging&first R2D trigger message

As listed by Rapporteur, the issue is

| Issue 2-2: Paging&first R2D trigger message | Whether the R2D trigger message is needed in CFRA, and whether the first R2D trigger message will be merged into paging message in CBRA. - The Rapp understands the discussion of the R2D trigger message focused on CBRA, and it’s not crystal clear whether the R2D trigger message is also needed in CFRA, and whether it can be merged to paging message if it’s the first R2D trigger message in CBRA. - Status in running CR: not captured yet. |
| --- | --- |

We illustrate the timing relationship between R2D and D2R messages in figure 1:

Figure 1: Timing relationship for a session (T=1)

Based on RAN1 agreements, the timing relationship requirements between R2D and D2R messages are:

- The device 1 transmits Msg 1 based on Toffset1 upon receiving Paging messaage.

- If there is new R2D message right after R2D paging messaage, the device 1 is not able to receive it in Toffset1, i.e., the reader can only transmit it after Toffset1.

If new trigger message is always transmitted before the access slot, additional latency is expected, i.e., Toffset1.We could not find the clear benefit to transmit a new trigger message right after Paging message, but only the drawback, e.g., additional delay, waste resources. Therefore

Proposal 2 (Issue 2-2): The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.

Another question is whether the new trigger message is useful for CFRA or not. As agreed in RAN2, the new R2D message is only used in CBRA procedure to indicate the timing of access occasion for Msg1 transmission.   As discussed in [3], we do not see the need to use this new R2D message for CFRA.

For paging triggered CF access, the target device id will be included in paging message, and the reader will know whether the target device has completed the service or not. If not, network can send paging again to trigger the device to re-access with the device ID. The transaction ID is not needed. As also mentioned by Rapporteur, the number of access occasions, Indication of Paging ID present/absence are also not needed for CFRA.

Proposal 3 (Issue 2-2): The new R2D trigger message is not applied for CFRA.

### 2.1.3 Issue 2-3: R2D trigger message byte alignment

As listed by Rapporteur, the issue is

| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. - Relevant agreements: - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message. - Status in running CR: not captured yet. |
| --- | --- |

As RAN2 agreed, the R2D trigger message is to determine the start of the access occasion for a set of Msg1 resource, the scheduling information contained in the paging message and the slot number/count down number are not included in the new R2D trigger message. Therefore, the only content of the new R2D trigger message is the message type. Even this R2D message is not byte alignment, it will not increase the device complexity, especially considering bit based design is used in RFID. In addition, this new R2D trigger message is transmitted frequently, if byte alignment is required, padding/reserved bits will be added, which is also a waste of resource. Therefore, we propose RAN2 to agree byte alignment is not required for the new R2D trigger message.

Proposal 4 (Issue 2-3): RAN2 to agree that byte alignment is not required for new R2D trigger message.

## 2.2 Subgroup: CBRA procedure related (issue 2-4)

### 2.2.1 Issue 2-4: CBRA failure detection

As listed by Rapporteur, the issue is

| Issue 2-4: CBRA failure detection | How to determine CBRA failure/contention resolution failure. - In SI, RAN2 agreed to support re-access in case of contention resolution failure as capture in TR 38.796. In WI phase, RAN2 has agreed to re-use the subsequent paging message to trigger re-access, but has not discussed how to determine CBRA failure/ contention resolution failure. - Status in running CR: captured as Editor’s Note in 5.5. |
| --- | --- |

The issue is related to NACK discussion. RAN2 has agreed that

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.

For CBRA, RAR is used for the contention resolution. If a device receives the RAR, the device shall consider the CBRA is successful. Therefore, for CBRA failure, there are 3 cases:

Case 1: Msg 1 (RN 16) transmission failure

Case 2: Contention resolution failure

Case 3: Msg 2 (RAR) reception failure

From device perspective, the device has no idea whether the failure is caused by case 1, 2 or 3. The device shall just wait for Msg2 after Msg 1 transmission. When the device receives a new RAR trigger message or an A-IoT paging message prior to receiving RAR, the device determines that the CBRA has failed.

Proposal 5 (Issue 2-4): In the CBRA procedure, after transmitting Msg 1, the device determines that the CBRA has failed if it receives either a new RAR trigger message or an A-IoT paging message prior to receiving the RAR.

## 2.3 Subgroup: Msg2 content (issue 2-5)

### 2.3.1 Issue 2-5: random ID differentiation in Msg2

As listed by Rapporteur, the issue is

| Issue 2-5: random ID differentiation in Msg2 | Whether/how to address random ID collision in Msg2, i.e. multiples devices generate same random ID using different Msg1 resources. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - Status in running CR: not captured yet. |
| --- | --- |

The issue was discussed in last meeting. As shown in the figure, if the device 1 and the device 2 trigger CB access with the same RN16 in different FDMA based access occasion within the same access slot, the reader can identify these two devices since the access occasion is different. But how to indicate these two devices in subsequent R2D message (Msg 2)? We think it is worth to solve the issue considering there is no ambiguous issue from the Reader perspective. As discussed in RAN1, the max number of devices to be multiplexed in the frequency domain of an access slot could be 4 or 6 (FFS in RAN1). At most 3 bits are required to indicate the used frequency of the access slot.

Figure 2 RN16 collision

Proposal 6 (Issue 2-5): To address the RN16 collision issue, i.e., introduce 3 bits in Msg 2 to indicate the frequency of the access slot together with responded RN16.

## 2.4 Subgroup: CFRA procedure specific (issue 2-8. 2-9)

### 2.4.1 Issue 2-8: no re-access for CFRA

As listed by Rapporteur, the issue is

| Issue 2-8: no re-access for CFRA | How to achieve “no re-access” for CFRA - Relevant agreements: - For CFRA, NACK feedback and re-access is not supported. FFS how to achieve. - Status in running CR: captured as Editor’s Note in 5.2. |
| --- | --- |

CFRA is totally controlled by the reader. For instance, if there is any issue, the reader can send paging with the device ID again to trigger the re-access. However in last RAN2 meeting, companies have different view on how to achieve this.

Option 1: the transaction ID is still contained in the Paging message for CFRA, but the device just ignores it if CFRA is performed.

Option 2: the transaction ID is not contained in the Paging message for CFRA.

The reason why some companies prefer option 1 because they would like to support unified Paging message, i.e., Same Paging message is used for CFRA and CBRA.

As discussed in [3], not only transaction ID, but also “the number of access occasions”, “Indication of Paging ID present/absence” are not applied for CFRA. In addition, based on RA type, we can design two Paging messages by using the same Message type for CFRA and CBRA separately which can reduce the message size of CFRA paging message, and also can simply the device implementation, i.e., without the need to check multiple branches.

Proposal 5 (Issue 1-5): For paging message, “transaction ID”, “the number of access occasions”, “Indication of Paging ID present/absence” are not contained for CFRA. Proposal 5.1 (Issue 1-5): For paging message, the message content for CBRA and CFRA is distinguished by RA Type.

Proposal 7 (Issue 2-8): For CFRA, NACK based feedback and re-access is not supported. The reader triggers the re-access by retransmitting the Paging message with the same device ID. “transaction ID”, “the number of access occasions”, “Indication of Paging ID present/absence” are not contained in the Paging message for CFRA (Same as issue 1-5).

### 2.4.2 Issue 2-9: AS ID assignment in multi-device CFRA

As listed by Rapporteur, the issue is

| Issue 2-9: AS ID assignment in multi-device CFRA | Whether to consider multiple device scenario as to the AS ID in CFRA. - Relevant agreements: - ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. - Status in running CR: not captured. |
| --- | --- |

The issue is related to RAN2 agreements on AS ID assignment for CFRA.

Agreements

7	For CFRA, command message is used for AS ID assignment

Some companies indicated that in Rel-20, if paging message can include multiple device IDs, then for CFRA, it is difficult to determine which devices the subsequent R2D message is intended for. We illustrate the scenario in the figure as below:

Figure 3: multiple devices for CFRA

As discussed in last RAN2 meeting, to resolve the issue, following options could be considered:

Options to handle:

-	Option 1: including AS ID in msg0

-	Option 2: no AS ID is used for CFRA

-	Option 3: including RN16 in Msg 1

-	Option 4: assume that a single device can perform a procedure with a reader

To our understanding, the CFRA cannot be used for congestion scenario, otherwise the paging load will be increased significantly. Therefore it is acceptable that the reader only performs the CFRA for a single device at a time.

Proposal 8 (Issue 2-9): RAN2 confirms that the reader only performs the CFRA for a single device at a time, i.e., do not support multiple device scenario for CFRA.

## 2.5 Subgroup: NACK feedback (issue 2-10)

### 2.5.1 Issue 2-10: NACK before paging or R2D trigger message

As listed by Rapporteur, the issue is

| Issue 2-10: NACK before paging or R2D trigger message | For the re-access due to reception of NACK indication before subsequent R2D message, whether the subsequent R2D message is the R2D trigger message or paging message. - Relevant agreements: - For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging - Status in running CR: captured as Editor’s Note in 5.5. |
| --- | --- |

During the online discussion, some companies raised the question that whether the reader shall send the NACK as quick as possible, i.e., within the same access round. There are two options:

Option 1: the reader sends NACK within the same access slot, i.e., between two new R2D trigger messages, or

Option 2: the reader sends NACK within the same paging round, i.e., before next (subsequent) paging.

RAN2 has agreed that re-access is only triggered by subsequent paging. Therefore, the device has to wait even if the reader can send the NACK within the same access slot. From this perspective, there is not difference between option 1 and option 2.

The main benefit for option 1 is that the device can release RN 16/assigned AS ID early. Therefore we slightly prefer option 1.

Proposal 9 (Issue 2-10): The NACK shall be sent within the same access slot, i.e., between two new R2D trigger messages.

### 2.5.2 Issue 2-12: multiplexing for NACK indication

As listed by Rapporteur, the issue is

| Issue 2-12: multiplexing for NACK indication | Whether to support multiplexing of information for multiple devices in NACK feedback. - Relevant agreements: - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages. - Status in running CR: not captured yet. |
| --- | --- |

Considering RAN1 has agreed to support FDMA of D2R transmissions in Msg3 for multiple devices. To respond the failure of multiple devices, it makes sense to support the multiplexing of NACK feedback, similar as multiplexing of Msg2.

Proposal 10 (Issue 2-12): multiplexing is support for NACK message.

# Conclusions

Based on the above discussion, we propose following proposals:

Subgroup: Subgroup: R2D trigger message and Msg1 related (issue 2-1, 2-2, 2-3)

Proposal 1 (Issue 2-1): Same as current TP, the A-IoT device randomly selects an access occasion among the access occasions configured in A-IoT Paging message. The details, e.g., count down or increase the number are left to Device implementation. No additional spec impacts. The issue 2-1 can be closed.

Proposal 2 (Issue 2-2): The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.

Proposal 3 (Issue 2-2): The new R2D trigger message is not applied for CFRA.

Proposal 4 (Issue 2-3): RAN2 to agree that byte alignment is not required for new R2D trigger message.

Subgroup: CBRA procedure related (issue 2-4)

Proposal 5 (Issue 2-4): In the CBRA procedure, after transmitting Msg 1, the device determines that the CBRA has failed if it receives either a new RAR trigger message or an A-IoT paging message prior to receiving the RAR.

Subgroup: Msg2 content (issue 2-5)

Proposal 6 (Issue 2-5): To address the RN16 collision issue, i.e., introduce 3 bits in Msg 2 to indicate the frequency of the access slot together with responded RN16.

Subgroup: CFRA procedure specific (issue 2-8. 2-9)

Proposal 7 (Issue 2-8): For CFRA, NACK based feedback and re-access is not supported. The reader triggers the re-access by retransmitting the Paging message with the same device ID. “transaction ID”, “the number of access occasions”, “Indication of Paging ID present/absence” are not contained in the Paging message for CFRA (Same as issue 1-5).

Proposal 8 (Issue 2-9): RAN2 confirms that the reader only performs the CFRA for a single device at a time, i.e., do not support multiple device scenario for CFRA.

Subgroup: NACK feedback (issue 2-10)

Proposal 9 (Issue 2-10): The NACK shall be sent within the same access slot, i.e., between two new R2D trigger messages.

Proposal 10 (Issue 2-12): multiplexing is support for NACK message.

# Reference

[1] R2-2502211 Email discussion report: [POST129][035][AIoT] Paging

[2] R2-2503998 Summary of A-IoT MAC open issues

[3] R2-2503281 Remaining open issues for Paging, Xiaomi