3GPP TSG-RAN WG2 Meeting #130	                                     R2-2503488

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Open issues for conditional LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the open issues for supporting conditional LTM.

Discussion

MAC-9 and MAC-10

| MAC-9 | In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM? Editor’s NOTE: FFS in case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for CLTM target cell, how UE selectes the valid CG resource for L3 based CLTM. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |
| MAC-10 | Beam selection for L3 based RACH-less CLTM determination Editor’s NOTE: FFS how to determine the selected beam in case the CLTM is triggered by L3 event. | Issue Type: Essential How to address it: based on companies’ contribution |

In current running MAC CR, how to determine a valid CG resource for CLTM is captured as following:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the selected SSB, as specified in clause 5.y.3:

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

We understand the text can also be applicable for L3 based RACH-less CLTM

For L3 based RACH-less CLTM, UE considers the CG resource as valid if an SSB corresponding to the configured UL grant has the same SSB index as the selected SSB.

For L1 based RACH-less CLTM, it has agreed that the target beam is the beam which satisfy the execution condition, and it is up to UE implementation to choose one if multiple beams fulfil the condition.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

For L3 based RACH-less CLTM, a similar way can be followed. UE assuming the cell fulfils CLTM L3 execution condition as target cell. and if multiple cells fulfill the L3 execution condition, it can leave for UE implementation to choose one as the target cell.

For L3 based RACH-less CLTM, the target cell is the candidate cell fulfilling the L3 execution condition.

If there are multiple candidate cells fulfil the L3 execution condition, it can leave for UE implementation to choose one as the target cell.

And regarding target beam, UE determined the target beam based on a pre-defined threshold and if multiple beams fulfill the threshold, it can leave for UE implementation to choose one as the target beam.

For L3 based RACH-less CLTM, the selected beam is the beam fulfilling the threshold.

If there are multiple beams fulfil the threshold, it can leave for UE implementation to choose one as the selected beam.

MAC-11

| MAC-11 | In case UE considers RACH-less CLTM is ongoing, i.e., UE has valid TA for LTM target cell, while UE couldn’t obtain valid CG resource, whether UE will fall back to RACH-based LTM? | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

Another open issue is whether UE can fallback to RACH-based CLTM if there is no valid CG resource. For NW triggered LTM, UE is not allowed to fallback to RACH-based LTM even if there is no valid CG resource. UE may perform DG monitoring on the indicated TCI state. For CLTM, DG-based initial UL transmission is not supported since NW is not able to know when UE will execute the CLTM procedure. Instead of waiting for T304 expiration, fallback to RACH-based CLTM could help to avoid unnecessary RRC-reestablishment.

UE can fallback from RACH-less CLTM to RACH-based CLTM if there is no valid CG resource.

MAC-13

| MAC-13 | FFS the coexistence between CLTM and (e)RedCap, CovEnh? Editor’s NOTE: Whether/How CLTM could co-exist with (e)RedCap is FFS, i.e. whether follow Rel-18 intra-CU LTM as below. Editor’s NOTE: Whether/How CLTM could co-exist with CovEnh is FFS, i.e. whether follow Rel-18 intra-CU LTM as below. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

In last meeting, RAN2 initially discussed the coexistence between R19 inter-CU LTM and other features, and the following conclusion has been reached.

Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

For CLTM, similar conclusion can be followed:

Same as Rel-18 intra-CU LTM, Rel-19 intra-CU CLTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU.

MAC-14

| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

In last meeting, it has agreed that NW can send an LTM cell switch command MAC CE indicating a CLTM candidate configuration. And no spec change is expected for supporting this agreement.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

As agreed in last meeting, CLTM candidate TAT timer is not stopped upon CLTM execution, and UE starts the PTAG TAT timer using remaining time.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

Based on the agreements above, there are two ways for CLTM TAT maintenance:

- Option 1: follow CLTM way

CLTM TAT for target cell is not stopped upon LTM cell switch command reception if it is running and starts the PTAG using the remaining time from the C-LTM TAT timer. In this case, the spec change is needed to define CLTM TAT timer handling for the NW triggered LTM cell switch.

- Option 2: follow LTM way

CLTM TAT for target cell is not stopped upon LTM cell switch command reception if it is running and starts the PTAG TAT timer of the target cell. And in this case, NW should always include TA in the LTM cell switch command even the CLTM TAT timer is running, which will not cause any spec change.

To align with the agreement in last meeting, i.e., no specification change is expected, we prefer to go option 2.

Upon reception of LTM cell switch command MAC CE indicating a CLTM candidate configuration index, CLTM TAT for target cell is not stopped if it is running and the PTAG TAT of the target cell is started.

Execution condition evaluation

Based on current TS38.300, if any HO command/ LTM cell switch command is received before any CLTM execution condition is satisfied, UE shall execute the received HO command/LTM cell switch command, regardless of any previously received CLTM configuration.

Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration) or LTM cell switch command MAC CE, the UE executes the HO procedure as described in clause 9.2.3.2 or LTM cell switch procedure as described in clause 9.2.3.5, regardless of any previously received CHO configuration.

For CLTM, same principle should also be followed. Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Coexistence with L3 HO

The issue on coexistence between different types of HO mechanism should also be studied. As shown below, Rel-18 LTM support the coexistence with all other L3 HO mechanisms except DAPS.

| Coexistence(Y/N) | Normal HO | DAPS | CHO | CPAC | SCPAC |
| --- | --- | --- | --- | --- | --- |
| LTM | Y | N | Y | Y | Y |

For CLTM, we can follow similar agreements made in Rel-18, CLTM can be coexisted with other L3 HO mechanism.

CLTM and L3 HO coexistence is supported.

CLTM and DAPS coexistence is not supported.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	For L3 based RACH-less CLTM, UE considers the CG resource as valid if an SSB corresponding to the configured UL grant has the same SSB index as the selected SSB.

Proposal 2	For L3 based RACH-less CLTM, the target cell is the candidate cell fulfilling the L3 execution condition.

Proposal 3	If there are multiple candidate cells fulfil the L3 execution condition, it can leave for UE implementation to choose one as the target cell.

Proposal 4	For L3 based RACH-less CLTM, the selected beam is the beam fulfilling the threshold.

Proposal 5	If there are multiple beams fulfil the threshold, it can leave for UE implementation to choose one as the selected beam.

Proposal 6	UE can fallback from RACH-less CLTM to RACH-based CLTM if there is no valid CG resource.

Proposal 7	Same as Rel-18 intra-CU LTM, Rel-19 intra-CU CLTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU.

Proposal 8	Upon reception of LTM cell switch command MAC CE indicating a CLTM candidate configuration index, CLTM TAT for target cell is not stopped if it is running and the PTAG TAT of the target cell is started.

Proposal 9	Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Proposal 10	CLTM and L3 HO coexistence is supported.

Proposal 11	CLTM and DAPS coexistence is not supported.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx