3GPP TSG-RAN WG2 Meeting #130	R2-2503494

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.3

Source:	OPPO

Title:	Discussion on Measurement Gap Enhancements

Document for:	Discussion, Agreement

Introduction

In the previous meeting, RAN2 discussed potential L2 impacts due to skipping of measurement. In this paper, the RAN2 impacts will be further discussed.

Discussion

Issue-1: UAI

The following LS has been sent to R2 from R4 to inform the related agreements and request RAN2 to implement the signalling

RAN4 thanks RAN1 for the LS on UE assistance information (R4-2411007/R1-2405736). Regarding UE assistance information related to measurement occasions, the following was agreed on RAN4#114 (R4-2502700): Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.. And the following was agreed in RAN4 #114bis (R4-2504911): - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%. - 2 Actions To RAN2 - ACTION: RAN4 respectfully asks RAN2 to implement the signaling of the above mentioned UAI reporting for XR.

RAN4 agreed to define UAI with optional UE capability, and the UAI includes a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

For the UAI signalling design, in the legacy, for the other UAI information, there are 2 types:

- Type-1: The UAI information report is a one-shot indication, i.e., it is transmitted only if the UE did not transmit since it is configured to provide.

- Type-2: The UAI information report is a multi-shot indication, i.e., it can be transmitted whenever there is an update.

In legacy, there are generally two types of UAI information, i.e., one or multi-shot assistance information, which depends on whether there is a requirement to frequent update the information.

Whether the UAI is a one-shot or multi-shot indication depends on whether there is a requirement to flexible update the assistance information or it is more like a UE feature which is not changed frequently. Based on this principle, we understand the assistance information for MG skipping, i.e., a ratio (R) of gap occasions that is recommended for cancellation doesn’t require frequent updates since it is more like a UE capability, the ratio/number of occasions that can be skipped without impacting measurement performance.

The ratio/number of occasions that can be cancelled during a time period generally represent the UE capability which doesn’t require frequent updates.

Therefore, UE only needs to provide the assistance information if the UE didn’t transmit a UEAssistanceInformation message with a ratio (R) of gap occasions that is recommended for cancellation since it was configured to provide in a one-shot manner, and the prohibit timer is not needed. The TP can be as follows:

1> if configured to provide ratio (R) of gap occasions recommended for cancellation: 2> if the UE did not transmit a UEAssistanceInformation message with [ratio (R) of gap occasions recommended for cancellation] since it was configured to provide recommendation: 3>initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide ratio (R) of gap occasions recommended for cancellation;

For the UAI with ratio (R) of gap occasions recommended for cancellation, it is triggered only if the UE is configured to provide and has not transmitted before. No subsequent updates are required, and the prohibit timer is not needed.

Issue-3: Capability

As agreed by RAN1 and RAN4, for DCI-based measurement gap skipping, there is a minimum time offset between indication to skip and the skipped measurement occasion.

RAN1 Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). RAN4 conclusion: RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability. Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

As shown in the above RAN1/4 agreements, the RRC configuration for the minimum time offset is needed. Further details on can wait for RAN1/4 progress.

As concluded by RAN1 and RAN4, the RRC configuration for minimum time offset is needed.

In addition, as indicated in RAN4 LS, at least for the support of X=3ms and UAI, the optional UE capabilities are needed.

For measurement gap skipping, new UE capability will be introduced for DCI-based approach, details pending RAN1/RAN4 further conclusion.

- Conclusion

We have the following observations:

Observation 1	RAN4 agreed to define UAI with optional UE capability, and the UAI includes a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

Observation 2	In legacy, there are generally two types of UAI information, i.e., one or multi-shot assistance information, which depends on whether there is a requirement to frequent update the information.

Observation 3	The ratio/number of occasions that can be cancelled during a time period generally represent the UE capability which doesn’t require frequent updates.

Observation 4	As concluded by RAN1 and RAN4, the RRC configuration for minimum time offset is needed.

We have the following proposals:

Proposal 1	For the UAI with ratio (R) of gap occasions recommended for cancellation, it is triggered only if the UE is configured to provide and has not transmitted before. No subsequent updates are required, and the prohibit timer is not needed.

Proposal 2	For measurement gap skipping, new UE capability will be introduced for DCI-based approach, details pending RAN1/RAN4 further conclusion.