3GPP TSG-RAN WG2 Meeting #130	R2-2503502

St Julian’s, Malta, 19th May 2025 – 23rd May 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues of on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several further agreements have been achieved [1]:

Agreements on OD-SIB1

1.	NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

2.	When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

3.	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

4.	SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

5.	Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

6.	The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

7.	Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

8.	Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

9.	If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

10.	WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

11.	Area id will be used as legacy.

12.	We do not need a separate new triggering condition of OD-SIB1 acquisition.

13.	When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this contribution, we will discuss the remaining issues of on-demand SIB1 operation.

# Discussion

## 2.1 Details of the UL WUS configuration

RAN1 has discussed UL WUS configuration contents during RAN1#118bis and RAN1#119 and reached the following agreements.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

-	rsrp-ThresholdSSB

-	prach-RootSequenceIndex

-	msg1-SubcarrierSpacing

-	restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

-	(working assumption) ULSubCarrierSpacing

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

-	FFS: FDD system

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | frequencyBandList |  |  |
|  | absoluteFrequencyPointA |  |  |
|  | offsetToCarrier |  |  |
|  | p-Max |  |  |
|  | ULSubCarrierSpacing |  |  |
|  | n-TimingAdvanceOffset |  |  |
|  | ssb-PeriodicityServingCell |  |  |
| Purpose | Parameters |  |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

RAN1 has agreed to include the above parameters in the WUS configuration. The CORESET#0 and the SS#0 can be included in the WUS configuration when SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Furthermore, whether the above parameters are mandatory or optional is also under RAN1’s discussion. RAN2 can consider this as the baseline and wait for more progress.

Due to the fact that the WUS config is provided by Cell A, if the Cell A bars a specific UE type (e.g., NTN or RedCap) from accessing or camping on it, that UE type would not be able to obtain the WUS config for neighbouring NES cells. In the current spec, if a UE bars the Cell, there is no need for it to acquire its OSI. Since these system information blocks are useless for a UE that is barred. But in the OD-SIB1 case, the WUS config in new SIBX provided by Cell A is useful for other NES Cells which might not be barred for this UE type. Therefore, we propose that NES UEs could acquire Cell A’s SIB1 to know whether Cell A is an anchor cell or has the broadcast information (i.e., the SchedulingInfo) of the new SIBX with WUS configs. If yes, the NES UE could acquire Cell A’s SIBX to be able to trigger OD-SIB1 in neighbouring NES Cells.

Proposal 1: Once a NES UE acquires Cell A’s SIB1 and knows that it provides the new SIBX, it can acquire Cell A’s SIBX to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

During the last meeting, we have discussed the issue of including Access related information in the UL WUS configuration. There was no consensus whether to include those parameters in the UL WUS configuration based on the discussion:

R2-2502916 On demand SIB1 handling Nokia, Nokia Shanghai Bell discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access. [Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information. Offline among interested companies. Comeback in CB session (Nokia). [Comeback session]: [Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia. Revisit it next meeting.

In previous releases, several new barring bits were added in SIB1 for access control of new UE types, such as RedCap, NTN, R18 NES, etc. Supporting Access related information has clear benefits to achieve both network and UE energy saving gains. If these bits are not provided beforehand, the UE would only find out if it is barred after acquiring OD-SIB1, which would waste UEs energy and cause a delay in network access because the UE would need to go back to Cell A or reselect to another cell if it is barred.

From the discussion above, two key issues have been identified:

1) Signaling overhead caused by the inclusion of PLMN ID, TAC, and other access related information,

2) Determining which features should be included in the UL WUS configuration and whether these parameters could be optional.

In our view, only the feature related barring information should be included in the UL WUS config. It is important to note that cell access related information does not necessarily imply that a UE is restricted or barred. The exchange of PLMN ID, TAC, and other unified access control information is unnecessary, since these IEes are not designed to bar a specific UE type. Furthermore, if a cell does not permit any type of UE to camp or access, it should not be considered as a NES Cell. Therefore, the exchange of CellAccessRelatedInfo and PLMN-IdentityInfo information is irrelevant.

As the NES cell may not support all cell barring related features, there is no need to exchange all feature specific cell barring information. It means that if the NES Cell has no cell barring restrictions, nothing needs to be exchanged between gNBs. In this case the signalling overhead is also small. Hence, the feature based cell barring information should be optional in the UL WUS config.

Proposal 2: Feature specific barring bits from SIB1 are included in the WUS config, if the OD-SIB1 Cell supports and has enabled a specific feature that has a barring functionality.

Next, we will discuss an FFS from the RRC running CR discussion, determining whether the parameters in SIBxx should be mandatory or optional. During RAN1’s previous meeting, they reached an agreement on the mandatory or optional status of certain parameters.

Agreement@RAN1#120bis meeting From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings: - PhysCellId and ARFCN-ValueNR are mandatory - frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification) - K_SSB is mandatory - searchSpaceZero and controlResourceSetZero are mandatory - ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

RAN2 can follow RAN1’s agreement and make the following parameters mandatory. In our view, other parameters should be designed to be optional whenever possible. This approach helps reduce the signaling overhead of SIBxx without impacting NES-capable UEs' ability to receive the OD-SIB1 configuration to trigger NES Cell to send OD-SIB1.

Proposal 3: Parameters in SIBxx should be designed as optional whenever possible to minimize SIBxx overhead.

In the following section, we will discuss the UE behaviour when a parameter from UL WUS configuration is inconsistent with the same parameter in SIB1 in the NES cell.

## 2.2 Discussion on the parameter inconsistency between OD-SIB1 and UL-WUS

In the RAN1#120 meeting, the parameter inconsistency issue has been discussed with no consensus and the topic was left for RAN2 to decide.

As observed in RAN1’s agreements about the UL WUS configuration, several parameters are shared with SIB1. RAN1 did not decide whether OD-SIB1 IEs can be different from the value of the corresponding IEs in the WUS configuration, and how to define the UE behaviour if the values are different.

### Copied from R1-2501380 ### FL Proposal 9-2-1 Huawei’s version: - If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1. ETRI’s version: - For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed. Rapporteur suggestion: - It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

From RAN2 perspective, the common parameters in WUS config and SIB1 should be aligned, since the WUS config is also generated by NES Cell itself. However, there could be a case where the obtained WUS config parameters from Cell A and the OD-SIB1 parameters acquired from the NES Cell are different. This may occur since there might be a delay between updating SIB1 in the NES cell and sending an updated WUS config to neighbouring Cell A’s. In our understanding SIB1 will always be updated first, so the UE should use the parameters from OD-SIB1 if there is a mismatch. That means that the value in OD-SIB1 should override the value of the corresponding IE in the WUS configuration.

Proposal 4: If the value in an OD-SIB1 IE is different from the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration.

# Conclusion

In this contribution, we discussed the remaining issues of on on-demand SIB1 operation, and made the following observations and proposals:

Proposal 1: Once a NES UE acquires Cell A’s SIB1 and knows that it provides the new SIBX, it can acquire Cell A’s SIBX to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Proposal 2: Feature specific barring bits from SIB1 are included in the WUS config, if the OD-SIB1 Cell supports and has enabled a specific feature that has a barring functionality.

Proposal 3: Parameters in SIBxx should be designed as optional whenever possible to minimize SIBxx overhead.

Proposal 4: If the value in an OD-SIB1 IE is different from the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration.

# References

- R2-2502981 Report from session on R18 SL, R18/19 MOB and R19 NES

- Chair notes RAN1#120 eom0.doc