3GPP TSG-RAN WG2 Meeting #130											R2-2503509

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.4.1

Source: 	Sharp

Title:  	Open Issues on LCP Enhancements

Document for: 	Discussion

# Introduction

In RAN2#129bis meeting, RAN2 made the following agreements:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

This document discusses remaining open issues on LCP enhancements.

# Discussion

## Working Assumption: Bj Enhancement

In RAN2#129bis meeting, RAN2 made a working assumption that no Bj enhancement is introduced. The issue was whether an LCH with an upgraded priority and negative Bj value can participate in the first round of LCP. Negative Bj value means that the logical channel already used more resources than that set by PBR and BSD from the QoS point of view. Negative Bj value is allowed to give a flexibility to the UE implementation, but it already means temporal over-usage of resources. The reason why the legacy LCP procedure does not allow LCH with negative Bj to participate in the first round of LCP is a minimal restriction to avoid starvation of lower priority LCHs and guarantee the minimum fairness. If we allow another exceptional case, there will be more performance degradation in fairness. Considering this aspect, we propose to confirm the working assumption.

RAN2 to confirm the RAN2 Working Assumption: No Bj enhancement is introduced.

## Joint Configurations of DSR and Additional Priority

- In RAN2#127bis, RAN2 agreed not to introduce any setting restrictions of the new remaining time threshold for additional priority with relation to DSR triggering threshold remainingTimeThreshold. But there was no discussion/conclusion on whether it is possible to independently configure additional priority and remainingTimeThreshold for DSR.

- In RAN2#129 meeting, the impact of additional priority on BSR was discussed, and it was not agreed because we can rely on DSR. That means that a common understanding is that additional priority is configured only if remainingTimeThreshold for DSR is configured. An expected technical issue is that without DSR configuration, gNB does not have any idea whether the UE has delay-critical data or whether additional priority is being used. It will probably give an impact to timely scheduling. We see that DSR should be a pre-requisite of additional priority.

Additional priority is configured only if remainingTimeThreshold for DSR is configured.

# Conclusions

Based on the discussion above, RAN2 is requested to agree the following proposals:

- RAN2 to confirm the RAN2 Working Assumption: No Bj enhancement is introduced.

- Additional priority is configured only if remainingTimeThreshold for DSR is configured.