3GPP TSG RAN WG2#130                                                                    R2-2503525

St Julian’s, Malta, 19th - 23rd May 2025

Agenda Item:	8.12.3

Source:	Ofinno

Title:	Enhancements for UE-initiated/event-driven beam management

Document for:	Discussion and Decision

# Introduction

The revised WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#105 meeting. The objectives related to UE-initiated/event-driven beam management are listed below:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we present our views on the remaining issues of UE-initiated/event-driven beam management.

# Open issues

## MAC-1: UL skipping for event-triggered beam reporting

In RAN2#129bis, the following agreements are made.

Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping

Based on the confirmed RAN2 understanding made in RAN2#129bis, the Type 1 CG PUSCH carrying the CSI report in Mode-B of UE-initiated CSI reporting is not used in MAC procedure to generate MAC PDU. Therefore, UL skipping in the MAC procedure is not applicable.

As for whether UL skipping is applied to Mode-A of UE-initiated CSI reporting, in our view it should be applicable to both Rel-15 UL skipping (i.e., when enhancedSkipUplinkTxDynamic is configured) as well as Rel-16 UL skipping (i.e., when enhancedSkipUplinkTxDynamic is configured). Since DG PUSCH scheduled for the CSI report Mode-A of UE-initiated CSI reporting reuses the legacy CSI request field similar to legacy aperiodic CSI reporting, the same behavior of aperiodic CSI reporting should be applied to Mode-A of UE-initiated CSI reporting at the MAC layer. In other words, DG PUSCH carrying the CSI report in Mode-A of UE-initiated CSI reporting is used in MAC procedure to generate MAC PDU.

- (MAC-1) UL skipping is not applicable to mode-B type-1 CG event-triggered beam report.

- (MAC-1) For Rel-16 UL skipping (enhancedSkipUplinkTxDynamic is configured), the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated).

- (MAC-1) For Rel-15 UL skipping (skipUplinkTxDynamic is configured), MAC PDU needs to be generated for the UCI for mode-A DG-based UE-initiated report.

# Other identified open issues

## Cell DRX

In RAN1#119, the following agreement has been reached regarding the container of first PUCCH.

RAN1#119 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

For the first PUCCH transmission in UE-initiated/event-driven beam reporting, it has been agreed in RAN1 #119 to introduce a new UCI type for the periodic PUCCH resource configuration. The new UCI type is different from SR and is not linked to SchedulingRequestId.

In the legacy systems, when a serving cell configured/activated with cell DRX is not in a cell DRX active period, the UE does not transmit periodic CSI on PUCCH and semi-persistent CSI on PUSCH/PUCCH but transmits aperiodic CSI (see the text copied below from 38.321 Section 5.34.3). The UE further does not transmit SR when the serving cell is not in the cell DRX active period and does not deliver any configured uplink grant for a new transmission or retransmission (see the text copied below from 38.321 Section 5.34.3).

- 5.34.3 Cell Discontinuous Reception Cell DRX is configured if cellDTX-DRX-ConfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by: … When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while: - cellDTX-DRX-onDurationTimer is running for the associated Serving Cell. For each Serving Cell configured with cell DRX, the MAC entity shall: 1> if cell DRX is activated for this Serving Cell: 2> if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset): 3> start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe. NOTE 1: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the cell DRX duration. 1> if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period: 2> not instruct the physical layer to signal a SR on a PUCCH resource for SR; 2> not increment the SR_COUNTER for a SR; 2> not start the sr-ProhibitTimer for a SR; 2> not deliver any configured uplink grant and the associated HARQ information to the HARQ entity; 2> not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission; 2> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH; 2> if an emergency service is initiated by upper layers and this Serving Cell is the SpCell: 3> initiate a Random Access procedure (as specified in clause 5.1.1).

- Outside of cell DRX active period, the UE does not transmit periodic CSI on PUCCH and semi-persistent CSI on PUSCH/PUCCH but transmits aperiodic CSI.

- Outside of cell DRX active period, the UE does not transmit SR and does not deliver any configured uplink grant for a new transmission or retransmission.

As shown in Figure 1 below, in Mode A of UE-initiated CSI reporting, after the transmission of the first PUCCH, the network may choose to transmit a DCI to schedule the second PUSCH carrying the UE-initiated CSI report outside of the cell DRX active period. If the network transmits such a DCI, the UE can transmit the UE-initiated CSI report outside the cell DRX active period, similar to the behavior for aperiodic CSI reporting.

Furthermore, since the first PUCCH transmission in Mode A is functionally similar to a legacy scheduling request (SR), the UE may follow the legacy behavior for SR under cell DRX, i.e., the UE does not transmit the first PUCCH when the serving cell is not in the cell DRX active period.

- : Mode A of UE-initiated CSI reporting under cell DRX

- In Mode A of UE-initiated CSI reporting, when a serving cell configured/activated with cell DRX is not in a cell DRX active period:

- the UE does not transmit the first PUCCH with a new UCI, and

- the UE transmits the UE-initiated CSI report in second PUSCH, if DCI scheduling the second PUSCH is received (similar to aperiodic CSI reporting).

As shown in Figure 2 below, in Mode B of UE-initiated CSI reporting, after the first PUCCH transmission, the subsequent Type 1 configured grant PUSCH (i.e., second PUSCH) may fall outside of the cell DRX active period. In legacy systems, when a serving cell configured with cell DRX is not in a DRX active period, the UE does not deliver any configured uplink grant for a new transmission or retransmission. If this legacy behavior is applied to Mode B, the UE would not transmit the Type 1 configured grant PUSCH carrying the UE-initiated CSI report when it falls outside of the cell DRX active period. However, this may create a behavioral mismatch with Mode A, where transmission of the second PUSCH outside of DRX active time may be permitted (upon DCI trigger). To enable consistent behavior across Mode A and Mode B, one option is to allow the UE to transmit the second PUSCH (using the first available CG resource) after the first PUCCH transmission even if the second PUSCH falls outside of the DRX active time. Another option is not to allow the UE to transmit the second PUSCH outside of the DRX active time (legacy behaviour). In this case, there is no need to transmit the first PUCCH transmission even if it falls inside of the DRX active time.

- : Mode B of UE-initiated CSI reporting under cell DRX

- In Mode B of UE-initiated CSI reporting, when a serving cell configured/activated with cell DRX is not in a Cell DRX active period, the UE does not transmit the first PUCCH with a new UCI. As for the Type 1 configured grant PUSCH transmission, down-select one of the following alternatives:

Alt1: After the UE transmits the first PUCCH with a new UCI in a Cell DRX active period, if the first available Type 1 configured grant PUSCH transmission occasion falls outside of Cell DRX active period, the UE transmits the UE-initiated CSI report in the second PUSCH (i.e., to align with Mode A of UE-initiated CSI reporting).

Alt2: After the UE transmits the first PUCCH with a new UCI in a Cell DRX active period, if the first available Type 1 configured grant PUSCH transmission occasion falls outside of Cell DRX active period, the UE does not transmit the UE-initiated CSI report in the second PUSCH (i.e., legacy behaviour).

Alt3: If the first available Type 1 configured grant PUSCH transmission occasion falls outside of Cell DRX active period, the UE does not transmit the first PUCCH with a new UCI in Cell DRX active period.

## FR2 Uplink Gap

The UE may perform FR2 transmission power management by using a FR2 UL gap during which regular uplink data transmissions are paused. The uplink slots used for FR2 UL gaps are used for body proximity sensing (BPS). In the legacy systems, in order not to degrade the system performance, the UE can still perform some critical uplink transmissions during FR2 UL gap such as Msg1/Msg3/MsgA, SR, LRR, UL-SCH for configured grant, and CSI/L1-RSRP report during SCell activation (see the text copied below from 38.321 Section 5.30).

## 5.30 Handling of FR2 UL gap During the FR2 UL gap configured by ul-GapFR2-Config as specified in TS 38.331 [5], the MAC entity shall, on the Serving Cell(s) of FR2 single CC and intra-band CA, or on the Serving Cell(s) of FR2 inter-band CA where UE does not support tx-Support-UL-GapFR2: 1> only perform transmission of: 2> PRACH preamble as specified in clause 5.1.2 and 5.1.2a; 2> UL-SCH for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 2> UL-SCH for configured grant; 2> the valid CSI report during SCell activation procedure where the valid CSI report is valid CQI with non-zero CQI index defined in TS 38.214 [7], clause 5.2.2.1, when the time period between UL gap colliding with CSI report of non-zero CQI and the slot where the SCell activation MAC CE or CSI report activation command is received is no less than 10 ms; 2> the valid L1 RSRP report during SCell activation procedure, where the valid L1 RSRP report is non lowest L1 RSRP defined in TS 38.133 [11], clause 10.1.6, when the time period between UL gap colliding with L1 RSRP reporting and the slot where the SCell activation MAC CE or CSI report activation command is received is no less than 10 ms; 2> the PUCCH transmission for SR, and link recovery request (LRR) defined in TS 38.133 [11], clause 8.5.

Since both Mode A and Mode B of UE-initiated CSI reporting uses the first PUCCH transmission with a new UCI, and a Type 1 configured grant PUSCH in Mode B of UE-initiated CSI reporting, the UE behaviour when these uplink transmissions overlap with FR2 UL gaps needs to be specified. In the case of Mode A, the second PUSCH is dynamically scheduled and can be transmitted in special slots or uplink slots not overlapping with FR2 UL gaps. As such, in our view handling of this overlap does not require special consideration.

However, for Mode B, the Type 1 configured grant PUSCH may fall within the FR2 UL gap. Given that UE-initiated CSI reporting is a key mechanism for radio connection maintenance, dropping the first PUCCH and/or the configured grant PUSCH during the FR2 UL gap could negatively impact system performance. Therefore, in our view these transmissions should be prioritized during FR2 UL gaps, similar to other critical uplink transmissions already allowed (e.g., Msg1/Msg3/MsgA, SR, LRR, UL-SCH for configured grant, CSI/L1-RSRP reports).

- During FR2 UL gap, the UE performs transmission of the first PUCCH with the new UCI for both Mode-A and Mode-B of UE-initiated CSI reporting and the Type 1 configured grant PUSCH transmission for the Mode-B of UE-initiated CSI reporting (that does not carry UL-SCH).

## Invalid first PUCCH and second PUSCH resource

In the rapporteur's summary on the list of open issues, one proposal is to address UE behavior when a UE-initiated report is triggered but no valid PUCCH or PUSCH resource is available for its transmission. TAT expiry is given as one example. However, TAT expiry is just one case where valid resources may be unavailable. Another example is when a UE-initiated report is triggered, but the uplink BWP configured with the first PUCCH resource and/or Type 1 CG PUSCH resource for the UE-initiated CSI reporting is not active.

In legacy systems, when the UE is in a "non-synchronized" UL synchronization status and uplink data arrives, or when SR is triggered but no valid SR resource is available on the active uplink BWP, the UE initiates a random-access procedure. During or after this procedure, the network provides the UE with uplink resources via DCI for UL data transmission.

However, in the case of UE-initiated CSI reporting if the first PUCCH transmission has not yet been received by the network, the network is not aware that the UE has triggered CSI reporting. As a result, the network may not transmit a DCI with a CSI request field indicating the CSI trigger state in Mode A, or may not monitor Type 1 CG PUSCH resources in Mode B.

One possible solution is to indicate the triggered UE-initiated CSI reporting in Msg3. However, this would require changes to the Msg3 format and increase its size. Another solution could be not to perform event instance evaluation when no valid PUCCH or PUSCH resource is available. However, this falls under RAN1 scope, and it is proposed that RAN1 discuss and clarify UE behavior in the following two scenarios: 1) when there is no valid PUCCH/PUSCH resource to transmit UE-initiated report due to TAT expiry, and 2) when the uplink BWP configured with the first PUCCH resource and/or Type 1 CG PUSCH resource for the UE-initiated CSI reporting is not active.

- RAN2 waits for RAN1 discussion/clarification on the UE behavior under the two cases below:

- Case 1: No valid PUCCH or PUSCH resource is available to transmit UE-initiated report due to expiry of TAT.

- Case 2: The uplink BWP configured with the first PUCCH resource and/or Type 1 CG PUSCH resource for the UE-initiated CSI reporting is not active.

# Remaining issues

## Cell DTX

In the legacy systems, for a serving cell configured with cell DTX, after transmission of an SR on PUCCH, the UE monitors PDCCH on the serving cell regardless of whether the serving cell is in the cell DTX active period or not (see the text copied below from 38.321 Section 5.34.2).

5.34.2 Cell Discontinuous Transmission Cell DTX is configured if cellDTX-DRX-ConfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by: … For each Serving Cell configured with cell DTX, the MAC entity shall: 1> if cell DTX is activated for this Serving Cell: 2> if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset): 3> start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe. NOTE: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the cell DTX duration. 1> if cell DTX operation is deactivated for this Serving Cell; or 1> if the Serving Cell is in the cell DTX Active Period: 2> monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification. 1> if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or 1> if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or 1> if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or 1> if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a): 2> monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification. 1> if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell: 2> monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

In RAN2#129b, the following agreement regarding DRX active time of Mode A of UE-initiated CSI reporting has been reached.

RAN2#129bis Agreement - In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH.

In Mode A of UE-initiated CSI reporting, the UE transmits the first PUCCH to request uplink resources for the UE-initiated beam report. After this transmission, the UE receives a DCI scheduling an uplink grant for the UE-initiated beam report. The UE then transmits the UE-initiated beam report on the second PUSCH. If cell DTX is configured, the UE should monitor PDCCH after the first PUCCH transmission to receive the DCI, even if the serving cell is not in the cell DTX active period.

In Mode B of UE-initiated CSI reporting, since the UE uses Type 1 configured grant (CG) PUSCH to transmit the UE-initiated beam report, no DCI is required. Therefore, the UE does not need to monitor PDCCH after the first PUCCH transmission. In Mode B, there is no impact on PDCCH monitoring due to the serving cell’s DTX active or inactive period.

- For a serving cell configured with cell DTX, after a new UCI for Mode A of UE-initiated beam reporting is sent on first PUCCH, the UE monitors PDCCH on serving cell(s) in the DRX group of the serving cell, regardless of whether the serving cell is in a cell DTX active period.

## DRX

In RAN2#AH1807, the following agreement has been reached regarding CSI reports outside of DRX active time.

RAN2 #AH1807 Agreement - confirm that periodic CSI and semi-persistent CSI on PUCCH is only reported when in active time. - confirm the RAN1 agreement that semi-persistent CSI on PUSCH is only reported when in active time and update the MAC specification accordingly. - confirm that aperiodic CSI reporting on PUSCH can be sent outside of active time if the gNB decides to request so.

According to RAN2#AH1807 agreement, a UE does not transmit periodic CSI on PUCCH and semi-persistent CSI on PUSCH/PUCCH outside of DRX active time, but transmits aperiodic CSI outside of DRX active time (see the text copied below from 38.321 Section 5.7).

## 5.7 Discontinuous Reception (DRX) … Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected. … When DRX is configured, the MAC entity shall: … 1> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and 1> if the current symbol n occurs within drx-onDurationTimer duration; and 1> if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause: 2> if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI configured on PUSCH; 3> not report semi-persistent CSI on PUCCH; 3> if ps-TransmitPeriodicL1-RSRP is not configured with value true: 4> not report periodic CSI that is L1-RSRP on PUCCH. 3> if ps-TransmitOtherPeriodicCSI is not configured with value true: 4> not report periodic CSI that is not L1-RSRP on PUCCH. 1> else: 2> in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and 2> if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group; 3> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

- Outside of DRX active time, a UE only transmits aperiodic CSI report, and does not transmit periodic CSI report or semi-persistent CSI report on PUCCH/PUSCH.

In RAN2#129b, the following agreement regarding Mode A of UE-initiated CSI reporting has been reached.

RAN2#129bis Agreement - In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH.

In Mode A of UE-initiated CSI reporting, since the DRX active time includes the time following the new UCI transmission on the first PUCCH, the UE can receive a DCI scheduling the second PUSCH transmission carrying the UE-initiated CSI report during the active time. The UE can then transmit the second PUSCH accordingly, in a manner similar to legacy aperiodic CSI reporting.

In contrast, Mode B of UE-initiated CSI reporting uses pre-configured Type 1 configured grant (CG) PUSCH resources. After transmitting the new UCI on the first PUCCH, the UE does not need to monitor the PDCCH for DCI, and therefore the DRX active time does not need to be extended beyond the new UCI transmission. As a result, the first available CG-PUSCH resource for Mode B may fall outside the DRX active time. It is currently unclear whether the UE is expected to transmit the UE-initiated CSI report outside DRX active time in this case.

To ensure consistent UE behavior across Mode A and Mode B, we propose that in Mode B of UE-initiated CSI reporting, the UE should transmit the UE-initiated CSI report even when the corresponding CG-PUSCH resource falls outside DRX active time.

- In Mode B of UE-initiated CSI reporting, the UE transmits the UE-initiated CSI report outside of DRX active time (i.e., similar to aperiodic CSI reporting).

# RRC configurations

In RAN1#118, the following agreement has been reached regarding the CC to report the beam report in Mode-B. In particular, the UE-initiated beam report of a CSI report configuration is transmitted on the CC where the CSI report configuration is configured.

RAN1#118 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC.

In the running CR, the parameter resourceForSecondChannelOfModeB-r19 comprises servCellIndex parameter indicating the CC where the configured grant configuration for the second PUSCH carrying the beam report is located. However, the servCellIndex parameter is not needed, as per RAN1#118 agreement, the CC where the configured grant configuration is located is already implicitly determined based on the CC where the CSI report configuration is configured.

- (RRC: UEI-BMR) Remove servCellIndex-r19 from resourceForSecondChannelOfModeB-r19.

# Conclusion

In this contribution, we make the following proposals.

- (MAC-1) UL skipping is not applicable to mode-B type-1 CG event-triggered beam report.

- (MAC-1) For Rel-16 UL skipping (enhancedSkipUplinkTxDynamic is configured), the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated).

- (MAC-1) For Rel-15 UL skipping (skipUplinkTxDynamic is configured), MAC PDU needs to be generated for the UCI for mode-A DG-based UE-initiated report.

- Outside of cell DRX active period, the UE does not transmit periodic CSI on PUCCH and semi-persistent CSI on PUSCH/PUCCH but transmits aperiodic CSI.

- Outside of cell DRX active period, the UE does not transmit SR and does not deliver any configured uplink grant for a new transmission or retransmission.

- In Mode A of UE-initiated CSI reporting, when a serving cell configured/activated with cell DRX is not in a cell DRX active period:

- the UE does not transmit the first PUCCH with a new UCI, and

- the UE transmits the UE-initiated CSI report in second PUSCH, if DCI scheduling the second PUSCH is received (similar to aperiodic CSI reporting).

- In Mode B of UE-initiated CSI reporting, when a serving cell configured/activated with cell DRX is not in a Cell DRX active period, the UE does not transmit the first PUCCH with a new UCI. As for the Type 1 configured grant PUSCH transmission, down-select one of the following alternatives:

Alt1: After the UE transmits the first PUCCH with a new UCI in a Cell DRX active period, if the first available Type 1 configured grant PUSCH transmission occasion falls outside of Cell DRX active period, the UE transmits the UE-initiated CSI report in the second PUSCH (i.e., to align with Mode A of UE-initiated CSI reporting).

Alt2: After the UE transmits the first PUCCH with a new UCI in a Cell DRX active period, if the first available Type 1 configured grant PUSCH transmission occasion falls outside of Cell DRX active period, the UE does not transmit the UE-initiated CSI report in the second PUSCH (i.e., legacy behaviour).

Alt3: If the first available Type 1 configured grant PUSCH transmission occasion falls outside of Cell DRX active period, the UE does not transmit the first PUCCH with a new UCI in Cell DRX active period.

- During FR2 UL gap, the UE performs transmission of the first PUCCH with the new UCI for both Mode-A and Mode-B of UE-initiated CSI reporting and the Type 1 configured grant PUSCH transmission for the Mode-B of UE-initiated CSI reporting (that does not carry UL-SCH).

- RAN2 waits for RAN1 discussion/clarification on the UE behavior under the two cases below:

- Case 1: No valid PUCCH or PUSCH resource is available to transmit UE-initiated report due to expiry of TAT.

- Case 2: The uplink BWP configured with the first PUCCH resource and/or Type 1 CG PUSCH resource for the UE-initiated CSI reporting is not active

- For a serving cell configured with cell DTX, after a new UCI for Mode A of UE-initiated beam reporting is sent on first PUCCH, the UE monitors PDCCH on serving cell(s) in the DRX group of the serving cell, regardless of whether the serving cell is in a cell DTX active period.

- Outside of DRX active time, a UE only transmits aperiodic CSI report, and does not transmit periodic CSI report or semi-persistent CSI report on PUCCH/PUSCH.

- In Mode B of UE-initiated CSI reporting, the UE transmits the UE-initiated CSI report outside of DRX active time (i.e., similar to aperiodic CSI reporting).

- (RRC: UEI-BMR) Remove servCellIndex-r19 from resourceForSecondChannelOfModeB-r19.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024