3GPP TSG RAN WG2#130	R2-2503531

St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	8.17

Source:	OPPO

Title:	Discussion on IoT NTN TDD mode

Document for:	Discussion, Agreement

Introduction

In RAN#105 meeting, a new WID on IoT NTN TDD mode was approved, which was further revised to [1] in RAN#106 meeting. This work item aims to enable Iridium’s TDD band to provide NB-IoT service leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures. The objectives include the following.

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In the last RAN2 meetings, TDD support for NB IoT NTN was discussed, and the following agreements were reached.

Agreements in RAN2#129:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- RAN2 can continue the discussion also on RAR window

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

Agreements in RAN2#129bis:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

In this contribution, we further discuss left open issues on TDD support for NB IoT NTN and present our view.

Discussion

- Regarding the operation in IoT NTN TDD mode, the following agreements have been made in RAN1#119 meeting.

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints. Observation RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

- Based on above RAN1 agreements, the TDD frame structure constraints that all the downlink and uplink transmission can only use one downlink slot and one uplink slot, respectively, with the same index across 90 ms periods. And it has been approved in RAN#106 meeting that D=U=8 and N=9.

- In RAN2#129bis meeting, paging in NTN TDD mode was discussed, and it has been agreed that in NTN TDD mode, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. Following this principle, if the parameter nB used to derive UE’s PF/PO is configured to one of the values highlighted green below, there may be multiple POs postponed to the same D subframe, which would cause the PO collision.

PCCH-Config ::=						SEQUENCE {

defaultPagingCycle					ENUMERATED {

rf32, rf64, rf128, rf256},

nB									ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, oneEighthT,

oneSixteenthT, oneThirtySecondT}

}

- If the parameter nB is configured to a value of fourT, twoT, oneT, halfT, quarterT, or oneEighthT, there may be multiple POs postponed to the same D subframe, which would cause the PO collision.

- To address this issue, the following solutions can be considered.

- Option1: Disable the nB values highlighted green in NTN TDD mode to avoid PO collision.

- Option2: Merge these collided POs into a single PO

- Option3: These collided POs are located back to back, should ensure that the multiple POs in one TDD period (i.e. 90ms) are all transmitted within 8 D subframe.

- Comparing these options, option 1 is most simple while option2 and option 3 will bring more spec impact. Due to the limited time left in R19, we prefer to adopt option 1.

In NTN TDD mode, for parameter nB, only values of oneSixteenthT and oneThirtySecondT T are supported.

- Regarding NPSS/NSSS/NPBCH/SIB1, it has been agreed in RAN1 that legacy NPSS/NSSS/NPBCH/SIB1 pattern is reused, and NPSS/NSSS/NPBCH/SIB1 transmissions are dropped in non-D NB-IoT subframes, so there is no collision issue within NPSS/NSSS/NPBCH/SIB1.

- There is no collision issue for NPSS/NSSS/NPBCH/SIB1.

- According the current spec, an NB-IoT UE determines the start of the SI-window for the concerned SI message as follows:

3> determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT); 3> if the UE is a NB-IoT UE: 4> the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset);

- SI message is transmitted within the corresponding SI window. A SI message is transmitted over 2 or 8 DL subframe, depending on its TB size. Repetition transmission of SI message within the SI window can be supported, and start radio frame for each repetition transmission is determined by the parameter si-RepetitionPattern.

- The value range for si-Periodicity, si-WindowLength and si-RepetitionPattern are given respectively, as below.

si-WindowLength-r13					ENUMERATED {ms160, ms320, ms480, ms640,

ms960, ms1280, ms1600, spare1},

SchedulingInfo-NB-r13::=		SEQUENCE {

si-Periodicity-r13				ENUMERATED {rf64, rf128, rf256, rf512,

rf1024, rf2048, rf4096, spare},

si-RepetitionPattern-r13		ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF},

sib-MappingInfo-r13				SIB-MappingInfo-NB-r13,

si-TB-r13						ENUMERATED {b56, b120, b208, b256, b328, b440, b552, b680}

}

- If the current SI window determination is reused in NTN TDD mode, as all the values for si-Periodicity, si-WindowLength, and si-RepetitionPattern are not aligned with the TDD frame structure periodicity (i.e. 90ms), the start of SI window or the start of each SI message repetition transmission may be located in the non-D NB-IoT subframe. Following the same principle as paging, a simple and straightforward way is to postpone SI message transmission in non-D NB-IoT subframes to the next D NB-IoT subframes.

In NTN TDD mode, legacy NB-IoT SI scheduling mechanism is used. When the determined SI message transmission subframe is not a valid downlink subframe, the SI message transmission is postponed to the nearest valid downlink subframe.

- Then, we need to discuss the following potential collision cases:

- Case 1: Overlapping between SI window(s) in periodicity n-1 and periodicity n

- Case 2: Overlapping between consecutive SI windows

- Case 3: Overlapping between repetitions within a SI window

- For case 1, as the current si-Periodicity value range is from 640ms to 40960ms, it can be up to network to configure the si-Periodicity with a large enough value to avoid this issue.

- For case 2, as the current si-WinfowLength range is from 160ms to 1600ms, it can also to up to network to configure the si-Periodicity with a large enough value to avoid this issue.

- For case 3, this issue can be avoided by disabling some si-RepetitionPattern values (e.g. every2ndRF, every2ndRF, every4thRF, every8thRF) in NTN TDD mode.

In NTN TDD mode, it is up to network to configure si-Periodicity and si-WinfowLength, with proper values to avoid collisions between SI message transmission.

In NTN TDD mode, for si-RepetitionPattern, only a value of every16thRF is supported.

- In RAN1#120bis, RAN1 made the following agreements regarding NPRACH transmission in NTN TDD mode.

Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported

- Based on RAN1 agreements, in NTN TDD mode, the minimum NPRACH periodicity is 90ms, which is equal to the TDD frame periodicity, so in each periodicity of 90ms, there would be at most one NPRACH transmission in non-U NB-IoT subframes to be postponed to the next U NB-IoT subframes. Therefore, there is no collision for NPRACH transmission.

- There is no collision issue for NPRACH transmission.

Regarding ULSPS，RAN2 has agreed that when the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next U subframe, so there may be SPS collision between different UEs. As UE cannot be aware of the UL SPS collision, it should be up to network configuration to avoid this collision, e.g.UE can be configured with a (starting) subframe id within the 8 UL NB-IoT subframes for UL SPS transmission. Since there are also UL SPS occasions located within the U NB-IoT subframes, these UL SPS occasions without any postponement may collide with other postponed UL SPS occasions. To resolve the problem, it is needed that for a SPS occasion within the 8 UL NB-IoT subframes, its (starting) subframe id also follow the configured (starting) subframe id. In other words, the configured (starting) subframe id should be applied to all UL SPS transmissions, regardless of the UL SPS occasion overlapping with non-U NB-IoT subframes or not.

- There may be SPS collision between different UEs, and UEs cannot be aware of the UL SPS collision.

In NTN TDD mode, UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

- In TS36.321, a note on scheduling restriction is given as follow.

NOTE 4: For NB-IoT, for operation in FDD mode, and for operation in TDD mode with a single HARQ process when the associated HARQ RTT timer has been started, DL and UL transmissions will not be scheduled in parallel, i.e. if a DL transmission has been scheduled an UL transmission will not be scheduled until HARQ RTT Timer of the DL HARQ process has expired (and vice versa).

- According to the above note, for a NB-IoT UE configured with a single HARQ process, the UE is not required to monitor PDCCH since a DL or UL transmission has been scheduled. This has also been reflected in the start condition of onDurationTimer as highlighted below.

- if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

- In NTN TDD mode, for UL or DL transmission with repetition, if transmission duration is longer than 8ms, the transmission may be segmented. For UL segmented transmission, there will be DL NB-IoT subframes located between successive UL segmented transmissions. Following the principle in the above note, UE should not monitor PDCCH during these DL NB-IoT subframes, and this should be considered as an additional start condition restriction for onDurationTimer

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Conclusion

Based on the discussion we propose the following:

- In NTN TDD mode, for parameter nB, only values of oneSixteenthT and oneThirtySecondT T are supported.

In NTN TDD mode, legacy NB-IoT SI scheduling mechanism is used. When the determined SI message transmission subframe is not a valid downlink subframe, the SI message transmission is postponed to the nearest valid downlink subframe.

In NTN TDD mode, it is up to network to configure si-Periodicity and si-WinfowLength, with proper values to avoid collisions between SI message transmission.

In NTN TDD mode, for si-RepetitionPattern, only a value of every16thRF is supported.

In NTN TDD mode, UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Reference

[1] RP-243293, Revised WID on introduction of IoT-NTN TDD mode