3GPP TSG-RAN WG2 #130	R2-2503540

St Julian's, Malta, 19th May – 23rd May, 2025

Agenda Item:	8.13.2

Source:	NEC

Title:	Discussion on multi-hop relay discovery and relay reselection

Document for:	Discussion, Decision

Introduction

In the last meeting of Rel-19 multi-hop relay, we need to finalize all remaining issues related to relay discovery and reselection.

Discussion

Previously, RAN2 has made the following agreement related to relay discovery and reselection

Agreements: If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration. If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages. For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold. For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE. FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first. For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE. For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. Agreements: Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected). When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message. RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item. This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI. Agreements: When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. FFS detailed information in the indication and child UE handling. FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

So far, it has been agreed that U2N intermediate relay UE shall regard itself as a U2N remote UE to perform its own relay reselection. But it is not necessary for the U2N intermediate relay UE to have a connected PC5 link with last relay UE when the actual U2N remote UE performing model B discovery since the U2N intermediate relay UE may not have any intention to set up the link due to without any Uu traffic. Therefore, the case mentioned in the below agreement will happen:

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE. FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

It is not necessary for the U2N intermediate relay UE to have a connected PC5 link with last relay UE when the actual U2N remote UE performing model B discovery.

So far in Rel-17 U2N relay, upon the U2N remote UE receives an PC5 RRC message from U2N relay UE, which contains an indication indicated to one of the following causes:

- Relay UE has performed cell reselection

- Relay UE has performed handover

- Relay UE has encountered Uu RLF

- Relay UE has encountered Uu RRC connection establishment failure

#### – NotificationMessageSidelink The NotificationMessageSidelink message is used to send notification message from U2N Relay UE to the connected U2N Remote UE. Signalling radio bearer: SL-SRB3 RLC-SAP: AM Logical channel: SCCH Direction: U2N Relay UE to U2N Remote UE NotificationMessageSidelink message -- ASN1START -- TAG-NOTIFICATIONMESSAGESIDELINK-START NotificationMessageSidelink-r17 ::= SEQUENCE { criticalExtensions CHOICE { notificationMessageSidelink-r17 NotificationMessageSidelink-r17-IEs, criticalExtensionsFuture SEQUENCE {} } } NotificationMessageSidelink-r17-IEs ::= SEQUENCE { indicationType-r17 ENUMERATED { relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection, relayUE-Uu-RRC-Failure } OPTIONAL, -- Need N lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-NOTIFICATIONMESSAGESIDELINK -STOP -- ASN1STOP

For the abovementioned those cases, the U2N remote UE shall trigger relay reselection. The PC5 RRC message is defined as NotificationMessageSidelink message. When it comes to Rel-19 multi-hop U2N relay, RAN2 can consider to reuse this message when U2N intermediate relay UE triggers relay reselection. Additional cause value can be added, which can be defined as parent UE triggering relay reselection.

The message NotificationMessageSidelink can be reused to carry the indication for the child UE.

Additional cause value indicating parent relay UE triggering relay reselection can be added in the indicationType within NotificationMessageSidelink

In addition, it is for sure that when parent relay UE performing relay reselection, a service interruption would be caused no matter if there is any other case happened, e.g. reselection under the same or different serving cell, or reselection would or would not change hop count. Therefore, child UE need to know the reason caused for service interruption. Thus, the message is always needed without any exceptional cases.

The message is always needed without any exceptional cases when parent UE performing relay reselection.

Conclusion

In this contribution, we have discussed remaining issues for relay reselection, a brunch of proposals have been provided in the following:

Proposal 1	It is not necessary for the U2N intermediate relay UE to have a connected PC5 link with last relay UE when the actual U2N remote UE performing model B discovery.

Proposal 2	The message NotificationMessageSidelink can be reused to carry the indication for the child UE.

Proposal 3	Additional cause value indicating parent relay UE triggering relay reselection can be added in the indicationType within NotificationMessageSidelink

Proposal 4	The message is always needed without any exceptional cases when parent UE performing relay reselection.

Reference